MID TERM PERFORMANCE REVIEW OF SBU-D FOR FY-20 & REVISED ESTIMATE FOR FY-21 &FY-22

CHAPTER 10: MID TERM PERFORMANCE REVIEW OF SBU-D FOR FY-20 &

REVISED ESTIMATE FOR FY-21 &FY-22

- Introduction

10.1	Kerala State Electricity Board Limited supplies electricity to the entire population of about 3.47 Cr covering 14 districts in Kerala. SBU-D of KSEB Ltd directly supplies electricity to about 97 Lakh domestic consumers, 21.97 Lakh commercial/generall consumers, 1.41 Lakh industrial consumers, 4.71 Lakh agricultural consumers. The key statistics of distribution network is given below

| Table-10.1 Key Statistics of KSEBL Distribution Network as on 30.09.2019 |  |
| --- | --- |
| Particulars | Statistics |
| Area Sq.km. | 38863 km2 |
| Districts No’s | 14 |
| Electrical Circle Offices | 25 |
| Consumers (Nos) | 12686120 |
| Distribution transformers (Nos) | 80699 |
| HT lines (Ckt. Kms ) | 65370.02 |
| LT lines ( Kms ) | 291461 |

- Capital Expenditure

10.2.1	The gross fixed addition proposed by KSEB Ltd in the ARR petition for Capital Investment Plan under Distribution SBU is given below:

| Table :10.2- GFA addition proposed by KSEB Ltd in the ARR petition for Capital Investment Plan |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | Particulars | FY-19 | FY-20 | FY-21 | FY-22 | Total |
| 1 | Normal woks -Dhyuthi 2021 | 808.64 | 1300.55 | 1139.26 | 787.86 | 4036.31 |
| 2 | Estimated & other funded Works | 199.7 | 201.69 | 203.71 | 205.74 | 810.84 |
| 3 | System strengthening & IT works | 1593.91 |  |  |  | 1593.91 |
| 4 | IT related works (CAP) |  |  |  | 258.14 | 258.14 |
| 5 | Safety | 18 | 19.5 | 20 | 18.5 | 76 |
|  | Total | 2620.25 | 1521.74 | 1362.97 | 1270.24 | 6775.2 |

10.2.2	GFA addition provisionally approved by the Commission is given below:

| Table : 10.3- GFA addition provisionally approved by the Commission for Capital Investment Plan |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No | Particulars | FY-19 | FY-20 | FY-21 | FY-22 | Total |
| 1 | Normal woks -Dhyuthi 2021 | 312.23 | 691.67 | 607.98 | 403.37 | 2015.25 |
| 2 | Estimated& other funded Works | 199.70 | 201.69 | 203.71 | 205.74 | 810.84 |
| 3 | System strengthening & IT works | 1243.77 | 329.31 |  |  | 1573.08 |
| 4 | IT related works (CAP) |  |  |  | 258.14 | 258.14 |
| 5 | Safety | 18.00 | 19.50 | 20.00 | 18.50 | 76.00 |
|  | Total | 1773.70 | 1242.17 | 831.69 | 627.61 | 4475.17 |

10.2.3	After detailed performance evaluation of the plan proposal under ARR petition and reviewing the achievement for the FY- 2019, the petitioner is submitting herewith the revised GFA addition proposal based on the capital investment under Distribution SBU. Detailed performance review report follows:

| Table : 10.4- Revised GFA addition plan for Capital Investment Plan under SBU-D |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Particulars | 2019-20 | 2020-21 | 2021-22 |
| 1 | Normal woks -Dhyuthi 2021 | 847.63 | 1250.93 | 1563.88 |
| 2 | Estimated & other funded Works | 191.65 | 301.1 | 205.74 |
| 3 | Flood related work | 10 | 10 |  |
| 4 | System strengthening Central aided Projects | 1077.44 |  |  |
| 5 | IT related works (CAP) | 61.86 | 42.64 | 202.69 |
| 6 | Safety | 21.5 | 19 | 20.5 |
|  | Total | 2210.08 | 1623.67 | 1992.81 |

Annual performance review of works under SBU-D upto Sept 2019

Physical progress of works

10.2.4	The Physical achievement of the Distribution work which includes Normal Dhyuthi works /estimated cost/ other funded works such as deposit work MP, MLA funded works and Central aidedproject works such as DDUGJY, IPDS, R APDRP is given below.

| Table : 10.5- Physical achievement of distribution works |  |  |  |
| --- | --- | --- | --- |
| SlNo: | Particulars | FY -2019 | FY -2020 ( up to Sept.2019) |
| 1 | Service connections (No:s) | 368673 | 176769 |
| 2 | Construction of 11kV lines(ckt km) | 1773 | 802 |
| 3 | Construction of LT lines(ckt km) | 3401 | 1657 |
| 4 | Single phase to three phase conversion -(km) | 1292 | 721 |
| 5 | Installation of distribution transformers (No:s) | 2410 | 823 |
| 6 | Enhancement of distribution transformers (No:s) | 517 | 148 |
| 7 | Transformer faulty replacement | 1660 | 865 |
| 8 | Faulty meters replacement -Electronic (No:s) | 837858 | 378372 |
| 9 | Faulty meters replacement -mechanical (No:s) |  |  |
| 10 | Mechanical meter replacement (No:) |  |  |
| 11 | LT Reconductoring (ckt. km) | 8355 | 5356 |
| 12 | HT Reconductoring ckt. km) | 840 | 317 |

Financial progress of works under Distribution - SBU

10.2.5	As per annual audited accounts for capital expenditure under Distribution SBU, the cost incurred during the FY-2019 amounts to Rs.1434.67 Cr., and the capitalized cost during the year is Rs.1203.72 Cr. and also capital work in progress amounts to Rs. 1108.24 Cr.

Dhyuthi   Works

Progress of works under Dhyuthi– Works for 2018-19 and up to 30.09.2019 in FY 20

10.2.6	The Dhyuthi project is monitored using PROMOS software. The financial progress of completed works as per PROMOS software is given in table below.

| Table 10.6: Progress of Normal works planned under Dhyuthi( Rs. in Cr.) |  |  |  |
| --- | --- | --- | --- |
| No | Particulars | FY -2019 | FY -2020 ( up to Sept.2019) |
| 1 | Dhyuthi 2018-19 control period work | 43.48 | 124.64 |
| 2 | Dhyuthi 2019-20 control period work |  | 3.30 |
|  | Total | 43.48 | 127.94 |

10.2.7	The capital investment proposed under Dhyuthi scheme for FY-2019 was Rs 808.64Cr. As per PROMOS software, works amounting to Rs 43.48Cr was completed during 2018-19. In addition to this, works amounting to Rs 552.22Cr.was under implementation stage   as on 31.03.2019. The completion date for the same was extended up to January 2020. The completed works amounts to Rs 470.93 Cr. and the works under implementation stage amountsto Rs. 244.96 Cr as on 11.03.2019. Thus out of 808.17 Cr proposed for the FY-2019, works amounting to Rs.715.88Cr. is already under implementation and completion stage as on 11.03.2020. The scheduling of remaining works is under progress. 90 % of the works proposed under Dhyuthi for FY-19   is expected to be completed during the year 2019-20.

10.2.8	The detailed physical and financial progress report of Dhyuthi is attached herewith as Annexures I to III

Reason for delay in executing Dhyuthi work

10.2.9	Hon’ble Commission may please note that the execution of Dhyuthi works during FY-2019 and first two quarters of the FY-2020 were delayed due to allocation of entire resources for the execution of huge quantum of funded works like DDUGJY/IPDS works, Deposit works and unprecedented floods during August 2018 and August 2019. Now the Centrally Aided projects are completed and the focus is to execute Dhyuthi works.

As the works were not executed due to the reasons mentioned above, KSEBL rescheduled the works as detailed below:

Revised capital expenditure plan of capital works under Distribution SBU for the years2019-20, 2020-21 & 2021-22

10.2.10	The petitioner revised the schedule of works under distribution SBU based on revised annual plan 2019-20, and 2020-21 plan period. The plan work for the year 2021-22 is same as that included under Dhyuthi work. Hence the revised plan under Dhyuthi, other funded and flood resilient work is as detailed below:

| Table 10.7 :Revised plan for FY-20 to FY-22 vis a vis provisionally approved ARR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2019-20 |  |  | 2020-21 |  |  | 2021-22 |  |  |
| Particulars | ARR petition | Prov. approved | Revised plan | ARR petition | Prov. approved | Revised plan | ARR petition | Prov. approved | Revised plan |
| Dhyuthi | 1300.55 | 691.7 | 847.63 | 1139.26 | 607.98 | 1250.90 | 787.86 | 403.37 | 1563.88 |
| Other funded works incl ECSC | 201.69 | 201.69 | 191.65 | 203.71 | 203.71 | 301.10 | 205.75 | 205.74 | 205.74 |
| Flood resilient works |  |  | 10 |  |  | 10 |  |  |  |
| Total | 1502.24 | 893.39 | 1049.3 | 1342.97 | 811.69 | 1562.00 | 993.61 | 609.11 | 1769.62 |

10.2.11	In addition to the normal Development under Dhyuthi, the petitioner has to undertake and complete the Centrally Aided projects (ongoing works for system strengthening and IT related works) in a time bound manner. Safety plan is also included under capital investment plan under Distribution SBU.

System strengthening & IT works - Central aided projects

10.2.12The status of Central aided projects for System strengthening works as on 30th September 2019 is detailed below:

Completed works during the FY2019

10.2.13Central aided projects, ongoing RAPDRP Part B amounting to Rs 1115.18 Cr with 60 % grant completed during the FY-2019. Also RAPDRP Part A- IT works amounting to Rs 175.23 Cr with 100 % grant were completed during the FY-2019

Status of ongoing Central aided projects as on 30th September 2019

    - The central aided project Deena Dayal Upadhyaya Grama JyothiYojana (DDUGJY) is expected to be completed during the FY-2020. The project cost for DDUGJY amounts to Rs.485.37 Cr. Overall Financial Progress as per physical achievement as on 30.09-2019 is 99.5%. It is expected to submit the closure proposal by 31-03-2020. 60% is the Grant portion for the project.

    - The central aided project or IPDS work- Sanctioned Project cost is Rs.592.07Cr.Overall Financial Progress as per Physical Achievement   :   Rs. 489.23 Cr   – 82.63%. This project is also declared to be completed during the FY-2020. 60% is the Grant portion for the project.

10.2.16	The detailed project status and the physical and financial progress of DDUGJY and IPDS works as on 31st March 2019 and 30th Sept 2019 is attached as Annexure IV

Status of ITrelated new works proposed under Capital investment plan

    - RAPDRP Part A- SCADA work amounting to Rs. 61.86 Cr is expected to be completed during the FY-2020. It is 100 % grant project.

    - Capital Investment Plan of new IT related works (CAP)-under Distribution SBU for the control period was submitted under ARR petition. The Commission provisionally approved the capital investment plan with a GFA addition of Rs.258.14Cr for the FY-2022.  The status of IT related works is  attached as Annexure V

    - The revised forecast of IT projects with GFA addition plan follows:

| Table 10.8: Revised GFA addition plan for IT related works |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Particulars | FY-20 | FY-21 | FY-22 | Grant |
|  | New works |  |  |  |  |
| 1 | IPDS- Phase II Incr IT |  |  | 22.86 | 13.72 |
| 2 | IPDS--ERP |  | 42.64 |  | 22.58 |
| 3 | IPDS-Smart Metering -Kesevadasapuram |  |  | 19.525 |  |
| 4 | IPDS-Smart metering- UDAY |  |  | 64.36 | 38.62 |
| 5 | Smart Grid |  |  | 90.7 | 11.715 |
| 6 | RT-DAS |  |  | 5.25 | 3.15 |
| 7 | Cyber security |  |  |  |  |
|  | Ongoing projects |  |  |  |  |
| 8 | Part A RAPDRP SCADA | 61.86 |  |  | 61.86 |
|  | Sub Total | 61.86 | 42.64 | 202.695 | 151.645 |

Safety Plan

10.2.20	The major proposals for improving the safety culture of employees and adopting new technologies for reducing the number of accidents from KSEBL installations are listed below.

- Providing safety kit to all line staff in Distribution wing

- Procurement of Vehicle with Ariel lift for electrical section offices

- Procurement of Modern Safety Equipment’s in Transmission wing and Generation wing.

- Safety training to employees and contract workers in distribution5. Public awareness activities.

10.2.21The petitioner filed capital investment plan for Rs. 76 Cr. in the ARR petition for the Control period Honorable Commission provisionally approved the safety plan as such.

Achievement in safety Activities

10.2.22	The petitioner expended around Rs. 15 Cr during 2018- 19 for providing 9253 nos. safety kit to line staff under Distribution SBU. The amount expended for safety equipment during 2019-20 control period till 30th September2019 comes to Rs.1.49Cr.

10.2.23	Revised forecast of Safety activities for the control period 2018-19, 2019-20 & 2021-22

| Table 10.9: Rescheduled Capital investment plan for Safety (Rs Cr) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Particulars | FY-20 | FY-21 | FY-22 | Total |
| 1 | Providing safety kit to line staff | 4 | 4 | 5 | 13 |
| 2 | Aerial lift for SBU-D | 14 | 12 | 13 | 39 |
| 3 | Safety Equipment SBU-T&G wing | 3.5 | 3 | 2.5 | 9 |
|  | Total | 21.50 | 19 | 20.5 | 61 |

10.2.24	Based on the actual progress of the work under Distribution SBU and the revised capital outlay, the revised GFA addition plan for the rest of the control period is submitting for approval.

EV Policy – Initiatives

10.2.25	Following initiatives have been taken by KSEB so far, towards adoption of EV policy in the State of Kerala, which has notified KSEBL as the nodal agency for setting up EV charging stations across the State.

- Implementation of Pilot Projects

- KSEB has now invited tender for the installation of Charging Stations in Board’s own land along the National Highway in six Corporation areas viz. Thiruvananthapuram, Kollam, Kochi, Thrissur, Kozhikode & Kannur, as a Pilot Project. The PAC of the project is Rs. 1.41 Crore. The work shall include setting up one charging station with multiple charging points along with the required canopy. The work is expected to be completed within two months’ from the date of award of work.

- Installation of EVCS in 32 locations in Thiruvananthapuram District at an amount of Rs. 8.2 Crore, using e-mobility promotion fund of Government of Kerala. The work is expected to be commenced by May, 2020 and completed by September, 2020.

- Utilisation of FAME-II incentives

- The Department of Heavy Industries, Government of India has invited applications for availing incentives for the deployment of charging infrastructure in the cities. The incentives ranges from 50% of the capital cost to 100% capital cost depending on the category, specifically defined in this respect in the EOI. With Kerala having seven cities coming under the category of cities with population above one million, KSEB has applied under the scheme for seven million plus cities, where funds are being offered for setting up 25 charging stations having 7 or 8 chargers each under each city. Accordingly, 131 EV charging stations were approved in four cities as phase I, remaining will be approved in phase II.

10.2.26	KSEB has already rolled out an EOI for short listing firms for setting up PCS in Kerala. On completion of tendering process, it is planned to set up the PCS across the four cities through the selected agencies. The work is expected to be awarded by June 2020 and completed by March 2021.

10.2.27	Detailed write up on EV charging infrastructure proposal and the mode of implementation by KSEBL is attached as Annexure VI.

| Table : 10.10 Revised GFA addition plan for Capital Investment Plan under SBU-D |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Particulars | 2019-20 | 2020-21 | 2021-22 |
| 1 | Normal woks -Dhyuthi 2021 | 847.63 | 1250.93 | 1563.88 |
| 2 | Estimated & other funded Works | 191.65 | 301.1 | 205.74 |
| 3 | Flood related work | 10 | 10 |  |
| 4 | System strengthening Central aided Projects | 1077.44 |  |  |
| 5 | IT related works (CAP) | 61.86 | 42.64 | 202.69 |
| 6 | Safety | 21.5 | 19 | 20.5 |
|  | Total | 2210.08 | 1623.67 | 1992.81 |

10.3 Energy Sales forecast

10.3.1	KSEB Ltd vide MYT petition for the control period from 2018-19 to 2021-22 has projected the energy sales for the control period as follows

| Table 10.11 Energy projected in the MYT petition (MU) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Control period |  |  |  |
| Category | 2017-18 | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| LT I Domestic | 10,569.99 | 10,856.52 | 11,439.87 | 11,901.30 | 12,381.33 |
| LT VI&VII Commercial | 3,063.48 | 3,168.22 | 3,448.97 | 3,660.73 | 3,885.49 |
| LT IV Industrial | 1112.33 | 1,100.98 | 1,134.68 | 1,146.03 | 1,157.49 |
| LT V Agricultural | 346.03 | 350.02 | 368.65 | 377.80 | 387.19 |
| LT VIII Street Lights | 373.48 | 380.84 | 404.36 | 420.75 | 437.81 |
| LT II Colonies | 4.85 | 4.90 | 5.05 | 5.10 | 5.15 |
| LT IX Adv. Hoardings |  | 2.01 | 2.12 | 2.18 | 2.25 |
| LT Total | 15,470.16 | 15,863.49 | 16,803.70 | 17,513.89 | 18,256.71 |
| HT I Industrial | 2062.99 | 2,137.66 | 2,280.91 | 2,385.08 | 2,494.01 |
| HT II General | 761.64 | 797.91 | 864.87 | 918.70 | 975.88 |
| HTIII Agricultural | 9.61 | 9.69 | 10.17 | 10.46 | 10.76 |
| HT IV Commercial | 644.82 | 671.26 | 727.59 | 772.88 | 820.98 |
| HT V Domestic | 14.97 | 15.41 | 16.51 | 17.33 | 18.20 |
| EHT I 66kV Industrial | 247.34 | 355.56 | 369.22 | 375.75 | 382.38 |
| EHT II 110kV industrial | 631.13 | 748.67 | 778.40 | 793.12 | 808.12 |
| EHT III 220kV Industrial | 77.99 | 94.83 | 101.61 | 106.69 | 112.02 |
| EHT Non industrial | 70.14 | 70.48 | 73.75 | 75.62 | 77.54 |
| Railway Traction | 265.80 | 273.51 | 293.05 | 307.70 | 323.09 |
| KMRL | 15.35 | 15.79 | 16.92 | 17.77 | 18.66 |
| HT & EHT Total | 4801.78 | 5,190.77 | 5,533.00 | 5,781.10 | 6,041.64 |
| Bulk | 608.77 | 593.03 | 632.99 | 662.29 | 693.10 |
| Total | 20,880.71 | 21,647.29 | 22,969.69 | 23,957.28 | 24,991.45 |
| Growth (%) |  | 2.35 | 6.10 | 4.30 | 4.32 |

10.3.2	Honorable Commission has approved the same vide order dated 08.07.2019. It may please be noted that the energy sales projected for 2018-19 was 21647.29 MU which is inclusive of energy availed through open access. The actual energy sales for 2018-19 was 21536.77 MU and energy availed through openaccess were 205.44 MU. The details are given in chapter 4 of this petition

Revised Energy sale forecast for the current control period

10.3.3	The revised energy sales of each consumer category was estimated based on the past actual data such as (i) category wise consumer strength, (ii) category wise consumption, (iii) Compound Annual Growth Rate (CAGR) taking the data of 2018-19, (iv) actual consumption during the year 2019-20 till September-2019, (v) new service connections proposed during the ensuing years, (vi) DSM initiatives taken by the Board and impact of wheeled energy, etc. The energy consumption forecast has also considered the economic growth, regional characteristics of consumers, seasonal variations, change in consumer habits, cumulative average growth rate during the previous years etc. Now, the CAGR of sales upto 2018-19 is given in the table below:

| Table 10.12 : Historical trends in CAGR |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | 10 year CAGR | 7 year CAGR | 6 year CAGR | 5year CAGR | 4year CAGR | 3 year CAGR | 2 year CAGR | YOY increase |
| Domestic | 6.24 | 5.05 | 4.58 | 4.47 | 3.78 | 3.01 | 2.81 | 2.74 |
| Commercial | 7.93 | 6.00 | 6.36 | 7.65 | 7.45 | 5.60 | 4.36 | 5.16 |
| Industrial | 0.91 | 0.19 | 0.15 | 0.27 | 0.33 | 0.25 | -0.90 | -0.06 |
| Agricultural | 4.13 | 2.39 | 1.65 | 1.71 | 3.75 | 6.51 | 2.40 | -2.42 |
| Street Lights | 2.53 | 3.63 | 3.17 | 3.31 | 2.19 | 1.00 | 0.27 | 1.15 |
| LT II |  | -16.85 | -17.93 | -17.53 | -10.25 | -15.85 | -15.29 | -10.78 |
| LT Advt and Hoardings |  | 0.00 | 0.00 | 0.00 | -7.79 | 6.41 | 6.64 | 1.49 |
| Total | 5.90 | 4.72 | 4.44 | 4.62 | 4.16 | 3.32 | 2.76 | 2.86 |
| HT I | 5.04 | 4.47 | 4.31 | 4.13 | 4.15 | 5.27 | 4.75 | 3.92 |
| HT II | 22.09 | 31.58 | 35.91 | 43.07 | 9.39 | 5.25 | 4.62 | 3.13 |
| HTIII | 0.18 | 1.37 | 1.11 | 3.37 | 6.76 | 9.40 | -1.63 | -7.14 |
| HT IV | 1.87 | -3.08 | -3.66 | -4.61 | 4.73 | 6.02 | 7.36 | 8.00 |
| HT V | 0.00 | 0.00 | 0.00 | 0.00 | 18.87 | 19.50 | 9.24 | 8.95 |
| EHT-1 | 1.86 | -1.03 | -0.08 | -0.12 | -0.21 | 5.16 | -3.61 | -5.92 |
| EHT-II | 0.42 | -2.95 | -2.55 | -1.37 | 0.25 | 0.09 | -0.73 | -4.54 |
| EHT-III | 0.00 | 0.00 | 17.12 | 7.86 | 22.88 | 50.78 | 32.42 | 25.52 |
| EHT General | 0.00 | 0.00 | 0.00 | 1.91 | 0.57 | -0.90 | 1.39 | -5.13 |
| Railway Traction | 7.87 | 10.11 | 9.73 | 8.60 | 10.23 | 12.51 | 14.91 | 14.04 |
| Bulk | 6.54 | 3.43 | 3.00 | 2.71 | 1.92 | 1.13 | -1.16 | -1.77 |
| HT&EHT Total | 4.65 | 3.91 | 4.10 | 4.14 | 4.38 | 5.02 | 4.05 | 2.62 |
| Total | 5.76 | 4.50 | 4.35 | 4.49 | 4.22 | 3.77 | 3.10 | 2.80 |

10.3.4	The energy consumption for various customer categories are estimated primarily based on the CAGR trends during past years. Wherever it is observed that the trend is unreasonable or unsustainable, the growth factors have been corrected to arrive at more realistic projections based on experience.

10.3.5	In the case of year 2019-20, the energy demand for the first half of the financial year showed a considerable increaseduring the period April-July. The energy demand for the first six months from April-August of the financial year 2017-18 , 2018-19 and 2019-20 is given in table below.

| Table 10.13 : Energy Demand in the first half of FY-20 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Month | 2017-18 | 2018-19 | 2019-20 | Growth (FY-18 to FY-19) (%) | Growth (FY-19 to FY-20) (%) |
| April | 2233.75 | 2240.75 | 2429.556 | 0.31 | 8.43 |
| May | 2174.14 | 2154.72 | 2559.158 | -0.89 | 18.77 |
| June | 1889.13 | 1932.05 | 2176.4 | 2.27 | 12.65 |
| July | 1940.58 | 1894.86 | 2092.4 | -2.36 | 10.43 |
| August | 2009.53 | 1791.12 | 1915.51 | -10.87 | 6.94 |
| September | 1909.66 | 2017.95 | 1993.817 | 5.67 | -1.20 |
| Total | 12156.79 | 12031.45 | 13166.84 | -1.03 | 9.44 |

10.3.6	It can be seen from the above table the energy sales during the first quarter of FY-20 show an abnormal increase owing to delay in onset of monsoon and excess heat during summer. Thus the energy sales of 2019-20 is projected considering the actual energy consumption for the period from April-November and that of the remaining period is estimated by applying a reasonable growth rate based on previous trends from the energy sales of corresponding period of FY-19.The energy sales projections for the remaining part of control period is estimated by considering the data of FY-19 as base year; as the first quarter of FY-20 shows an abnormal increase in demand.

10.3.7	For the remaining control period, compound annual growth rate as described below is taken.

- For domestic category, the growth rate of domestic consumption shows a steady decreasing trend during last three years. Since the factors contributing reduction in consumption (Saturation, LED DSM impact, Solar penetration etc) are applicable to last four years, CAGR of 4 years seems to be appropriate for domestic category.As the first quarter of 2019-20 shows an abnormal increase in demand, the same is not used for projecting the next year data.The estimate is arrived by applying CAGR on the data of FY-19. The An allowance of 0.15% reduction is allowed for DSM activities savings and solar penetration for the year 2020-21 and 0.30 % is given for 2021-22 in anticipation of energy savings due toLED distribution. The3 year CAGR is used for energy sales projection of LT Commercial considering the trends, and 4 year CAGR for LT Agriculture and Public lighting category.

- As for LT Industrial category, the consumption shows a small negative growth rate CAGR for last 2 years and YOY change from 2017-18 is negative. Hence 1 % growth is considered for projection.

- Impact of project SOURA and future solar installations by consumers in future years not considered for projection because of difficulty in exact prediction of consumer behaviour.

- In the case of HT & EHT category, it is not possible to predict the quantum of energy purchase by open access consumers because they resort to short term power purchase and have not furnished any definite proposals for the future. Hence energy sales including energy wheeled by embedded open access consumers is also considered for future projections. Energy consumed from captive generation is not considered for future energy sales projection.

- The 4 year CAGR is used for energy sales projection of HT Industrial, Commercial & General combined, HT Agriculture category. HT domestic is relatively a new category formed in FY-15 and CAGR is on the higher side. Hence a growth of 5 % growth is considered.

- In the case of EHT categories, 2.5% is taken for EHT 66 KV, 110 KV and EHT non-industrial category as the CAGR taken with respect to 2018-19 shows abnormal values. In the case of EHT 220 KV, being a captive generator, there is a wide variation in their consumption; hence projection based on their previous trends cannot be relied and hence 3% increase is taken for projection.

- In addition to the sale to its own consumers, KSEBL has been providing electricity to other licensees at the BST approved by the Hon Commission. In the case of licensees, 4 year CAGR is taken for energy sales projection.

- Considering the present trends, 45 MU was considered towards electric vehicle charging stations for FY-21 and 90 MU was estimated for FY-22.

- The estimated energy consumption of the Bulk Consumers for the next control period based on 4 year CAGR

- Thus the energy sales forecast for the control period, estimated as described in preceding paragraphs, is given in table below.

| Table 10.14 :Revised Category wise sales projection (MU) |  |  |  |
| --- | --- | --- | --- |
| Category | 2019-20 | 2020-21 | 2021-22 |
| Domestic | 11584.02 | 11680.62 | 12087.61 |
| Commercial | 3466.51 | 3590.28 | 3791.44 |
| Industrial | 1131.11 | 1134.01 | 1145.35 |
| Agricultural | 364.46 | 363.45 | 377.08 |
| Street Lights | 383.85 | 394.50 | 403.13 |
| LT II | 4.11 | 4.23 | 4.27 |
| LT IX | 2.06 | 2.15 | 2.21 |
| LT X | 0.62 | 0.66 | 0.70 |
| LT Total | 16936.75 | 17169.90 | 17811.80 |
| HT I | 2205.50 | 2351.57 | 2449.18 |
| HT II | 843.61 | 881.59 | 931.01 |
| HTIII | 9.58 | 10.17 | 10.86 |
| HT IV | 751.66 | 776.65 | 820.19 |
| HT V | 17.50 | 17.98 | 18.88 |
| EHT I | 341.62 | 352.38 | 361.19 |
| EHT II | 719.76 | 751.99 | 770.79 |
| EHT III | 119.74 | 122.73 | 126.41 |
| EHT General | 72.03 | 70.59 | 72.71 |
| Railway Traction | 304.21 | 334.20 | 350.91 |
| Bulk | 628.35 | 644.88 | 668.66 |
| KMRL | 15.81 | 15.82 | 16.29 |
| HT VI | 0.00 | 45.00 | 90.00 |
| HT & EHT Total | 6029.36 | 6375.55 | 6687.07 |
| Total | 22966.11 | 23545.45 | 24498.87 |

10.3.8	Based on the above projection, the forecast of each tariff Category for the next control period is made and provided in table below.

| Table 10.15 : Sales Projection for LT Category (MU) |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Category | 2019-20 | 2020-21 | 2021-22 |
| 1 | LT I Domestic | 11584.02 | 11680.62 | 12087.61 |
| 2 | LT II Colony | 4.11 | 4.23 | 4.27 |
| 3 | LT IV A Industrial | 1120.23 | 1123.10 | 1134.33 |
| 4 | LT IV B Industrial | 10.88 | 10.91 | 11.02 |
| 5 | LT V A Agriculture | 301.34 | 300.51 | 311.78 |
| 6 | LT V B Agriculture | 63.12 | 62.94 | 65.30 |
| 7 | LT VI A General | 225.38 | 222.66 | 235.13 |
| 8 | LT VI B General | 128.40 | 129.39 | 136.64 |
| 9 | LT VI C General | 250.92 | 259.87 | 274.44 |
| 10 | LT VI D General | 22.50 | 23.31 | 24.61 |
| 11 | LT VI E General | 3.01 | 3.12 | 3.30 |
| 12 | LT VI F General | 945.89 | 979.66 | 1034.55 |
| 13 | LT VI G General | 78.25 | 81.05 | 85.59 |
| 14 | LT VII A Commercial | 1596.37 | 1678.50 | 1772.55 |
| 15 | LT VII B Commercial | 192.23 | 188.33 | 198.88 |
| 16 | LT VII C Commercial | 23.55 | 24.39 | 25.76 |
| 17 | LT VIII A Public Lighting | 281.71 | 289.53 | 295.86 |
| 18 | LT VIII B Public Lighting | 102.14 | 104.97 | 107.27 |
| 19 | LT IX Advertisements and Hoardings | 2.06 | 2.15 | 2.21 |
| 20 | LT X Electric vehicle charging stations | 0.62 | 0.66 | 0.70 |
|  | LT Subtotal | 16936.75 | 17169.90 | 17811.80 |

10.3.9	The Projection for HT and EHT consumer categories are given below:

| Table 10.16 : Sales Projection for HT & EHT Category (MU) |  |  |  |  |
| --- | --- | --- | --- | --- |
| HT category |  |  |  |  |
| No | Category | 2019-20 | 2020-21 | 2021-22 |
| 1 | HT I A Industrial | 2191.21 | 2336.33 | 2433.30 |
| 2 | HT I B Industrial | 14.29 | 15.24 | 15.87 |
| 3 | HT II A General | 201.46 | 210.53 | 222.33 |
| 4 | HT II B General | 642.15 | 671.05 | 708.67 |
| 5 | HT III A Agriculture | 7.31 | 7.76 | 8.28 |
| 6 | HT III B Agriculture | 2.27 | 2.41 | 2.58 |
| 7 | HT IV A Commercial | 453.98 | 340.87 | 359.98 |
| 8 | HT IV B Commercial | 297.68 | 435.78 | 460.21 |
| 9 | HT V Domestic | 17.50 | 17.98 | 18.88 |
| 10 | HT VI Electric charging vehicles | 0.00 | 45.00 | 90.00 |
|  | HT Sub Total | 3827.84 | 4082.96 | 4320.11 |
| EHT Category |  |  |  |  |
| No | Category | 2019-20 | 2020-21 | 2021-22 |
| 1 | EHT 66 KV Industrial | 341.62 | 352.38 | 361.19 |
| 2 | EHT 110KV Industrial | 719.76 | 751.99 | 770.79 |
| 3 | EHT 220 KV Industrial | 119.74 | 122.73 | 126.41 |
| 4 | EHT General A | 15.93 | 15.61 | 16.08 |
| 5 | EHT General B | 56.10 | 54.99 | 56.64 |
| 6 | EHT Railway Traction | 304.21 | 334.20 | 350.91 |
| 7 | KMRL | 15.81 | 15.82 | 16.29 |
| 8 | EHT Sub Total | 1573.17 | 1647.71 | 1698.30 |

10.3.10	Similarly, the projection for Licensees and others are given below:

| Table 10.17 : Sales Projection for Licensees and others (MU) |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Category | 2019-20 | 2020-21 | 2021-22 |
| 1 | KPUPL | 95.44 | 96.65 | 101.22 |
| 2 | CSEZ | 58.63 | 59.03 | 59.46 |
| 3 | RPL | 31.97 | 35.17 | 37.43 |
| 4 | Technopark | 93.90 | 99.31 | 104.74 |
| 5 | Cochin Port trust | 36.77 | 36.99 | 37.80 |
| 6 | Thrissur Corporation | 163.06 | 165.28 | 169.82 |
| 7 | KDHPCL | 61.25 | 64.04 | 67.88 |
| 8 | MES | 71.74 | 72.07 | 73.18 |
| 9 | Karnataka | 0.79 | 0.80 | 0.82 |
| 10 | Infopark | 10.51 | 11.04 | 11.59 |
| 11 | Smart City | 4.29 | 4.50 | 4.73 |
|  | Subtotal | 628.35 | 644.88 | 668.66 |
|  | Grand Total | 22966.11 | 23545.45 | 24498.87 |

10.4 AggregateTechnical and Commercial Losses

T& D losses

10.4.1	The T&D loss proposed by KSEBL and approved by Honorable Commission for the control period is given in table below.

| Table 10.18 T&D losses |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Particulars | Estimated |  |  |  | Approved |  |  |  |
|  |  | FY-19 | FY-20 | FY-21 | FY-22 | FY-19 | FY-20 | FY-21 | FY-22 |
| 1 | T&D loss reduction | 0.25% | 0.40% | 0.40% | 0.40% | 0.25% | 0.40% | 0.40% | 0.40% |
| 2 | T & D loss | 12.82% | 12.42% | 12.02% | 11.62% | 12.82% | 12.42% | 12.02% | 11.62% |
| 3 | Transmission Losses | 4.05% | 3.95% | 3.85% | 3.75% | 4.05% | 3.95% | 3.85% | 3.75% |

10.4.2	Against the estimated and approved T&D loss of 12.82% KSEBL has achieved 12.47% for the year 2018-19. KSEBL is considering the approved loss reduction targets for FY-20 to FY-22 for estimation.

| Table 10.19 : Revised claim of T&D loss during the control period |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Particulars | FY 19 | FY-20 | FY-21 | FY-22 |
|  |  | Actual | Estimated |  |  |
| 1 | T&D loss reduction | 0.60% | 0.40% | 0.40% | 0.40% |
| 2 | T & D loss | 12.47% | 12.07% | 11.67% | 11.27% |
| 3 | Transmission Losses | 3.74% | 3.65% | 3.55% | 3.45% |

Energy Requirement

10.4.3	Based on the revised energy sales and revised T&D losses the revised energy requirement is given in table below.

| Table 10.20 Energy sales and energy requirement for the control period |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Estimate vide MYT petition dated 31.10.2018 |  |  |  |  |  |
| Particulars | Unit | 2018-19 | 2019-20 | 2020-22 | 2021-22 |
| Energy Sales | MU | 21,647.29 | 22,969.69 | 23,957.28 | 24,991.45 |
| T&D Loss | % | 12.82% | 12.42% | 12.02% | 11.62% |
| Substation auxiliary consumption | MU | 15.58 | 16.36 | 17.18 | 18.04 |
| Energy Requirement | MU | 24,846.15 | 26,243.46 | 27,247.55 | 28,295.31 |
| Approved vide MYT order dated 08.07.2019 |  |  |  |  |  |
| Particulars | Unit | 2018-19 | 2019-20 | 2020-22 | 2021-22 |
| Energy Sales | MU | 21,647.29 | 22,969.69 | 23,957.28 | 24,991.45 |
| T&D Loss | % | 12.82% | 12.42% | 12.02% | 11.62% |
| Substation auxiliary consumption | MU | 15.58 | 16.36 | 17.18 | 18.04 |
| Energy Requirement | MU | 24,846.15 | 26,243.46 | 27,247.55 | 28,295.31 |
| Revised Estimate |  |  |  |  |  |
| Particulars | Unit | 2018-19 | 2019-20 | 2020-22 | 2021-22 |
| Energy Sales inside the State | MU | 21,536.77 | 22,966.11 | 23,545.45 | 24,498.87 |
| T&D Loss* | % | 12.47% | 12.07% | 11.67% | 11.27% |
| Substation auxiliary consumption | MU | 16.11 | 16.92 | 17.76 | 18.65 |
| Energy Requirement | MU | 24,646.89 | 26,135.55 | 26,673.99 | 27,629.24 |
| * For the year 2018-19,T&D loss is calculated considering actual open access transportation also |  |  |  |  |  |

10.4.4	Based on the above, the distribution loss and AT&C loss estimates are revised as follows:

| Table 10. 21 Revised estimate of AT&C losses and collection efficiency |  |  |  |  |
| --- | --- | --- | --- | --- |
| Estimate |  |  |  |  |
| A | Particulars | 2019-20 | 2020-21 | 2021-22 |
|  | Distribution losses (%) | 10 | 9.92 | 9.86 |
|  | Collection efficiency (%) | 98 | 98 | 98 |
|  | Approved |  |  |  |
| B | Particulars | 2019-20 | 2020-21 | 2021-22 |
|  | Collection efficiency (%) | 98 | 99 | 99 |
|  | AT &C losses | 10.7 | 9.47 | 9.15 |
|  | Revised Estimate |  |  |  |
| C | Particulars | 2019-20 | 2020-21 | 2021-22 |
|  | Energy Requirement (MU) | 26,135.55 | 26,673.99 | 27,629.24 |
|  | Transmission loss (MU) | 953.95 | 946.93 | 953.21 |
|  | Auxiliary consumption | 16.92 | 17.76 | 18.65 |
|  | Net Energy input (MU) (A-B) | 25,164.68 | 25,709.30 | 26,657.38 |
|  | Energy Sold (MU) | 22,966.11 | 23,545.45 | 24,498.87 |
|  | Distribution loss(MU) | 2,198.57 | 2,163.85 | 2,158.51 |
|  | Distribution loss (%) | 8.74 | 8.42 | 8.10 |
|  | Collection efficiency | 98 | 98 | 98 |
|  | Units realised | 22,506.79 | 23,074.54 | 24,008.89 |
|  | Units unrealised (MU) | 2,657.89 | 2,634.76 | 2,648.49 |
|  | AT&C loss (%) | 10.6 | 10.2 | 9.9 |

Revised Forecast of Energy for the control period

    - The consolidated sales projections for the control period from FY 20 to FY 22 are as shown below; It is proposed to reduce the T&D loss by a factor @0.4% for FY-20 to FY-22. The transmission loss is 3.65% and 3.55% and 3.45% for FY-20, FY-121 and FY-22 respectively.

    - Accordingly, the energy requirement at the generation points is computed. The average and peak load requirement is computed. Peak load requirement for the remaining control period is estimated based on past trends by taking 4 year CAGR after taking suitable correction factors.

| Table 10.22 : Forecast of Energy requirement and Peak Demand |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Particulars | FY-19* | FY-20 | FY-21 | FY-22 |
| 1 | Total energy sales projected | 21750.25 | 22,966.11 | 23,545.45 | 24,498.87 |
| 2 | T&D Loss In % | 12.47% | 12.07% | 11.67% | 11.27% |
| 3 | Energy at IF Points in MU | 24849.15 | 26118.63 | 26656.23 | 27610.59 |
| 4 | Substation auxiliary Consumption | 16.11 | 16.9155 | 17.76128 | 18.64934 |
| 5 | Energy at Gen. Points in MU | 24865.25 | 26135.5473 | 26673.99 | 27629.24 |
| 6 | Average MW Requirement | 2838.50 | 2983.51 | 3044.98 | 3154.02 |
| 7 | Peak MW requirement @.668% LF | 4242 | 4482.34 | 4597.02 | 4761.65 |
|  | Peak demand expected | 4242 | 4400 | 4500.34 | 4635.35 |

10.5	Norms for Operation for the control period 2018-19 to 2021-22

Supply Availability

10.5.1	KSEBL has taken earnest efforts to ensure supply availability at 100% as detailed in Format D.6.1. The supply availability for the next control period is given in table below.

| Table 10.23 : Supply Availability |  |  |  |
| --- | --- | --- | --- |
| Particulars | 2019-20 | 2020-21 | 2021-22 |
| Supply availability | 100% | 100% | 100% |

10.6	Revised estimate for Aggregate Revenue Requirement of SBU-D for the control period 2018-19 to 2021-22

10.6.1	As per Regulation 75 of theKSERC (Terms and Conditions for Determination of Tariff) Regulations, 2018, the AggregateRevenue Requirement shall comprise of the following items of expenditure:-

(i)	Cost of own power Generation

(ii)	Cost of Power Purchase

(iii) 	Transmission charges

(iv)	NLDC/RLDC/SLDC charges if any

(v)	Operation and maintenance expenses;

(vi)	Interest and finance-charges;

(vii) 	Depreciation;

(viii)	Interest on working capital and on security deposits and deposits from users of the distribution system;

(ix) 	Contribution to contingency reserves; and

(x) 	actual write off, if any approved by the Commission;

(xi)	Return on Equity

(xii) 	Revenue surplus or revenue gap on account of truing up of accounts for previousyears and carrying costs if any approved by the Honorable Commission

Following items, if any, shall be deducted from thegross amount of Aggregate Revenue Requirement computed as specifiedabove:-

(i) 	Non-tariff income;

(ii) 	Other income

(iii)		Income from wheeling charges recovered from open access consumers;

(iv) 	Income from other business, to the extent specified in theseRegulations.

(v)	Receipts on account of cross subsidy surcharge on wheeling charges from open access consumers; and

(vi)	Receipts on account of additional surcharge on wheeling charges from open access consumers

Based on the above Principles, the estimation of cost are made as under:

10.7	 Cost of Internal Generation

10.7.1	The total energy required for meeting the electricity demand for the control period and corresponding T&D losses and auxiliary consumption is given in table below.The month wise peak demand is estimated based on 4year CAGR.The revised energy requirement and peak demand requirement for the next control period is given in table below:

| Table 10. 24 : Energy Requirement for FY 2019-22 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Particulars | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| Energy sales(MU) | 21750.25 | 22,966.11 | 23,545.45 | 24,498.87 |
| T&D loss in % | 12.47% | 12.07% | 11.67% | 11.27% |
| Substation auxiliary Consumption(MU) | 24849.15 | 26118.6318 | 26656.23 | 27610.59 |
| Energy requirement (MU) | 24865.25 | 26135.5473 | 26673.99 | 27629.24 |
| Peak Demand Requirement (MW) | 4242 | 4400 | 4500.34 | 4635.35 |

10.7.2	The above requirement will be met through own generation plants by SBU-G and purchase of power from various sources. The details of revised energy generation and cost of generation is given in Chapter 2 of the petition. The source-wise details of own generation are given in table below.

| Table 10. 25 : Net Generation from own Stations (MU) and Cost (Rs Cr) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Source | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| Hydel(net) | 7570.6 | 6449.13 | 6638.85 | 6799.36 |
| Thermal | 2.66 | 11.91 |  |  |
| Solar | 18.54 | 22.57 | 47.60 | 47.60 |
| Wind |  | 1.33 | 1.37 | 1.37 |
| Aux cons of thermal and wind | 1.33 | 1.56 | 1.62 | 1.62 |
| Total | 7593.12 | 6483.38 | 6686.20 | 6846.71 |
| Cost (Rs Cr) |  | 486.37 | 542.98 | 671.82 |

10.7.3	As indicated in the chapter 7 there was generation of 2.66 MU during 2018-19 and 11.91 MU during 2019-20 from thermal power plants inorder to meet the demand. It is expected that no energy from thermal stations will need to be used for the remaining part of control period. Thermal stations will be reserved for use only during extreme exigencies.

10.8 Cost of power purchase

10.7.4	The entire energy requirement of the state cannot be met from own generation alone. Balance requirement is met bypurchasing power from different sources such as Central Generating Stations (CGS), Independent power producers (IPP)within as well as outside the State,traders and powerexchanges. KSEBL has entered into several long term agreements which have been approved by the Hon Commission.

The revised forecast or projection is made based on the following factors and assumptions:

- Forecast of purchase of power from different sources and cost are based oncontracts and Merit Order Dispatch Principle except renewable energy purchase.

- The transmission charges payable to the ISTS operator PGCIL are only considered for cost forecast, in addition to fixed and variable charges payable to CGS/ IPPs / Traders. Other supplementary claims will be claimed during the time of truing up based on actual figures.

- The revised forecast for FY 2020 was estimated based on the actual quantum and cost for the firsthalf of FY 2020 and based on the actual outage schedule for 2019-20 and terms of contract.

- The fixed charges of FY 2019-20 are taken considering the actual of first half of FY-2020. Honorable CERC is yet to approve the tariff of central generating stations based on the tariff regulation for the control period 2019-2024.The fixed charges for FY 2019-20 are estimated at the annual fixed charges approved by Honorable CERC for the year 2018-19 .Hence KSEBL humbly requests the variation in fixed charges at the time of truing up regarding tariff revision based on new Tariff Regulations.

- In the MYT petition fixed charges has been estimated by considering 2 % reduction of fixed charges from FY 2018  expecting that fixed charges will be reduced on account of the reduction in return on equity specified  in Draft Tariff Regulations 2019. Honorable CERC has not reduced rate of return on equity as specified in Draft Regulations.Hence 4 %increase is considered from FY 2018-19 for estimating the fixed charges of FY-21.

- The Government of India, Ministry of Environment, Forest& Climate Change (MOEFCC) vide its Notification No. S.O. 3305(E) dated 7.12.2015 notified the Environment (Protection) Amendment Rules, 2015 (hereinafter called „the MOEFCC Notification‟) amending/introducing the standards for emission of environmental pollutants to be followed by all existing as well as future thermal power plants. By the said Notification, all thermal power plants are mandatorily required to comply with the revised norms within the period of two years from the date of the said Notification. By the said amendment, MOEFCC has revised the norms as detailed hereunder:

- Revised the emission parameters for Particulate Matter for thermal power plants;

- Introduced additional parameters/ limits for thermal power plants -

- Emission norms for Sulphur dioxide (SO), Oxides of Nitrogen(NO)& Mercury(Hg),

ii)	      Amount of cooling water to be used per unit

- Directed all thermal plants with Once-Through Cooling (OTC) to install Cooling Tower(CT)

- In order to comply with the revised norms the thermal power plants is required to install ECS, which will result in-

(a)	Additional Capital expenditure because of installation of Emission Control system (ECS) required meeting the revised norms

(b)Increase in O & M expenses of the power station on account of installation of ECS to meet the revised norms

(c)Increase in auxiliary consumption of the power station due to installation of ECS

(d)Disruption in power generation during the installation phase of the above equipment

(e)Issue of fixed cost recovery for shut down period

- The implementation of Flue Gas Desulphurisation (FGD) in the existing and upcoming thermal power plants to curb SOx emissions is expected to complete by FY 2021-22. The cost of FGDD is estimated to be 0.4 to 0.5 Cr per MW. The fixed charges are likely to increase by about 30 paise due to the above notification. Central Electricity Authority has published a schedule for phasing of FGD which is enclosed as Annexure-VII. As per the phasing of FGD, one unit of NLC Tuticorin Expansion may complete the implementation by June 2020. Hence additional fixed charge of 30 paise per unit for the energy purchase from corresponding unit is estimated in the year 2021-22. The remaining stations are implementing during the year 2021-22 and 2022-23. Honorable Commission may kindly approve the same at the time of truing up for the year FY 22.

- The variable charges for first half of FY-20 is taken as such and the average of variable charges of first half is of FY-20 is taken for the estimation of variable charges for the remaining period of FY-20.The variable charges of subsequent years are estimated by giving 3% increase on the rate of FY-2020. Transmission charges are arrived based on the average of actual PoC charges from April to September 2019. Transmission losses and transmission charges for subsequent years are taken at the same rate. Honorable Commission may please note that Honorable CERC issued Draft Central Electricity Regulatory Commission (Sharing of Inter State Transmission charges and Losses Regulations), 2019 in which the mechanism of point of Connection charges is significantly modified wherein major component of transmission charges  are derived based on the total quantum of long term contracts and medium term contracts at national level. It is estimated that there will be an increase of about 50 % in transmission charges for Kerala as per the methodology in draft regulations. As the regulation is yet to be notified, KSEBL humbly request before the Honorable Commission to consider the changes in transmission charges on account of change in Regulations at the time of truing up.

Power from Central Generating Stations

10.7.5	While estimating power from Central GeneratingStations,the total allocated share of power from CGS for the State is 1677.92 MW as on  Oct-19 and while estimation It was expected that about 35.92 MW of power, that is 4.49 % of 1 x 800 MW, would be additionally available fromNTPC Kudgi III unit from January 2019 onwards,43 MW of the power from Bhavini Nuclear Power Plant, (43 MW = 8.6 % of 1 x 500 MW) would be available from June 2019 onwards. The total capacity available from CGS projected vide MYT petitionfor the control periodis given in table below and the same is approved by Honorable Commission.

| Table 10. 26 : Capacity allocation from CGS (proposed and approved) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No | Station | Installed Capacity (MW) | Allocation (%) | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| 1 | RSPTS Stage I & II | 2100 | 11.67% | 245.07 | 245.07 | 245.07 | 245.07 |
| 2 | RSTPS Stage III | 500 | 12.20% | 61.00 | 61.00 | 61.00 | 61.00 |
| 3 | TALCHER - Stage II | 2000 | 21.35% | 427.00 | 427.00 | 427.00 | 427.00 |
| 4 | SimhadriExp | 1000 | 8.29% | 82.90 | 82.90 | 82.90 | 82.90 |
| 5 | NLC-II- Stage-1 | 630 | 10.00% | 63.00 | 63.00 | 63.00 | 63.00 |
| 6 | NLC-II- Stage-2 | 840 | 10.71% | 89.96 | 89.96 | 89.96 | 89.96 |
| 7 | NLC- Exp- Stage-1 | 420 | 15.93% | 66.91 | 66.91 | 66.91 | 66.91 |
| 8 | NLC - II Exp | 500 | 15.93% | 79.65 | 79.65 | 79.65 | 79.65 |
| 9 | MAPS | 440 | 5.23% | 23.01 | 23.01 | 23.01 | 23.01 |
| 10 | KAIGA Stg I | 440 | 8.64% | 38.02 | 38.02 | 38.02 | 38.02 |
| 11 | KAIGA Stg II | 440 | 7.96% | 35.02 | 35.02 | 35.02 | 35.02 |
| 12 | Kudamkulam Unit I | 1000 | 13.91% | 139.10 | 139.10 | 139.10 | 139.10 |
| 13 | Kudamkulam unit II | 1000 | 13.30% | 133.00 | 133.00 | 133.00 | 133.00 |
| 14 | Vallur JV with | 1500 | 3.33% | 49.95 | 49.95 | 49.95 | 49.95 |
| 15 | NTPL(Tuticorin JV) | 1000 | 7.25% | 72.50 | 72.50 | 72.50 | 72.50 |
| 16 | Kudgi Unit I | 800 | 4.49% | 35.92 | 35.92 | 35.92 | 35.92 |
| 17 | Kudgi Unit II | 800 | 4.49% | 35.92 | 35.92 | 35.92 | 35.92 |
| 18 | Kudgi Unit III | 800 | 4.49% | 35.92 | 35.92 | 35.92 | 35.92 |
| 19 | Bhavini | 800 | 8.60% |  | 43.00 | 43.00 | 43.00 |
|  | Total |  |  | 1,713.85 | 1,756.85 | 1,756.85 | 1,756.85 |

10.7.6	However, 43 MW from Bhavini nuclear power plant was not available from June 2019. Power from Bhavini nuclear power plant is expected to be available from April 2020 onwards. In addition to the above, Unit-I of New Neyveli Thermal Power Plant (NNTPP) commissioned during December 2019 for which KSEBL share of allocation is 16.2 MW. Unit-II of NNTPP is expected to available from April 2020 which enhances the availability by another 16.2 MW.

10.7.7	As per tariff regulations for the control period from FY 2019-2024 issued Hon’ble CERC, the normative plant load factor, plant availability factor, auxiliary consumptionand gross station heat rate of certain Central Generating Stations are revised and the details are given in table below

| Table-10.27 Revised capacity allocation from CGS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Station |  |  | Allocated Capacity to KSEBL (MW) |  |  | Aux Consumption (%) | Target PLF |
|  |  | IC | Allocation | 2019-20 | 2020-21 | 2021-22 |  |  |
|  |  | MW | % | MW | MW | MW |  |  |
| 1 | RSPTS Stage I & II | 2100 | 11.67% | 245.07 | 245.07 | 245.07 | 7.04% | 85.00% |
| 2 | RSTPS Stage III | 500 | 12.20% | 61.00 | 61.00 | 61.00 | 6.25% | 85.00% |
| 3 | TALCHER - Stage II | 2000 | 21.35% | 427.00 | 427.00 | 427.00 | 6.25% | 85.00% |
| 4 | Simhadri Exp | 1000 | 8.56% | 85.55 | 85.55 | 85.55 | 5.75% | 85.00% |
| 5 | NLC-II- Stage-1 | 630 | 10.00% | 63.00 | 63.00 | 63.00 | 10.00% | 85.00% |
| 6 | NLC-II- Stage-2 | 840 | 10.72% | 90.02 | 90.02 | 90.02 | 10.00% | 85.00% |
| 7 | NLC- Exp- Stage-1 | 420 | 16.25% | 68.24 | 68.24 | 68.24 | 8.50% | 85.00% |
| 8 | NLC - II Exp | 500 | 16.25% | 81.23 | 81.23 | 81.23 | 10.00% | 75.00% |
| 9 | MAPS | 440 | 5.23% | 23.01 | 23.01 | 23.01 | 10.50% | 68.50% |
| 10 | KAIGA Stg I | 440 | 8.64% | 38.01 | 38.01 | 38.01 | 10.50% |  |
| 11 | KAIGA Stg II | 440 | 7.95% | 34.98 | 34.98 | 34.98 | 10.50% |  |
| 12 | Kudamkulam Unit I | 1000 | 14.01% | 140.10 | 140.10 | 140.10 | 7.80% |  |
| 13 | Kudamkulam unit II | 1000 | 13.30% | 133.00 | 133.00 | 133.00 | 7.80% |  |
| 14 |  | Vallur STPS |  | 1500 |  | 3.33% |  | 49.95 |  | 49.95 |  | 49.95 |  | 7.19% |  | 85.00% |  |
| 15 | NTPL(Tuticorin JV) | 1000 | 7.25% | 72.50 | 72.50 | 72.50 | 6.25% | 85.00% |
| 16 | Kudgi Unit I,II & III | 2400 | 4.66% | 111.72 | 111.72 | 111.72 | 6.25% | 85.00% |
| 17 | Bhavini | 500 | 8.60% |  | 43.00 | 43.00 | 10.00% | 85.00% |
| 18 | NNTPP | 1000 | 3.24% | 16.2 | 32.4 | 32.4 | 6.25% |  |
|  | Total |  |  | 1740.58 | 1799.62 | 1799.62 |  |  |

10.7.8  The actual energy received from Central Generating Stations for the year 2018-19 is given in table below:

| Table-10.28 Energy purchased from CGS in MU during 2018-19 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sl No | Stations | Energy at exbus | External loss | Energy at Kerala periphery |
| 1 | NTPC RSPTS Stage I&II | 1608.17 | 66.88 | 1541.29 |
| 2 | NTPC RSPTS StageIII | 400.76 | 16.56 | 384.20 |
| 3 | Talcher II | 2889.61 | 121.02 | 2768.59 |
| 4 | Simhadri STPS Stg IIUnit I | 541.92 | 22.39 | 519.53 |
| 5 | NTPC KUDGI | 458.51 | 19.04 | 439.47 |
| 6 | NLC II stage I | 396.01 | 16.54 | 379.46 |
| 7 | NLC II stage II | 586.18 | 24.46 | 561.72 |
| 8 | NLC I Expansion | 429.78 | 17.76 | 412.02 |
| 9 | NLC II Expansion | 281.08 | 11.58 | 269.50 |
| 10 | Vallur STPS | 267.73 | 11.13 | 256.60 |
| 11 | NTPL | 420.25 | 17.32 | 402.93 |
| 12 | MAPS | 60.72 | 2.52 | 58.20 |
| 13 | Kaiga Stage I&II | 541.76 | 22.64 | 519.12 |
| 14 | Kudankulam | 759.53 | 31.07 | 728.46 |
|  | Sub Total | 9642.01 | 400.92 | 9241.09 |

10.7.9	The revised energy availability for the year 2019-20 was estimated by taking the actual of first half of FY 2019-20 and considering the outages anticipated during second half of FY-2020 and considering the actual PLF during FY-19.

10.7.10	The revised energy availability from CGS was estimated for the rest of the  control period after making judicious assumptions for actual PLF during 2018-19,and revised auxiliary consumption considering the operational Norms specified in CERC (Terms and Conditions of Tariff) Regulations, 2019 and  actual outage schedule.

10.7.11	The revised energy availability for the remaining part of control period is given in table below:

| Table 10.29 Energy procurement from CGS for FY 20 to FY 22 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | FY 20 |  |  | FY 21 & FY 22 |  |  |
| No | Station | Allocation (MW) | Aux Consumption (%) | Expected PLF | Exbus energy (MU) | ISTS losses (MU) | Energy at KSEB periphery (MU) | Energy at exbus (MU) | ISTS losses (MU) | Energy at KSEB periphery (MU) |
| 1 | RSPTS I & II | 245.07 | 7.04% | 85% | 1768.46 | 66.83 | 1701.63 | 1696.33 | 66.50 | 1629.83 |
| 2 | RSTPS Stg III | 61 | 6.25% | 85% | 485.54 | 18.41 | 467.13 | 425.82 | 16.69 | 409.13 |
| 3 | TALCHER - Stg II | 427 | 6.25% | 85% | 2854.92 | 113.48 | 2741.44 | 2980.73 | 124.30 | 2856.43 |
| 4 | SimhadriExp | 85.55 | 5.75% | 80% | 600.88 | 22.03 | 578.85 | 565.06 | 20.74 | 544.32 |
| 5 | NLC-II- Stg-1 | 63 | 10.00% | 85% | 434.94 | 16.41 | 418.53 | 422.19 | 16.55 | 405.64 |
| 6 | NLC-II- Stg-2 | 90.02 | 10.00% | 85% | 620.15 | 23.56 | 596.59 | 603.26 | 23.65 | 579.61 |
| 7 | NLC- Exp- Stg-1 | 68.24 | 8.50% | 85% | 511.29 | 19.21 | 492.08 | 464.90 | 18.22 | 446.67 |
| 8 | NLC- II ExpStg II | 81.23 | 10.00% | 70% | 369.17 | 14.02 | 355.14 | 480.33 | 18.83 | 461.50 |
| 9 | MAPS | 23.012 | 10.50% | 70% | 79.29 | 2.89 | 76.40 | 126.29 | 4.63 | 121.65 |
| 10 | KAIGA Stg I& II | 72.99 | 10.50% | 70% | 530.64 | 22.49 | 508.14 | 400.58 | 19.71 | 380.87 |
| 11 | Kudamkulam I& II | 273.1 | 7.80% | 70% | 1325.29 | 51.37 | 1273.92 | 1544.02 | 64.39 | 1479.64 |
| 12 | Vallur JV with | 49.95 | 7.19% | 70% | 320.61 | 11.01 | 309.60 | 345.19 | 10.94 | 334.25 |
| 13 | NTPL(Tuticorin JV) | 72.5 | 6.25% | 70% | 435.47 | 17.83 | 417.64 | 506.10 | 22.37 | 483.73 |
| 14 | Kudgi Unit I II & III | 111.72 | 6.25% | 85% | 580.44 | 23.75 | 556.69 | 779.88 | 32.52 | 747.36 |
| 15 | Bhavini | 43 | 10.00% | 75% | 0 | 0.00 | 0 | 237.31 | 8.71 | 228.60 |
| 16 | NNTPP | 32.4 | 6.25% | 75% | 24.42 | 0.96 | 23.46 | 226.17 | 8.87 | 217.31 |
|  | Total |  |  |  | 10941.50 | 424.26 | 10517.24 | 11804.16 | 477.61 | 11326.55 |

10.7.12	As shown in table above the net energy availability for the year 2019-20 was projected at 10517.24 MU and for FY 2020-21 and FY 2021-22 at 11326.55 MU at Kerala periphery against the approved quantum of 11241.94 MU and 11274.76 MU respectiively.

10.7.13	On analyzing the variable charges of first half of FY 2020 it is seen that the variable charges has increased considerably from that of FY-19. The details are given below.

| Table 10. 30 Variable cost of CGS April to September 2019 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | Approved for FY 2020 | April | May | June | July | August | September | Average |
|  |  | Rs/kWh | Rs/kWh | Rs/kWh | Rs/kWh | Rs/kWh | Rs/kWh | Rs/kWh |
| RSTPS I & II | 2.49 | 2.73 | 2.55 | 2.62 | 2.73 | 2.62 | 2.61 | 2.64 |
| RSTPS III | 2.45 | 2.68 | 2.50 | 2.50 | 2.66 | 2.66 | 2.73 | 2.62 |
| Talcher Stage-II | 1.73 | 1.85 | 2.00 | 1.92 | 1.84 | 2.19 | 2.29 | 1.98 |
| Simhadri TPS Stage II | 2.89 | 3.37 | 3.22 | 3.20 | 3.18 | 3.12 | 3.36 | 3.24 |
| NLC Stage-I | 2.64 | 2.64 | 2.64 | 2.64 | 2.66 | 2.64 | 2.64 | 2.64 |
| NLC Stage-II | 2.64 | 2.64 | 2.64 | 2.64 | 2.66 | 2.64 | 2.64 | 2.64 |
| NLC 1 Expansion | 2.44 | 2.42 | 2.43 | 2.41 | 2.42 | 2.42 | 2.43 | 2.42 |
| NLC II Expansion | 2.44 | 2.48 | 2.48 | 2.48 | 2.48 | 2.48 | 2.48 | 2.48 |
| Vallur STPS | 3.07 | 3.81 | 3.77 | 4.04 | 3.94 | 3.65 | 3.78 | 3.86 |
| MAPS | 2.78 | 2.74 | 2.74 | 2.74 | 2.74 | 2.74 | 2.74 | 2.74 |
| KAIGA | 3.71 | 3.64 | 3.64 | 3.65 | 3.65 | 3.65 | 3.65 | 3.65 |
| Kudankulam | 4.19 | 4.11 | 4.11 | 4.10 | 4.10 | 4.12 | 4.11 | 4.11 |
| NTPL | 3.13 | 3.44 | 3.55 | 3.55 | 3.07 | 2.80 | 3.10 | 3.21 |
| Kudgi | 3.91 | 3.83 | 3.70 | 3.80 | 3.90 | 4.07 | 4.11 | 3.83 |

10.7.14	The variable cost increase of first half of FY 20 is more than 5 % from that of FY-19 in most of Central Generating stations except MAPS, KAIGA and Kudamkulam and NLC Stations. The main reasons for the increase in variable charges compared to approved value is due to following reasons.There was revision in normative station heat rate and auxiliary consumption for all Central Generating Stations and stations whose tariff is determined by CERC, through notification of Central Electricity Regulatory Commission (Terms and Conditions of Tariff) Regulations, 2019 dated 07.03.2019, with effect from 01.04.2019, due to which energy charges increased considerably compared to last quarter of previous year. The energy charge rate calculated as per Tariff regulations is based on the following formula.

Energy charge rate= {(GHR-SFC*CVSF)*LPPF/CVPF+ (SFC*LPSFi) + (LC*LPL}*100/ (100-Aux))

Where, GHR is Gross station heat rate in Kcal per kWh and Aux=Normative auxiliary energy consumption in %. Thus, increase of normative auxiliary consumption and station heat rate results in increase of the rate of energy charge.

10.7.15	The variation of energy charges also depends on the price notified for each mine which are different from mine to mine, grade of coal, blending ratio of imported coal and domestic coal, distance from source (Coal mines), mode of transport etc.During coal shortage periods the Generating Companies resorted to imported coal and mines other than that of linkage mines which resulted in increase in variable cost.

10.7.16	The variable cost of FY 20 for CGS other than NLC stations  is estimated by taking actual variable charges for first half of FY 2019-20 and considering the same average cost for estimating the variable charges of second half of FY 2020. NLC India Limited issued revised Lignite transfer price guidelines 2019-24 dated 17.09.2019 wef October 2019 and as a result there is an increase in energy charges from October2019 by about 5 %. Hence the average of the variable charges for the period October to December 2019 is taken for estimating the variable charges of second half of FY 2020.

10.7.17	The variable cost of CGS for FY 21 and FY-22 is estimated by considering an escalation of 3% in the variable cost of FY 20 on YoY basis as the variable cost increase of first half of FY 20 is more than 5 % from that of FY-19 in most of Central Generating stations owing to the shortage of coal. Hence considering the trend, an escalation of 3 % is uniformly considered for estimating the variable cost of of power purchase for the remaining control period.The variable charges of NLC stations for the period October to December 2019 is given in table below.

10.7.18	The additional fixed charges due to implementation of FGD for NTPL Tuticorin described in para 11.7.4 is given in table below

| Table 10. 31 Additional fixed charges due to implementation of FGD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Station | Unit capacity | Allocation | Allocated capacity | Ex bus | Addition to fixed charges due to FGD | Fixed charges estimated at 2 % increase | Total fixed charges |
|  | MW | % | MW | MU | Rs Cr | Rs Cr | Rs Cr |
| NTPL | 1000 | 7.25 | 72.5 | 506.10 | 15.18 | 83.87 | 99.06 |

10.7.19	Based on the above the variable and fixed charges are shown in below.

| Table 10. 32 Average variable cost and fixed cost for the control period |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Average Variable cost (Rs./Unit) |  |  |  | Fixed cost (Rs. Cr) |  |  |  |
|  | Actual | Estimated |  |  |  |  |  |  |
| Station | 2018-19 | Oct-Mar 2019 | 2020-21 | 2021-22 | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| RSPTS Stage I & II | 2.53 | 2.64 | 2.72 | 2.80 | 123.66 | 124.67 | 129.70 | 132.30 |
| RSTPS Stage III | 2.47 | 2.62 | 2.70 | 2.78 | 33.76 | 33.24 | 34.58 | 35.27 |
| TALCHER - Stage II | 1.73 | 1.98 | 2.04 | 2.10 | 214.87 | 216.19 | 224.92 | 229.42 |
| Simhadri Exp | 2.90 | 3.24 | 3.34 | 3.44 | 103.23 | 92.54 | 96.28 | 98.20 |
| NLC-II- Stage-1 | 2.58 | 2.78 | 2.86 | 2.95 | 31.00 | 29.98 | 31.19 | 31.82 |
| NLC-II- Stage-2 | 2.59 | 2.78 | 2.86 | 2.95 | 46.21 | 44.40 | 46.19 | 47.12 |
| NLC- Exp- Stage-1 | 2.40 | 2.55 | 2.63 | 2.71 | 45.60 | 44.87 | 46.68 | 47.62 |
| NLC - II Exp | 2.38 | 2.60 | 2.68 | 2.76 | 66.64 | 74.70 | 77.71 | 79.27 |
| MAPS | 2.64 | 2.74 | 2.83 | 2.91 |  | 0.00 |  |  |
| KAIGA Stg I & II | 3.64 | 3.65 | 3.76 | 3.87 |  | 0.00 | 0.00 |  |
| Kudamkulam Unit I& II | 4.05 | 4.11 | 4.23 | 4.36 |  | 0.00 | 0.00 |  |
| Vallur JV | 3.01 | 3.86 | 3.97 | 4.09 | 64.49 | 61.90 | 64.40 | 65.69 |
| NTPL(Tuticorin JV) | 3.17 | 3.21 | 3.30 | 3.40 | 74.20 | 79.04 | 82.23 | 99.06 |
| Kudgi Unit I,II & III | 3.84 | 3.83 | 3.95 | 4.07 | 100.02 | 121.67 | 126.58 | 129.11 |
| Bhavni |  |  | 4.23 | 4.36 |  | 0.00 |  | 0.00 |
| NNTPP |  | 2.39 | 2.46 | 2.54 |  | 4.84 | 40.81 | 41.63 |
| Nuclear power stations are having single part tariff, for the purpose of estimating the power purchase cost, 3% escalation year on year is allowed |  |  |  |  |  |  |  |  |

10.7.20	The revised power purchase cost for the remaining control period from Central Generating Stations is given in table below

| Table 10.33 Energy procurement from CGS for FY-20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Station |  |  | Allocation (MW) |  | Aux Consumption (%) |  | Energy at exbus (MU) |  | ISTS losses (MU) |  | Net Energy at KSEB periphery (MU) |  | Fixed Charges(Rs Cr) |  | Variable Charges(Rs Cr) |  | Total(Rs Cr) |
| 1 | RSPTS Stage I & II |  |  | 245.07 |  | 6.25% |  | 1768.46 |  | 66.83 |  | 1701.63 |  | 124.67 |  | 467.09 |  | 591.76 |
| 2 | RSTPS Stage III |  |  | 61.00 |  | 6.25% |  | 485.54 |  | 18.41 |  | 467.13 |  | 33.24 |  | 127.13 |  | 160.37 |
| 3 | TALCHER - Stage II |  |  | 427.00 |  | 5.75% |  | 2854.92 |  | 113.48 |  | 2741.43 |  | 216.19 |  | 564.91 |  | 781.10 |
| 4 | SimhadriExp |  |  | 85.55 |  | 10.00% |  | 600.88 |  | 22.03 |  | 578.85 |  | 92.54 |  | 194.68 |  | 287.22 |
| 5 | NLC-II- Stage-1 |  |  | 63.00 |  | 10.00% |  | 434.94 |  | 16.41 |  | 418.53 |  | 29.98 |  | 117.83 |  | 147.81 |
| 6 | NLC-II- Stage-2 |  |  | 90.02 |  | 8.50% |  | 620.15 |  | 23.56 |  | 596.59 |  | 44.40 |  | 168.51 |  | 212.91 |
| 7 | NLC- Exp- Stage-1 |  |  | 68.24 |  | 10.00% |  | 511.29 |  | 19.21 |  | 492.08 |  | 44.87 |  | 127.13 |  | 172.00 |
| 8 |  | NLC - II Exp | 81.23 |  | 10.50% |  | 369.17 |  | 14.02 |  | 355.14 |  | 74.70 |  | 94.58 |  | 169.28 |  |
| 9 |  | MAPS | 23.01 |  | 10.50% |  | 79.29 |  | 2.89 |  | 76.40 |  | 0.00 |  | 21.75 |  | 21.75 |  |
| 10 |  | KAIGA Stg I& II | 72.99 |  | 7.80% |  | 530.64 |  | 22.49 |  | 508.14 |  | 0.00 |  | 193.52 |  | 193.52 |  |
| 11 |  | Kudamkulam Unit I& II | 273.10 |  | 7.80% |  | 1325.29 |  | 51.37 |  | 1273.92 |  | 0.00 |  | 544.64 |  | 544.64 |  |
| 12 |  | Vallur JV with | 49.95 |  | 6.25% |  | 320.61 |  | 11.01 |  | 309.60 |  | 61.90 |  | 123.67 |  | 185.58 |  |
| 13 |  | NTPL(Tuticorin JV) | 72.50 |  | 6.25% |  | 435.47 |  | 17.83 |  | 417.64 |  | 79.04 |  | 139.60 |  | 218.63 |  |
| 14 |  | Kudgi Unit I II & III | 111.72 |  | 10.00% |  | 580.44 |  | 23.75 |  | 556.69 |  | 121.67 |  | 222.60 |  | 344.26 |  |
| 15 |  | Bhavini |  |  | 6.25% |  | 0.00 |  | 0.00 |  | 0.00 |  | 0.00 |  | 0.00 |  | 0.00 |  |
| 16 |  | NNTPP | 16.20 |  | 0.00% |  | 24.42 |  | 0.96 |  | 23.46 |  | 4.84 |  | 5.84 |  | 10.68 |  |
|  |  | Total | 1740.38 |  |  |  | 10941.50 |  | 424.26 |  | 10517.24 |  | 928.03 |  | 3113.49 |  | 4041.51 |  |

| Table 10.34 Energy procurement from CGS for FY-21 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Station | Allocation (MW) | Aux Consumption (%) |  | Energy at exbus (MU) | ISTS losses (MU) | Net Energy at KSEB periphery (MU) | Fixed Charges(Rs Cr) | Variable Charges(Rs Cr) | Total(Rs Cr) |
| 1 | RSPTS Stage I & II | 245.07 | 6.25% | 1696.33 |  | 66.50 | 1629.83 | 129.70 | 461.48 | 591.19 |
| 2 | RSTPS Stage III | 61.00 | 6.25% | 425.82 |  | 16.69 | 409.13 | 34.58 | 114.84 | 149.42 |
| 3 | TALCHER - Stage II | 427.00 | 5.75% | 2980.73 |  | 124.30 | 2856.43 | 224.92 | 607.50 | 832.42 |
| 4 | SimhadriExp | 85.55 | 10.00% | 565.06 |  | 20.74 | 544.32 | 96.28 | 188.56 | 284.84 |
| 5 | NLC-II- Stage-1 | 63.00 | 10.00% | 422.19 |  | 16.55 | 405.64 | 31.19 | 120.89 | 152.08 |
| 6 | NLC-II- Stage-2 | 90.02 | 8.50% | 603.26 |  | 23.65 | 579.61 | 46.19 | 172.74 | 218.93 |
| 7 | NLC- Exp- Stage-1 | 68.24 | 10.00% | 464.90 |  | 18.22 | 446.67 | 46.68 | 122.10 | 168.79 |
| 8 | NLC - II Exp | 81.23 | 10.50% | 480.33 |  | 18.83 | 461.50 | 77.71 | 128.63 | 206.35 |
| 9 | MAPS | 23.01 | 10.50% | 126.29 |  | 4.63 | 121.65 | 0.00 | 35.68 | 35.68 |
| 10 | KAIGA Stg I& II | 72.99 | 7.80% | 400.58 |  | 19.71 | 380.87 | 0.00 | 150.47 | 150.47 |
| 11 | Kudamkulam U I& II | 273.10 | 7.80% | 1544.02 |  | 64.39 | 1479.64 | 0.00 | 653.56 | 653.56 |
| 12 | Vallur JV with | 49.95 | 6.25% | 345.19 |  | 10.94 | 334.25 | 64.40 | 137.15 | 201.55 |
| 13 | NTPL(Tuticorin JV) | 72.50 | 6.25% | 506.10 |  | 22.37 | 483.73 | 82.23 | 167.11 | 249.34 |
| 14 | Kudgi Unit I II & III | 111.72 | 10.00% | 779.88 |  | 32.52 | 747.36 | 126.58 | 308.06 | 434.64 |
| 15 | Bhavini | 43.00 | 6.25% | 237.31 |  | 8.71 | 228.60 | 0.00 | 100.45 | 100.45 |
| 16 | NNTPP | 32.40 | 6.25% | 226.17 |  | 8.87 | 217.31 | 40.81 | 55.70 | 96.51 |
|  | Total | 1799.78 |  | 11804.16 |  | 477.61 | 11326.55 | 1001.29 | 3524.92 | 4526.22 |

| Table 10.35 Energy procurement from CGS for FY-22 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Station | Allocation (MW) | Aux Consumption (%) | Energy at exbus (MU) | ISTS losses (MU) | Energy at KSEB periphery (MU) | Fixed Charges(Rs Cr) | Variable Charges(Rs Cr) | Total(Rs Cr) |  |
| 1 | RSPTS Stage I & II | 245.07 | 6.25% | 1696.33 | 66.50 | 1629.83 | 132.30 | 475.33 | 607.63 |
| 2 | RSTPS Stage III | 61 | 6.25% | 425.82 | 16.69 | 409.13 | 35.27 | 118.28 | 153.55 |
| 3 | TALCHER - Stage II | 427 | 5.75% | 2980.73 | 124.30 | 2856.43 | 229.42 | 625.72 | 855.14 |
| 4 | SimhadriExp | 85.55 | 10.00% | 565.06 | 20.74 | 544.32 | 98.20 | 194.22 | 292.42 |
| 5 | NLC-II- Stage-1 | 63 | 10.00% | 422.19 | 16.55 | 405.64 | 31.82 | 124.52 | 156.33 |
| 6 | NLC-II- Stage-2 | 90.02 | 8.50% | 603.26 | 23.65 | 579.61 | 47.12 | 177.92 | 225.03 |
| 7 | NLC- Exp- Stage-1 | 68.24 | 10.00% | 464.90 | 18.22 | 446.67 | 47.62 | 125.77 | 173.38 |
| 8 | NLC - II Exp | 81.23 | 10.50% | 480.33 | 18.83 | 461.50 | 79.27 | 132.49 | 211.76 |
| 9 | MAPS | 23.012 | 10.50% | 126.29 | 4.63 | 121.65 | 0.00 | 36.75 | 36.75 |
| 10 | KAIGA Stg I& II | 72.99 | 7.80% | 400.58 | 19.71 | 380.87 | 0.00 | 154.98 | 154.98 |
| 11 | Kudamkulam U I& II | 273.1 | 7.80% | 1544.02 | 64.39 | 1479.64 | 0.00 | 673.17 | 673.17 |
| 12 | Vallur JV with | 49.95 | 6.25% | 345.19 | 10.94 | 334.25 | 65.69 | 141.26 | 206.96 |
| 13 | NTPL(Tuticorin JV) | 72.5 | 6.25% | 506.10 | 22.37 | 483.73 | 99.06 | 172.12 | 271.18 |
| 14 | Kudgi Unit I II & III | 111.72 | 10.00% | 779.88 | 32.52 | 747.36 | 129.11 | 317.30 | 446.41 |
| 15 | Bhavini | 43 | 6.25% | 237.31 | 8.71 | 228.60 | 0.00 | 103.46 | 103.46 |
| 16 | NNTPP | 32.40 | 6.25% | 226.17 | 8.87 | 217.31 | 41.63 | 57.37 | 99.00 |
|  | Total | 1799.78 |  | 11804.16 | 477.61 | 11326.55 | 1036.50 | 3630.67 | 4667.17 |

10.7.21	In addition to the fixed cost and variable energy charges the beneficiary utilities are liable to pay incentive, income tax,water cess ,foreign exchange rate variation etc . However these expenses are not taken into the present forecast and the same may be allowed at the time of truing up. The revised power purchase cost from central Generating Stations for the remaining part of control period is given in table below.

| Table 10. 36 Total revised Power Purchase cost for CGS for the control period (Rs Cr) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station |  |  |  |  |  |
|  | 2018-19 | April to Sept2019 | 2019-20 | 2020-21 | 2021-22 |
| RSPTS Stage I & II | 530.31 | 312.00 | 591.76 | 591.19 | 607.63 |
| RSTPS Stage III | 132.78 | 81.98 | 160.37 | 149.42 | 153.55 |
| TALCHER - Stage II | 715.24 | 342.25 | 781.10 | 832.42 | 855.14 |
| Simhadri Exp | 260.58 | 153.76 | 287.22 | 284.84 | 292.42 |
| NLC-II- Stage-1 | 127.61 | 74.84 | 147.81 | 152.08 | 156.33 |
| NLC-II- Stage-2 | 189.00 | 96.61 | 212.91 | 218.93 | 225.03 |
| NLC- Exp- Stage-1 | 151.66 | 83.55 | 172.00 | 168.79 | 173.38 |
| NLC - II Exp | 133.64 | 56.33 | 169.28 | 206.35 | 211.76 |
| MAPS | 16.63 | 10.09 | 21.75 | 35.68 | 36.75 |
| KAIGA Stg I &II | 201.73 | 98.86 | 193.52 | 150.47 | 154.98 |
| Kudamkulam Unit I&II | 330.14 | 288.79 | 544.64 | 653.56 | 673.17 |
| Vallur JV with | 145.30 | 85.49 | 185.58 | 201.55 | 206.96 |
| NTPL(Tuticorin JV) | 207.58 | 88.49 | 218.63 | 249.34 | 271.18 |
| Kudgi Unit I,II,III | 275.99 | 117.30 | 344.26 | 434.64 | 446.41 |
| Bhavini |  | 0.00 | 0.00 | 100.45 | 103.46 |
| NNTPP |  | 0.00 | 10.68 | 96.51 | 99.00 |
| Total | 3,418.19 | 1,890.34 | 4,041.51 | 4,526.22 | 4,667.17 |

10.7.22	The summary of revised power purchase cost for Central Generating Station is given in table below.

| Table 10. 37 Summary of Power Purchase cost from CGS for the control period |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Allocation (MW) | Ex-bus Energy (MU) | ISTS losses (MU) | Net Energy (MU) | Fixed Cost (Rs. Crore) | Variable cost | Total cost | Avg.Rate (Rs/Unit) |
|  |  |  |  |  |  | (Rs. Crore) | (Rs. Crore) |  |
| Approved by the Commission |  |  |  |  |  |  |  |  |
| 2019-20 | 1,756.95 | 11,672.89 | 430.94 | 11,241.96 | 892.01 | 3,227.96 | 4,135.05 | 3.54 |
| 2020-21 | 1,756.95 | 11,706.79 | 432.02 | 11,274.77 | 874.17 | 3,307.00 | 4,196.25 | 3.58 |
| 2021-22 | 1,756.95 | 11,706.79 | 432.02 | 11,274.77 | 856.69 | 3,373.14 | 4,244.91 | 3.63 |
| As per Revised Estimate |  |  |  |  |  |  |  |  |
| 2019-20 | 1756.76 | 10941.50 | 424.26 | 10517.24 | 928.03 | 3113.49 | 4041.51 | 3.69 |
| 2020-21 | 1799.76 | 11804.16 | 477.6103 | 11326.55 | 1001.29 | 3463.27 | 4,526.22 | 3.83 |
| 2021-22 | 1799.76 | 11804.16 | 477.6103 | 11326.55 | 1036.50 | 3567.17 | 4,667.17 | 3.95 |

Purchase through Long Term agreements

10.7.23	KSEB Ltd entered contracts with power producers for meeting the power requirements for the next 25 years for 450 MW at CERC approved rates with the approval of the Commission and another 865MW on DBFOO basis. Out of the 865MW, 100MW from M/s East Cost Energy Limited has not materialized.

10.7.24	Honorable Commission vide MYT order dated 08.07.2019 approved the power purchase through long term contracts as given in table below.

| Table 10. 38 Power Purchase through long term contracts approved by the Hon Commission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Trader/Source | contracted capacity (MW) | Aux. consumption (%) | Actual PLF | 2018-19 |  | 2019-20 to 2021-22 |  |
|  |  |  |  | Ex-bus Energy (MU) | Energy at KSEB periphery (MU) | Ex-bus Energy (MU) | Energy at KSEB periphery (MU) |
| Maithon power limited | 150 | 6.5 | 90% | 991.63 | 960.83 | 1,105.73 | 1,070.92 |
| Maithon power limited | 150 | 6.5 | 90% | 991.63 | 960.83 | 1,105.73 | 1,070.92 |
| DVC-Mejia TPS | 100 | 5.25 | 85% | 686.96 | 666.03 | 705.5 | 683.99 |
| DVC-Reghunathpur TPS | 50 | 6.5 | 85% | 244.94 | 237.45 | 348.1 | 337.49 |
| Jindal Power Limited | 200 | 5 | 90% | 1,486.29 | 1,420.67 | 1,497.96 | 1,431.81 |
| Jindal Power Limited | 150 | 5 | 90% | 1,117.19 | 1,067.86 | 1,123.47 | 1,073.86 |
| Jindal India Thermal Ltd | 100 | 5 | 90% | 695.25 | 669.93 | 748.98 | 721.9 |
| Jhabua Power Limited | 115 | 5 | 90% | 733.26 | 700.19 | 861.33 | 822.57 |
| Jhabua Power Limited | 100 | 5 | 90% | 625.27 | 597.06 | 748.98 | 715.28 |
| BALCO | 100 | 5 | 90% | 748.98 | 718.46 | 748.98 | 718.32 |
| TOTAL | 1215 |  |  | 8,321.40 | 7,999.32 | 8,994.76 | 8,647.06 |

10.7.25	The power purchase cost approved by the Honorable Commission for long term contracts is given in table below

| Table 10. 39 Power Purchase through long term contracts approved by the Hon Commission |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Trader/Source | contracted capacity (MW) | Aux. consumption (%) | PLF | Ex-bus Energy (MU) | Energy at KSEB periphery (MU) | Ex-bus Energy (MU) | Energy at KSEB periphery (MU) | Total Power Purchase cost(Rs Cr) |  |  |  |
|  |  |  |  | 2018-19 |  | 2019-20 to2021-22 |  | FY-19 | FY-20 | FY -21 | FY- 22 |
| Maithon power limited | 150 | 6.5 | 90% | 991.63 | 960.83 | 1,105.73 | 1,070.92 | 387.03 | 415.79 | 418.03 | 420.43 |
| Maithon power limited | 150 | 6.5 | 90% | 991.63 | 960.83 | 1,105.73 | 1,070.92 | 387.03 | 415.79 | 418.03 | 420.43 |
| DVC-Mejia TPS | 100 | 5.25 | 85% | 686.96 | 666.03 | 705.5 | 683.99 | 267.44 | 273.27 | 274.74 | 276.32 |
| DVC-Reghunathpur TPS | 50 | 6.5 | 85% | 244.94 | 237.45 | 348.1 | 337.49 | 113.29 | 136.92 | 137.38 | 137.89 |
| Jindal Power Limited | 200 | 5 | 90% | 1,486.29 | 1,420.67 | 1,497.96 | 1,431.81 | 554.67 | 555.28 | 551.58 | 548.13 |
| Jindal Power Limited | 150 | 5 | 90% | 1,117.19 | 1,067.86 | 1,123.47 | 1,073.86 | 466.45 | 466.17 | 462.05 | 458.33 |
| Jindal India Thermal Ltd | 100 | 5 | 90% | 695.25 | 669.93 | 748.98 | 721.9 | 290.28 | 310.78 | 308.03 | 305.56 |
| Jhabua Power Limited | 115 | 5 | 90% | 733.26 | 700.19 | 861.33 | 822.57 | 279.67 | 332.47 | 332.22 | 332.31 |
| Jhabua Power Limited | 100 | 5 | 90% | 625.27 | 597.06 | 748.98 | 715.28 | 261.06 | 310.78 | 308.03 | 305.56 |
| BALCO | 100 | 5 | 90% | 748.98 | 718.46 | 748.98 | 718.32 | 312.72 | 310.78 | 308.03 | 305.56 |
| TOTAL | 1215 |  |  | 8,321.40 | 7,999.32 | 8,994.76 | 8,647.06 | 3319.64 | 3528.03 | 3518.12 | 3510.52 |

10.7.26	Due to coal shortage and variation in outage schedule there was a reduction in energy availability for the first half of the FY-19. Further the actual availability of DVC Mejia Station for the year 2018-19 and 2019-20 was only around 71 % and for RTPS it is only around 30 %. The revised energy availability for the year 2019-20 was estimated by taking the actual of first half of FY 2019-20 and considering the outages expected to occur during second half of FY-2020 and considering the actual PLF during FY-19 and the normative factors.The revised forecast of FY 21 is estimated based on the revised PLF and normative auxiliary consumption. The parameters taken for revised forecast is given in table below.

| Table 10. 40 Revised forecast of FY 21 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sl.no | Trader/Source | contracted capacity MW | Auxillary consumption % | Normative PLF | Expected PLF |
| 1 | Maithon Power limited | 150 | 6.25 | 85% | 90% |
| 2 | Maithon Power limited | 150 | 6.25 | 85% | 90% |
| 3 | DVC-Mejia TPS | 100 | 5.75 | 85% | 85% |
| 4 | DVC-Reghunathpur TPS | 50 | 5.75 | 85% | 85% |
| 5 | Jindal Power Limited | 200 | 5 | 90% | 90% |
| 6 | Jindal Power Limited | 150 | 5 | 90% | 90% |
| 7 | Jindal Thermal limited | 100 | 5 | 90% | 90% |
| 8 | Jhabua Power Limited | 115 | 5 | 90% | 90% |
| 9 | Jhabua Power Limited | 100 | 5 | 90% | 90% |
| 10 | BALCO | 100 | 5 | 90% | 90% |
|  | TOTAL |  |  |  |  |

10.7.27	The revised forecast of energy availability for the remaining part of control period is given below.

| Table 10. 41 Energy availability through long Term contracts-revised forecast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl.no | Trader/Source | Energy at exbus (MU) |  |  |  | Energy at Kerala periphery (MU) |  |  |  |
|  |  | First half of FY-20 | FY-20 | FY -21 | FY-22 | First half of FY-20 | FY-20 | FY -21 | FY-22 |
| 1 | Maithon Power limited | 491.61 | 1,047.47 | 1,108.69 | 1,108.69 | 536.96 | 1,006.81 | 1,070.99 | 1,070.99 |
| 2 | Maithon Power limited | 491.59 | 1,047.45 | 1,108.69 | 1,108.69 | 536.96 | 1,006.81 | 1,070.99 | 1,070.99 |
| 3 | DVC-Mejia TPS | 288.39 | 579.69 | 701.79 | 701.79 | 282.12 | 562.03 | 679.68 | 679.68 |
| 4 | DVC-Reghunathpur TPS | 120.43 | 264.16 | 350.89 | 350.89 | 139.20 | 255.90 | 339.84 | 339.84 |
| 5 | Jindal Power Limited | 718.33 | 1,469.36 | 1,497.96 | 1,497.96 | 717.69 | 1,404.70 | 1,431.45 | 1,431.45 |
| 6 | Jindal Power Limited | 539.10 | 1,102.38 | 1,123.47 | 1,123.47 | 538.26 | 1,053.73 | 1,073.59 | 1,073.59 |
| 7 | Jindal Thermal limited | 376.96 | 752.47 | 748.98 | 748.98 | 362.75 | 727.07 | 723.51 | 723.51 |
| 8 | Jhabua Power Limited | 328.16 | 760.01 | 861.33 | 861.33 | 413.75 | 728.23 | 825.24 | 825.24 |
| 9 | Jhabua Power Limited | 285.36 | 660.87 | 748.98 | 748.98 | 359.78 | 633.24 | 717.60 | 717.60 |
| 10 | BALCO | 390.45 | 765.97 | 748.98 | 748.98 | 358.84 | 732.54 | 715.73 | 715.73 |
|  | TOTAL | 4,030.38 | 8,449.83 | 8,999.75 | 8,999.75 | 4,246.32 | 8,111.05 | 8,648.62 | 8,648.62 |

Cost of Purchase of power from projects for which tariff is determined by CERC

10.7.28	Among the purchase of Power through long term contracts, the projects for which tariff is determined by Honorable CERC are Maithon Power Ltd, DVC MTPS and DVC RTPS. Honorable CERC is yet to  approve the  fixed charges of the above stations for the control period FY-2020 to FY-2024 as per Tariff Regulations 2019. The fixed charges for FY  2019-20 are estimated at  the annual fixed charges approved by Honorable CERC for the year 2018-19. KSEBL humbly requests before the Honorable Commission to allow the variation in fixed charges for the control period at the time of truing up.

10.7.29	In the MYT petition in the case of Maithon Power Ltd and DVC stations, fixed charges has been estimated by considering 2 % reduction of fixed charges from FY 2018 expecting that fixed charges will be reduced on account of the reduction in return on equity proposed in Draft Tariff Regulations 2019. However, Honorable CERC has not reduced rate of return on equity as proposed in Draft Regulations. Hence 4 % increase is considered from FY 2018-19 for estimating the fixed charges of FY 2020-21 and 2 % increase is considered for FY 2021-22. The actual variable charges of first half of financial year increased considerably and the monthwise details of actual variable cost are given in table below.

|  | Table 10. 42 Variable cost of Long Term contracts April to September 2019 (Rs/kWh) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Source | Approved for FY 2020 | April | May | June | July | August | Sep | Average |
| 1 | Maithon power limited | 2.39 | 2.69 | 2.68 | 2.73 | 2.73 | 2.68 | 2.71 | 2.70 |
| 2 | Maithon power limited | 2.39 | 2.69 | 2.68 | 2.73 | 2.73 | 2.68 | 2.71 | 2.70 |
| 3 | DVC-Mejia TPS | 2.46 | 2.789 | 2.718 | 2.720 | 2.851 | 2.992 | 2.885 | 2.80 |
| 4 | DVC-Reghunathpur TPS | 2.30 | 2.71 | 2.80 | 2.74 | 2.93 | 3.02 | 3.09 | 2.88 |

10.7.30	The variable cost of FY 19 is estimated by taking actual variable charges for first half of FY 2019-20 and by estimating the variable charges of second half of FY 2020@ average variable cost of first half of FY-20. The variable cost of FY 21 and FY-22  is estimated by considering an escalation of 3% in the variable cost of FY 20.

10.7.31	The estimated fixed cost and variable cost for purchase of power from projects for which tariff is determined by CERC is given below

| Table 10. 43 Average variable cost and fixed cost approved for the control period |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station | Average Variable cost (Rs./Unit) |  |  |  | Fixed cost (Rs. Cr) |  |  |  |
|  | Estimated |  |  |  |  |  |  |  |
|  | April-Sept2019 | 2019-20 | 2020-21 | 2021-22 | April-Sept 2019 | 2019-20 | 2020-21 | 2021-22 |
| Maithon power limited | 2.70 | 2.70 | 2.78 | 2.87 | 75.36 | 154.85 | 165.41 | 168.65 |
| Maithon power limited | 2.70 | 2.70 | 2.78 | 2.87 | 75.36 | 154.85 | 165.41 | 168.65 |
| DVC-Mejia TPS | 2.80 | 2.80 | 2.88 | 2.97 | 43.57 | 83.93 | 87.33 | 90.82 |
| DVC-Reghunathpur TPS | 2.88 | 2.88 | 2.96 | 3.05 | 21.04 | 47.89 | 49.82 | 51.81 |

10.7.32	The revised power purchase cost from the above stations based on above parameters is as follows

| Table 10. 44 Power Purchase cost-Revised claim (Rs Cr) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sl No | Station | April-September 2019 |  |  | 2019-20 |  |  |
|  |  | Fixed Charges | Variable charges | Total | Fixed Charges | Variable charges | Total |
| 1 | Maithon power limited | 75.36 | 132.74 | 208.10 | 154.85 | 282.99 | 437.84 |
| 2 | Maithon power limited | 75.36 | 132.74 | 208.09 | 154.85 | 282.99 | 437.84 |
| 3 | DVC-Mejia TPS | 43.57 | 80.84 | 124.41 | 83.93 | 162.40 | 246.33 |
| 4 | DVC-Reghunathpur TPS | 21.04 | 34.67 | 55.71 | 47.89 | 76.03 | 123.92 |
|  | Total | 215.33 | 380.98 | 596.31 | 441.52 | 804.40 | 1245.93 |
| Sl No | Station | 2020-21 |  |  | 2021-22 |  |  |
|  |  | Fixed Charges | Variable charges | Total | Fixed Charges | Variable charges | Total |
|  |  | Rs Cr |  |  | Rs Cr |  |  |
| 1 | Maithon power limited | 165.41 | 308.67 | 474.07 | 168.65 | 317.93 | 486.58 |
| 2 | Maithon power limited | 165.41 | 308.67 | 474.07 | 168.65 | 317.93 | 486.58 |
| 3 | DVC-Mejia TPS | 87.33 | 202.39 | 289.72 | 90.82 | 208.47 | 299.29 |
| 4 | DVC-Reghunathpur TPS | 49.82 | 104.02 | 153.84 | 51.81 | 107.14 | 158.95 |
|  | Total | 467.96 | 923.75 | 1391.71 | 479.93 | 951.46 | 1431.39 |

Cost of Purchase of power from projects through DBFOO

10.7.33	Honorable Commission approved purchase of power from projects through DBFOO as shown in table D 38. In the MYT order dated 08.07.2019 , Honorable Commission allowed scheduling of power from 350 MW of power from DBFOO contracts, of M/s Jindal Power Limited 150 MW, M/s Jhabua Power Limited -100 MW and M/s Jindal India Thermal Power Ltd -100 MW, under bid II,  for which rates are yet to be adopted, at the rate equivalent to the cost of power from M/s BALCO which is the L1 of Bid II.It may kindly be noted DBFOO framework allows pass through of actual fuel cost based on predetermined Station Heat Rate, which shall not exceed 2350 kCal/kWh, subject to parameters specified to arrive at the variable charge as on bid date. Further, the Station Heat Rate has to be certified by the generators by following tests prescribed in the PSA prior to date of commissioning. The Station Heat rates duly certified by generators under Bid 2 viz; M/s Jindal PowerLtd (150 MW), M/s Jhabua Power Ltd (100 MW)  , M/s Jindal India Thermal Power Ltd (100 MW) and M/s BALCO (100 MW)  are 2327 kCal/kWh, 2347.9kCal/kWh, 2350 kCal/kWh and 2350 kCal/kWh respectively. Further, the delivered cost of coal to each generator would depend on the grade of coal supplied by Coal India Ltd or its subsidiaries, the notified price of coal of the respective mine  from which coal is supplied and the cost of transportation which again depends on the mode of transportation and distance from mine. At the same time, the ratio of the notified price of coal and transportation with the quoted fuel price and transportation cost is not allowed to change for the benefit of the generator. In view of the above framework prescribed in DBFOO model as notified by MoP,GoI, it is respectfully submitted that the rate applicable for BALCO cannot be used for estimating the power fpurchase cost of other stations, eventhough the rates were matched as on date of biddig.

10.7.34	The applicable fixed charges for each year for the power from projects through DBFOO contracts is determined as per the clause 21.2, 21.3 and 21.4 of power supply agreement entered as per DBFOO contracts.The variable charges for FY 2019-20 is estimated @ average variable cost of first half of FY 20 and the variable cost for the rest of control period is estimated by considering an escalation of 3 % over FY-2020.The actual variable cost for first half of financial year is given in table below.

|  | Table 10. 45 Variable cost of Power purchase from projects through DBFOO contracts for the period April to September 2019 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Source | Approved for FY 2020 | April | May | June | July | August | September | Average |
|  |  |  | Rs/kWh | Rs/kWh | Rs/kWh | Rs/kWh | Rs/kWh | Rs/kWh | Rs/kWh |
| 1 | Jindal Power Ltd-Bid I | 1.29 | 1.31 | 1.31 | 1.31 | 1.31 | 1.27 | 1.29 | 1.3 |
| 2 | Jindal Power Ltd-Bid II |  | 1.30 | 1.30 | 1.30 | 1.30 | 1.26 | 1.29 | 1.29 |
| 3 | Jindal India Thermal Lltd |  | 1.14 | 1.14 | 1.14 | 1.14 | 1.14 | 1.14 | 1.14 |
| 4 | Jhabua Power Ltd-Bid I | 1.91 | 1.95 | 1.95 | 1.95 | 1.95 | 1.95 | 1.95 | 1.95 |
| 5 | Jhabua Power Ltd-Bid II |  | 1.87 | 1.87 | 1.87 | 1.87 | 1.87 | 1.87 | 1.87 |
| 6 | BALCO | 1.19 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 | 1.15 |

10.7.35	The revised estimate for fixed charges and variable charges for the control period is given in table below

| Table 10. 46 Average variable cost and fixed cost approved for the control period |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Average Variable cost (Rs./Unit) |  |  |  | Fixed cost (Rs per unit) |  |  |  |
| Station | April-Sep2019 | 2019-20 | 20-21 | 21-22 | April-Sep 2019 | 19-20 | 20-21 | 21-22 |
| Jindal Power Ltd-Bid I | 1.30 | 1.30 | 1.34 | 1.38 | 2.61 | 2.61 | 2.56 | 2.51 |
| Jindal Power Ltd-Bid II | 1.29 | 1.29 | 1.33 | 1.37 | 3.33 | 3.33 | 3.27 | 3.2 |
| Jindal India Thermal Lltd | 1.14 | 1.14 | 1.17 | 1.21 | 3.43 | 3.44 | 3.37 | 3.3 |
| Jhabua Power Ltd-Bid I | 1.95 | 1.95 | 2.01 | 2.07 | 2.05 | 2.05 | 2.01 | 1.97 |
| Jhabua Power Ltd-Bid II | 1.87 | 1.87 | 1.93 | 1.98 | 2.89 | 2.89 | 2.83 | 2.77 |
| BALCO | 1.15 | 1.15 | 1.18 | 1.22 | 3.06 | 3.16 | 3.1 | 3.03 |

10.7.36	The revised cost of  purchase of power through DBFOO contracts is given in table below

| Table 10. 47 Power purchase cost –Revised claim |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station | April-September 2019 |  |  |  | 2019-20 |  |  |  |
|  | Fixed Charges | Variable charges | Other charges* | Total | Fixed Charges | Variable charges | Other charges* | Total |
|  | Rs Cr |  |  |  | Rs Cr |  |  |  |
| Jindal Power Ltd-Bid I | 187.51 | 93.37 | -14.88 | 265.99 | 383.56 | 190.98 | -29.38 | 545.16 |
| Jindal Power Ltd-Bid II | 179.55 | 69.62 | -15.47 | 233.70 | 367.14 | 142.36 | -30.57 | 478.93 |
| Jindal India Thermal Lltd | 129.33 | 42.97 | -8.91 | 163.40 | 258.51 | 85.78 | -17.51 | 326.78 |
| Jhabua Power Ltd-Bid I | 67.27 | 64.06 | -6.04 | 125.29 | 155.80 | 148.36 | -14.51 | 289.64 |
| Jhabua Power Ltd-Bid II | 82.47 | 53.36 | -6.86 | 128.97 | 190.99 | 123.58 | -16.11 | 298.46 |
| BALCO | 119.55 | 44.90 | -9.93 | 154.52 | 238.21 | 88.09 | -19.00 | 307.30 |
| Total | 765.68 | 368.28 | -62.09 | 1071.87 | 1594.22 | 779.15 | -127.09 | 2246.28 |
| Station | 2020-21 |  |  |  | 2021-22 |  |  |  |
|  | Fixed Charges | Variable charges | Other charges* | Total | Fixed Charges | Variable charges | Other charges* | Total |
|  | Rs Cr |  |  |  | Rs Cr |  |  |  |
| Jindal Power Ltd-Bid I | 383.48 | 200.54 | -28.88 | 555.13 | 375.99 | 206.56 | -28.83 | 553.71 |
| Jindal Power Ltd-Bid II | 367.37 | 149.43 | -30.03 | 486.77 | 359.51 | 153.92 | -29.88 | 483.55 |
| Jindal India Thermal Lltd | 252.41 | 87.95 | -17.08 | 323.27 | 247.16 | 90.58 | -16.98 | 320.77 |
| Jhabua Power Ltd-Bid I | 173.13 | 173.18 | -13.03 | 333.27 | 169.68 | 178.37 | -13.09 | 334.96 |
| Jhabua Power Ltd-Bid II | 211.96 | 144.26 | -15.03 | 341.19 | 207.47 | 148.59 | -15.02 | 341.04 |
| BALCO | 232.18 | 88.72 | -14.61 | 306.29 | 226.94 | 91.38 | -14.50 | 303.82 |
| Total | 1620.53 | 844.07 | -118.66 | 2345.95 | 1586.75 | 869.40 | -118.30 | 2337.84 |

*Other charges comprises of the transmission charges, compensation of transmission losses upto Kerala periphery borne by the Generrator as per PSA

Summary of powerpurchase from LTA

10.7.37	The summary of the power purchase cost from LTA is as shown below:

| Table 10. 48 Summary of Power purchase cost from LTA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Approved by Honorable Commission |  |  |  |  |
| Financial Year | Capacity | Ex-bus energy | Energy at periphery | Total charges |
|  | MW | MU | MU | Rs. Crore |
| 2019-20 | 1,215.00 | 8,994.76 | 8,647.06 | 3,528.02 |
| 2020-21 | 1,215.00 | 8,994.76 | 8,647.06 | 3,518.11 |
| 2021-22 | 1,215.00 | 8,994.76 | 8,647.07 | 3,510.51 |
| Revised Estimate |  |  |  |  |
| April-September 2019(Actual) | 1,215.00 | 2,872.70 | 2,751.07 | 1668.18 |
| 2019-20 (Estimate) | 1,215.00 | 8,449.83 | 8,111.05 | 3,492.20 |
| 2020-21 (Estimate) | 1,215.00 | 8,999.75 | 8648.62 | 3,737.65 |
| 2021-22 (Estimate) | 1,215.00 | 8,999.75 | 8648.62 | 3,769.23 |

Energy procurement from liquid fuel stations

10.7.38	The capacity and energy availability from, RGCCPP, Kayamkulam is given below.   Due to high fuel cost and consequent power cost, no power is proposed to be used from this plant. This plant will be used only during extreme exigencies.  However, there is a fixed charge obligation of Rs 200 Cr per year. Honorable Commission approved Rs 200 Cr. for FY-19 and directed to renegotiate the annual fixed charges of RGCCPP as there was a provision of review.

10.7.39	Even after several round of discussions with NTPC, they were not willing to reduce the fixed charges from Rs 200 Cr.  Hence KSEBL provisionally admitted an annual fixed charges of Rs 62.3544 Cr. from April 2019 onwards eventhough the same was not agreeable to NTPC. Hence for the time being, KSEBL humbly requests before the Honorable Commission to allow 62.3544 Cr. from 2019-20 onwards and requests to allow the variations if any, that materializes on future dates, at the time of truing up.

| Table 10. 49 Energy availability from liquid fuel stations |  |  |  |  |
| --- | --- | --- | --- | --- |
| Station | Capacity (MW) | Capacity Available (MW) | Energy availability (MU/day) |  |
| RGCCPP | 360 | 320 | 7.45 |  |
| Fixed charges (Rs Cr) |  |  |  |  |
|  | April-September 2020 | 2019-20 | 2020-21 | 2021-22 |
| Approved by Honorable Commission |  | 0 | 0 | 0 |
| Revised Estimate | 31.18 | 62.35 | 62.35 | 62.35 |

Interstate transmission charges

10.7.40	The transmission charges to be paid for ISTS for 2019-20 are estimated by taking the actual transmission charges for the first half of FY 20 and the transmission charges for second half and remaining part of control period is estimated on the basis of averagePoC charges for the period from April to September 2019 approved by CERC vide order dated 02.05.2019 and 10.10.2019.The transmission charges approved for the two quarters of FY 20 and the projection of transmission charges are given in Table below.

| Table 10. 50 Interstate transmission charges claimed and approved vide MYT order |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Quarter | CERC order date | PoC charges | Reliability support charges | HVDC charges | Total |
|  |  | Rs/MW |  |  |  |
| Apri-June | 02.05.2019 | 170715 | 31315 | 17858 | 219888 |
| July to September | 10.10.2019 | 124104 | 31725 | 17886 | 173715 |
| Average |  | 147409.5 | 31520 | 17872 | 196801.5 |
| POC charges taken for contoll period |  |  |  |  |  |
| 2019-20 |  | 147409.5 | 31520 | 17872 | 196801.5 |
| 2020-21 |  | 147409.5 | 31520 | 17872 | 196801.5 |
| 2021-22 |  | 147409.5 | 31520 | 17872 | 196801.5 |

10.7.41	Based on above PoC charges the interstate transmission is estimated as follows.

| Table 10. 51 Interstate transmission charges (Rs Cr) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Source | Allocated capacity (MW) | Apr to Sep 2019 | 2019-20 | 2020-21 | 2021-22 |
|  |  |  | Rs Cr | Rs Cr | Rs Cr |
| RSPTS Stage I & II | 245.07 | 24.89 | 57.88 | 57.88 | 57.88 |
| RSTPS Stage III | 61 | 6.18 | 14.41 | 14.41 | 14.41 |
| TALCHER - Stage II | 427 | 42.93 | 100.84 | 100.84 | 100.84 |
| Simhadri Exp | 85.55 | 9.10 | 20.20 | 20.20 | 20.20 |
| NLC-II- Stage-1 | 63 | 6.02 | 14.88 | 14.88 | 14.88 |
| NLC-II- Stage-2 | 90.02 | 8.55 | 21.26 | 21.26 | 21.26 |
| NLC- Exp- Stage-1 | 68.24 | 6.59 | 16.11 | 16.11 | 16.11 |
| NLC - II Exp | 81.23 | 7.70 | 19.18 | 19.18 | 19.18 |
| MAPS | 23.01 | 2.17 | 5.43 | 5.43 | 5.43 |
| KAIGA Stg I& II | 38.01 | 6.88 | 8.98 | 8.98 | 8.98 |
| KAIGA Stg II | 34.98 |  | 8.26 | 8.26 | 8.26 |
| Kudamkulam Unit I | 140.1 | 26.51 | 33.09 | 33.09 | 33.09 |
| Kudamkulam unit II | 133 |  | 31.41 | 31.41 | 31.41 |
| Vallur JV with | 49.95 | 5.20 | 11.80 | 11.80 | 11.80 |
| NTPL(Tuticorin JV) | 72.5 | 7.71 | 17.12 | 17.12 | 17.12 |
| Kudgi Unit I | 111.72 | 11.31 | 26.38 | 26.38 | 26.38 |
| 20 Bhavini | 43 |  | 0.00 | 10.15 | 10.15 |
| NNLTPS | 32.4 |  | 1.28 | 7.65 | 7.65 |
| Sub Total | 1799.78 | 171.73 | 408.51 | 425.04 | 425.04 |
| Long Term Agreements |  |  |  |  |  |
| Maithon power | 150 | 14.86 | 35 | 35.42 | 35.42 |
| Maithon power | 150 | 14.86 | 35 | 35.42 | 35.42 |
| DVC-Mejia TPS | 100 | 9.98 | 24 | 23.62 | 23.62 |
| DVC-R’pur | 50 | 4.92 | 12 | 11.81 | 11.81 |
| Sub Total | 450 | 44.62 | 106.27 | 106.27 | 106.27 |
| RGCCPP | 359 | 4.20 | 8.16 | 8.16 | 8.16 |
| Total | 2608.78 | 220.55 | 522.94 | 539.47 | 539.47 |

Power procurement from IPPs

10.7.42	Honorable Commission approved the power procurement from IPPs within the State for the control period as follows.

| Table 10. 52 Purchase of power from IPPs for the control period |  |  |  |  |
| --- | --- | --- | --- | --- |
| Small Hydel | Capacity (MW) | Annual Energy availability (MU) | Per unit cost (Rs/unit) | Total cost (Rs.Cr.) |
| Ullumkal | 7 | 17.79 | 2.44 | 4.34 |
| Iruttukanam I & II | 4.5 | 24.69 | 2.70 | 6.67 |
| Karikkayam | 10.5 | 37.56 | 4.16 | 15.62 |
| Meenvallom | 3 | 8.44 | 4.88 | 4.12 |
| Pambumkayam | 0.11 | 0.09 | 4.88 | 0.04 |
| Kallar | 0.05 | 0.08 | 5.47 | 0.04 |
| SHP Total | 25.16 | 88.65 | 3.48 | 30.84 |
| Agali | 13.8 | 29.72 | 3.14 | 9.33 |
| Koundikkal | 4.8 | 12.56 | 3.14 | 3.94 |
| Ramakkalmedu | 14.25 | 27.87 | 3.14 | 8.75 |
| Ahalya | 8.4 | 20.98 | 5.23 | 10.97 |
| INOX | 16 | 30.84 | 4.09 | 12.61 |
| Wind Total | 57.25 | 121.97 | 3.74 | 45.61 |
| Kasargod solar Park | 50 | 70.08 | 3.90 | 27.33 |
| Sub Total | 50 | 70.08 | 3.90 | 27.33 |
| Grand Total | 132.41 | 280.70 | 3.70 | 103.78 |

10.7.43	The revised forecast is done considering the following

- Actual of first half of financial year 2019-20

- Honorable Commission vide order dated 6.2.2019 approved the levelised tariff for the electricity generated from the 50 MW SolarPV project at Kasargode as @Rs 3.83/unit.

- Due to flood  there is no energy availability from Mankulam Grama Panchayath and Western Kallar

- Honorable Commission has not considered power from wind power project of M/s Kosamattom Finance Ltd  in the MYT order. M/s Kosamattom Finance Ltd was connected to grid on 20.03.2018 and injected 1.13 MU during FY 2018-19 at a total power purchase cost of Rs 0.06 Cr. It may please be noted that KSEBL initialled draft PPA with M/s Kosamattom Pvt Ltd on 27.10.2018 @ interim tariff of Rs 3.07 per unit and submitted before Honorable Commission for approval.  Honorable Commission approved the tariff for the project  at Rs 4.23 per unit without accelerated depreciation and at Rs 3.93 per unit with accelerated depreciation benefit.  Hence the energy availability from Kosamattom is also considered in this projection @ Rs 3.93 per unit taking into account of accelerated depreciation benefit.

10.7.44	The revised forecast for the remaining part of control period is as follows:

| Table 10. 53 Power purchase cost from IPPs |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Small Hydel | Capacity (MW) | Per unit cost (Rs/unit) | Energy in MU |  |  |  | Cost of Power Purchase (Rs Cr.) |  |  |  |
|  |  |  | April-September | 2019-20 | 2020-21 | 2021-22 | April-September | 2019-20 | 2020-21 | 2021-22 |
| Ullumkal | 7 | 2.44 | 10.17 | 17.98 | 17.79 | 17.79 | 2.48 | 4.39 | 4.34 | 4.34 |
| Iruttukanam I & II | 4.5 | 2.7 | 9.84 | 19.42 | 24.69 | 24.69 | 2.66 | 5.24 | 6.67 | 6.67 |
| Karikkayam | 10.5 | 4.16 | 23.45 | 38.17 | 37.56 | 37.56 | 8.62 | 15.88 | 15.62 | 15.62 |
| Meenvallom | 3 | 4.88 | 4.18 | 7.30 | 8.44 | 8.44 | 2.04 | 3.56 | 4.12 | 4.12 |
| Pambumkayam | 0.11 | 4.88 | 0.00 | 0.00 | 0 | 0.00 | 0.00 |  | 0.00 | 0.00 |
| Kallar | 0.05 | 5.47 | 0.00 | 0.00 | 0 | 0.00 | 0.00 |  | 0.00 | 0.00 |
| Pathamkayam | 8 | 5.47 | 0.00 | 0.00 | 0 | 0.00 | 0.00 |  | 0.00 | 0.00 |
| SHP Total | 33.16 |  | 47.63 | 82.87 | 88.48 | 88.48 | 15.79 | 29.07 | 30.75 | 30.75 |
| Agali | 13.8 | 3.14 | 23.84 | 27.04 | 29.72 | 29.72 | 7.49 | 8.49 | 9.33 | 9.33 |
| Koundikkal | 4.8 | 3.14 | 8.76 | 12.34 | 12.56 | 12.56 | 2.75 | 3.87 | 3.94 | 3.94 |
| Ramakkalmedu | 14.25 | 3.14 | 14.67 | 21.54 | 27.87 | 27.87 | 4.60 | 6.76 | 8.75 | 8.75 |
| Ahalya | 8.4 | 5.23 | 12.39 | 19.47 | 20.98 | 20.98 | 6.48 | 10.18 | 10.97 | 10.97 |
| INOX | 16 | 4.09 | 23.54 | 36.90 | 38.90 | 38.90 | 9.63 | 15.09 | 15.91 | 15.91 |
| Kosamattom | 1 | 3.93 | 1.14 | 1.22 | 2.10 | 2.10 | 0.35 | 0.48 | 0.83 | 0.83 |
| Wind Total | 57.25 | 3.74 | 84.35 | 118.52 | 132.14 | 132.14 | 31.30 | 44.89 | 49.74 | 49.74 |
| Kasargod solar Park | 50 | 3.83 | 33.85 | 77.17 | 70.08 | 70.08 | 12.96 | 29.56 | 26.84 | 26.84 |
| Kuzhalmannam | 2 | 3.14 | 1.19 | 2.63 | 2.54 | 2.54 | 0 | 0.82 | 0.80 | 0.80 |
| Sub Total | 50 |  | 35.05 | 79.79 | 72.62 | 72.62 | 12.96 | 30.38 | 27.64 | 27.64 |
| Grand Total | 132.41 | 3.7 | 167.03 | 281.18 | 293.23 | 293.23 | 60.06 | 104.34 | 108.13 | 108.13 |

Power procurement from renewable energy sources

10.7.45	Honorable Commission approved RPO targets and the required renewable energy as follows

| Table 10. 54 RPO Targets and additional Renewable energy approved by Hon’ble Commission |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| Sales Excluding Bulk supply (MU) | 21054.25 | 22336.68 | 23294.98 | 24298.35 |
| Net Hydro excluding SHP (MU) | 7,458.23 | 6,435.76 | 6,229.25 | 6,214.47 |
| Energy for RPO (MU) | 13,596 | 15,901 | 17,066 | 18,084 |
| Non-Solar Obligation (%) | 7.00% | 8.00% | 9.00% | 10.25% |
| Non-Solar Obligation (MU) | 951.72 | 1,272.07 | 1,535.91 | 1,853.60 |
| Existing banking arrangement (MU) | 65.8 | 65.8 | 65.8 | 65.8 |
| Net internal Small Hydro (MU) | 349.2 | 490.3 | 566.4 | 787.41 |
| IPPs-SHP Total (Existing) (MU) | 88.65 | 88.65 | 88.65 | 88.65 |
| Wind (internal) (MU) | 1.77 | 2.02 | 2.06 | 2.06 |
| Wind IPPs (Existing) (MU) | 121.97 | 121.97 | 121.97 | 121.97 |
| Total Non-Solar available (MU) | 627.38 | 768.73 | 844.87 | 1,065.88 |
| Balance requirement of Non-Solar energy (MU) | 324.34 | 503.35 | 691.04 | 787.71 |
| Solar Obligation (%) | 2.75% | 4.00% | 5.25% | 6.75% |
| Solar Obligation (MU) | 373.89 | 636.03 | 895.96 | 1,220.67 |
| Solar (internal) (MU) | 14.81 | 42.18 | 47.12 | 47.12 |
| Banking arrangement & GBI (MU) | 38.26 | 38.26 | 38.26 | 38.26 |
| Solar (Existing IPPs) (MU) | 70.08 | 70.08 | 70.08 | 70.08 |
| Total Solar Energy available (MU) | 123.15 | 150.52 | 155.46 | 155.46 |
| Balance Requirement Solar Energy (MU) | 250.74 | 485.51 | 740.5 | 1,065.21 |

10.7.46	Based on the revised energy sales and energy generation the renewable purchase obligation to be met as per the KSERC (Renewable Energy) Regulations, 2020 is as follows

| Table 10. 55 : Revised RPO Targets for next control period |  |  |  |
| --- | --- | --- | --- |
| Particulars | Quantum in MU |  |  |
|  | 2019-20 | 2020-21 | 2021-22 |
| Energy sales excl bulk licensees | 22337.76 | 22900.56 | 23830.22 |
| Hydro | 6079.86 | 6122.07 | 6179.86 |
| Net Energy sales | 16257.90 | 16778.49 | 17650.36 |
| Non-Solar Obligation (%) | 8.00% | 9.00% | 10.25% |
| Non Solar RPO Targets as per RE regulations | 1,300.63 | 1,510.06 | 1,809.16 |
| Solar Obligation (%) | 4.00% | 5.25% | 6.75% |
| Solar RPO Targets as per RE regulations | 650.32 | 880.87 | 1191.40 |

10.7.47	The petitioner has been earnestly attempting to meet the RPO through own generation as well as through procurement. The Renewable energy generation of KSEB Ltd for the control period FY 20 to FY-22 is given in table below.

| Table 10. 56 Renewable Energy Generation from own SHEPs |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Energy availability in MU |  |  |
| Sl No | Particulars | FY-20 | FY-21 | FY-22 |
| 1 | Existing SHEP | 436.66 | 497.78 | 502.45 |
| 2 | Bhothathan kettu |  | 41.7 | 83.4 |
| 3 | Upper Kallar |  | 1.284 | 2.568 |
| 4 | Poringal AES |  | 45 | 45 |
| 5 | Chathankottunada |  |  | 7.38 |
| 6 | Pazhassisagar |  |  | 49.5 |
|  | Sub Total | 436.66 | 585.76 | 690.30 |
| Renewable Energy Generation from own wind projects |  |  |  |  |
|  |  | Energy availability in MU |  |  |
| Sl No | Particulars | FY-20 | FY-21 | FY-22 |
|  | Wind Kanjikkode | 2.04 | 2.08 | 2.08 |
| Renewable Energy Generation from own solar projects |  |  |  |  |
|  |  | Energy availability in MU |  |  |
| Sl No | Particulars | FY-20 | FY-21 | FY-22 |
|  | Existing and new | 22.57 | 47.60 | 47.60 |

10.7.48	The RE procurement arrangement from existing SHP, Wind and Solar sources as described in para 10.7.44 is reproduced below

| Table 10. 57 Power purchase cost from IPPs |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Small Hydel | Capacity (MW) | Per unit cost (Rs/unit) | Energy in MU |  |  |  |
|  |  |  | April-September | 2019-20 | 2020-21 | 2021-22 |
| Ullumkal | 7 | 2.44 | 10.17 | 17.98 | 17.79 | 17.79 |
| Iruttukanam I & II | 4.5 | 2.7 | 9.84 | 19.42 | 24.69 | 24.69 |
| Karikkayam | 10.5 | 4.16 | 23.45 | 38.17 | 37.56 | 37.56 |
| Meenvallom | 3 | 4.88 | 4.18 | 7.30 | 8.44 | 8.44 |
| Pambumkayam | 0.11 | 4.88 | 0.00 | 0.00 | 0 | 0.00 |
| Kallar | 0.05 | 5.47 | 0.00 | 0.00 | 0 | 0.00 |
| Pathamkayam | 8 | 5.47 | 0.00 | 0.00 | 0 | 0.00 |
| SHP Total | 33.16 |  | 47.63 | 82.87 | 88.48 | 88.48 |
| Agali | 13.8 | 3.14 | 23.84 | 27.04 | 29.72 | 29.72 |
| Koundikkal | 4.8 | 3.14 | 8.76 | 12.34 | 12.56 | 12.56 |
| Ramakkalmedu | 14.25 | 3.14 | 14.67 | 21.54 | 27.87 | 27.87 |
| Ahalya | 8.4 | 5.23 | 12.39 | 19.47 | 20.98 | 20.98 |
| INOX | 16 | 4.09 | 23.54 | 36.90 | 38.90 | 38.90 |
| Kosamattom | 1 | 3.93 | 1.14 | 1.22 | 2.10 | 2.10 |
| Wind Total | 57.25 | 3.74 | 84.35 | 118.52 | 132.14 | 132.14 |
| Kasargod solar Park | 50 | 3.83 | 33.85 | 77.17 | 70.08 | 70.08 |
| Kuzhalmannam | 2 | 3.14 | 1.19 | 2.63 | 2.54 | 2.54 |
| Sub Total | 50 |  | 35.05 | 79.79 | 72.62 | 72.62 |
| Grand Total | 132.41 | 3.7 | 167.03 | 281.18 | 293.23 | 293.23 |

10.7.49	In order to meet the RPO obligation KSEBL has entered into new Power Purchase Agreements/Power Sale agreements during 2019-20 period for the purchase of renewable power, the details of which are enumerated below:

| Table 10. 58 Details of contracts entered for purchase during the second control period |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sl.No | Name of Station | Date of PPA | Capacity MW | Tariff, Rs./kWh | Expected date of power purchase |
|  | Solar Projects |  |  |  |  |
| 1 | NTPC Floating Solar Project at Kayamkulam | 28.08.2019 | 92 | 3.16 | Dec-20 |
| 2 | Solar Ground mounted THDCIL, Paivalike | 16-01-2019 | 50 | 3.10 | Mar-20 |
| 3 | Floating Solar project of NHPC at West Kallada. | 31.08.2019 | 10 | 3.43 | Aug-20 |
| 4 | Solar project of THDCIL at Nellithadam, Kasargod | 25.09.2019 (initialled) | 5 | 3.10 | Mar-20 |
|  | Non solar projects |  |  |  |  |
| 5 | PSA with SECI for purchase of Wind Power | 14-06-2019 | 200 | 2.83+trading margin 0.07 | Jan-21 |
| 6 | PSA with SECI for purchase of Wind Power | 06.09.2019 | 100 | 2.81+trading margin 0.07 | Jul-21 |

Further, discussions are going on for procuring 40 MW from NHPC at West Kallada and expected COD is September 2020 @ Rs 3.43 per unit

10.7.50	In addition to the above the following floating solar projects on various dam sites are under pipeline.

| Table 10. 59 Details of contracts entered for purchase during the second control period |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Location | Details of water body | Capacity in MW | Project executing Agency | Present status | Role of KSEBL |
| Idukki |  |  |  |  |  |
| 1)Cheruthoni | Idukki reservoir of Idukki 780MW HEP | 50 | NTPC | Agency identified for conducting bathymetric and topographic survey by NTPC. | Development of transmission system for evacuation of power to Vazhathope substation proposed to be upgraded to 110KV and to procure power generated from the project. |
| 2) Anchuruli | -do- | 250 | NTPC | To be done as 2nd phase by NTPC. | Development of transmission system for evacuation of power to Kattappana substation proposed to be upgraded to 110KV, development of 220kV Kuyilimala substation and to procure power from the project. |
| 3) Kulamavu | Idukki reservoir of Idukki 780MW HEP | 100 | SECI | Initial surveying carried out and power evacuation proposal finalised | Development of transmission system for evacuation of power to Kulamavu substation proposed to be upgraded to 110KV and to procure power generated from the project. |
| Banasurasagar | Banasurasagar reservoir of Kuttiyadi 225MW HEP | 100 | SECI | -do- | Development of transmission system for evacuation of power to Padinjarathara substation proposed to be upgraded to 220KV and to procure power from the project. |

10.7.51	Among the above projects 50 MW of Cheruthoni is expected to complete by March 2021,100 MW of Anchuruli is expected to complete during September 2021, 100 MW of Kulamavu expected to be completed by September 2021,100 MW Banasurasagar expected to complete by September 2021. As PPA is yet to be executed power purchase cost is provisionally estimated @ Rs 3.16 per unit (Rate proposed for NTPC floating solar project at Kayamkulam).

10.7.52	In addition to the above under Saura scheme, 250 MW of roof top solar are expected to complete during the year 2020-21 and 250 MW of roof top solar is expected to complete during 2021-22. Since the RESCO bid under rooftop solar is to be retendered and in respect of other modes no power procurement is involved, the cost of power procurement is not estimated under this instant petition. The same is considered hereunder for the limited purpose of meeting the RPO.

10.7.53	Further, Ministry of New and Renewable Energy (MNRE) has launched the Pradhan Mantri Kisan Urja Suraksha evem Utthan Mahabhiyan (PM KUSUM) Scheme for farmers for installation of solar pumps and grid connected solar and other renewable power plants in the country. The scheme aims to add solar and other renewable capacity of 25,750 MW by 2022 with total central financial support of Rs. 34,422 Crore including service charges to the implementing agencies.The target allocation for KSEBL is 10 MW and the expected rate is Rs 3.50 per unit.

10.7.54	The revised estimate for renewable energy procurement from other sources is provided in the table below.

| Table 10. 60 Renewable energy expected during the control period |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solar Projects | Capacity | CUF | Expected date of completion | Energy availability |  | Rate | Power Purchase cost (Rs Cr) |  |
|  | MW | % |  | MU |  | Rs/unit |  |  |
|  |  |  |  | FY-21 | FY-22 |  | FY-21 | FY-22 |
| NTPC Floating Solar Project at Kayamkulam | 92 | 0.22 | Dec-20 | 58.78 | 177.30 | 3.16 | 18.57 | 56.03 |
| Ground mounted THDCIL, Paivalike | 50 | 0.19 | Mar-20 | 83.22 | 83.22 | 3.10 | 25.80 | 25.80 |
| Floating Solar project of NHPC at West Kallada. | 10 | 0.22 | Sep-20 | 12.23 | 24.53 | 3.43 | 4.20 | 8.41 |
| Solar project of THDCIL at Nellithadam, Kasargod | 5 | 0.19 | Mar-20 | 8.32 | 8.32 | 3.10 | 2.58 | 2.58 |
| Floating Solar Project at Idukki reservoir(Cheruthoni) | 50 | 0.22 | Mar-21 |  | 96.36 | 3.16 |  | 30.45 |
| Floating Solar project of NHPC at West Kallada. | 40 | 0.22 | Sep-20 | 38.44 | 77.09 | 3.43 | 13.18 | 26.44 |
| Floating Solar Project at Kulamavu | 100 | 0.22 | Sep-21 | 0.00 | 96.10 | 3.16 | 0.00 | 30.37 |
| Floating Solar Project at Anchuruli | 100 | 0.22 | Sep-21 |  | 96.10 | 3.16 | 0.00 | 30.37 |
| Floating Solar Project at Banasurasagar | 100 | 0.22 | Sep-21 |  | 96.10 | 3.16 | 0.00 | 30.37 |
| Power procurement through reverse bid process | 200 | 0.19 | Dec-21 | 0.00 | 82.08 | 3.00 | 0.00 | 24.62 |
| Roof Top Solar -Saura-250 MW | 250 | 0.17 | Oct-20 | 154.02 | 372.30 |  |  |  |
| Roof Top Solar -Saura-250 MW | 250 | 0.17 | Oct-21 | 0.00 | 154.02 |  |  |  |
| PM KUSUM | 10 | 0.19 | Dec-21 | 8.30 | 16.64 |  |  |  |
| Sub Total | 1257 |  |  | 362.27 | 1374.90 | 31.86 | 63.98 | 263.63 |
| Non solar projects |  |  |  |  |  |  |  |  |
| PSA with SECI for purchase of Wind Power | 200 | 0.35 | Sept-20 | 136.38 | 613.16 | 2.90 | 39.55 | 177.82 |
| PSA with SECI for purchase of Wind Power | 100 | 0.35 | Jul-21 | 0.00 | 192.18 | 2.88 | 0.00 | 55.35 |
| Sub Total | 300.00 |  |  | 136.38 | 805.34 |  | 39.55 | 233.16 |
| Total | 1557.00 |  |  | 498.65 | 2180.23 |  | 103.52 | 496.79 |

10.7.55	Further KSEBL has made banking arrangements with the following prosumers for which RPO can be claimed. The actual details and the estimate for the remaining part of control period is given in table below.

| Table 10. 61 Banking Arrangements with prosumers for which RPO can be claimed |  |  |  |  |
| --- | --- | --- | --- | --- |
| Particulars | Energy in MU |  |  |  |
| Banking arrangements | As on 31.12.2019 | FY-20 | FY-21 | FY-22 |
| Small Hydro Electric Project |  |  |  |  |
| Maniyar | 23.36 | 25.21 | 25.21 | 25.21 |
| Kuthungal | 40.34 | 48.82 | 48.82 | 48.82 |
| Wind Projects |  |  |  |  |
| Malayala Manorama Ltd | 18.83 | 26.28 | 26.28 | 26.28 |
| Subtotal nonsolar | 82.53 | 100.31 | 100.31 | 100.31 |
| Solar |  |  |  |  |
| CIAL | 33.31 | 44.48 | 44.48 | 44.48 |
| Other solar prosumers (grid connected) (LT & HT) | 34.285 | 89.53 | 91.32 | 93.15 |
| Off grid consumes availing GBI | 2.67 | 2.67 | 2.72 | 2.78 |
| Subtotal Solar | 70.27 | 136.68 | 138.53 | 140.41 |

10.7.56	Additional energy requirement for meeting RPO is estimated in table below

| Table 10. 62 Balance energy requirement for meeting RPO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sl No | Particulars | 2019-20 | 2020-21 | 2021-22 |
| 1 | Sales Excluding Bulk supply (MU) | 22337.76 | 22900.56 | 23830.22 |
| 2 | Net Hydro excluding SHP (MU) | 6079.86 | 6122.07 | 6179.86 |
| 3 | Energy for RPO (MU) | 16257.90 | 16778.49 | 17650.36 |
| 4 | Non-Solar Obligation (%) | 0.08 | 0.09 | 0.10 |
| 5 | Non-Solar Obligation (MU) | 1300.63 | 1510.06 | 1809.16 |
| 6 | Existing banking arrangement (MU) | 100.31 | 100.31 | 100.31 |
| 7 | Net internal Small Hydro (MU)Existing | 436.66 | 585.76 | 690.30 |
| 8 | IPPs-SHP Total (Existing) (MU) | 82.87 | 88.48 | 88.48 |
| 9 | Wind (internal) (MU) | 2.04 | 2.08 | 2.08 |
| 10 | Wind IPPs (Existing) (MU) | 118.52 | 132.14 | 132.14 |
| 11 | Wind new purchase expected |  | 136.38 | 805.34 |
| 12 | Total Non-Solar available (MU) | 740.39 | 1045.14 | 1818.64 |
| 13 | Balance requirement of Non-Solar energy (MU) | 560.25 | 464.92 | -9.48 |
| 14 | Solar Obligation (%) | 0.04 | 0.05 | 0.07 |
| 15 | Solar Obligation (MU) | 650.32 | 880.87 | 1191.40 |
| 16 | Solar (internal) (MU) | 22.57 | 47.60 | 47.60 |
| 17 | Roof Top under Saura scheme |  | 185.64 | 526.32 |
| 18 | PM KUSUM scheme |  | 8.30 | 16.64 |
| 19 | Banking arrangement & GBI (MU) | 136.68 | 138.53 | 140.41 |
| 20 | Solar (Existing IPPs) (MU) | 79.79 | 72.62 | 72.62 |
| 21 | Solar new purchase expected |  | 199.95 | 831.93 |
| 22 | Total Solar Energy available (MU) | 239.04 | 652.63 | 1635.51 |
| 23 | Balance Requirement Solar Energy (MU) | 411.27 | 228.24 | -444.11 |

10.7.57	M/s IEX,  as per submission before CERC in Petition No.25/MP/2019 dated 26th November 2018 ,  is preparing to launch Green Term-Ahead Market (Renewable Energy)Contracts at Indian Energy Exchange Ltd for transactions of renewable energy power. KSEBL is planning to procure the balance requirement through the green energy exchange as it materializes or through contracts at the ceiling rate of Rs 3.5 per unit for solar energy and Rs 3 per unit for non solar energy for FY-21. The total cost of renewable power is given in table below:

| Table 10. 63 Power Purchase Cost for renewable Energy procurement approved by the Honorable Commission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2019-20 |  |  | 2020-21 |  |  | 2021-22 |  |  |
| Source | Annual energy availability | Avg. Unit Cost | Total amount | Annual energy availability | AvgUnit Cost | Total amount | Annual energy availability | Avg. Unit Cost | Total amount |
|  | MU | Rs/unit | Rs Cr. | MU | Rs/unit | Rs Cr. | MU | Rs/unit | Rs Cr. |
| SHP Total | 88.65 | 3.48 | 30.84 | 88.65 | 3.48 | 30.84 | 88.65 | 3.48 | 30.84 |
| Wind Total | 121.97 | 3.74 | 45.61 | 121.97 | 3.74 | 45.61 | 121.97 | 3.74 | 45.61 |
| Solar Total | 70.08 | 3.9 | 27.33 | 70.08 | 3.9 | 27.33 | 70.08 | 3.9 | 27.33 |
| Additional Non-solar power procurement | 503.35 | 2.9 | 145.97 | 691.04 | 2.9 | 200.4 | 787.71 | 2.9 | 228.44 |
| Additional solar power (procurement) | 485.51 | 3 | 145.65 | 740.5 | 3 | 222.15 | 1,065.21 | 3 | 319.56 |
| Total | 1,269.55 | 3.11 | 395.4 | 1,688.64 | 3.12 | 526.33 | 2,179.04 | 2.99 | 651.78 |
| Revised Estimate |  |  |  |  |  |  |  |  |  |
| Existing SHEP | 82.87 | 3.51 | 29.07 | 88.48 | 3.48 | 30.75 | 88.48 | 3.48 | 30.75 |
| Existing Wind | 118.52 | 3.79 | 44.89 | 132.14 | 3.76 | 49.74 | 132.14 | 3.76 | 49.74 |
| Existing Solar | 79.79 | 3.81 | 30.38 | 72.62 | 3.81 | 27.64 | 72.62 | 3.81 | 27.64 |
| NTPC Floating Solar Project at Kayamkulam |  |  |  | 58.78 | 3.16 | 18.57 | 177.30 | 3.16 | 56.03 |
| Ground mounted THDCIL, Paivalike |  |  |  | 83.22 | 3.10 | 25.80 | 83.22 | 3.10 | 25.80 |
| Floating Solar project of NHPC at West Kallada.-10MW |  |  |  | 11.19 | 3.43 | 3.84 | 19.27 | 3.43 | 6.61 |
| Floating Solar project of NHPC at West Kallada-40 MW. |  |  |  | 38.44 | 3.43 | 13.18 | 77.09 | 3.43 | 26.44 |
| Solar project of THDCIL at Nellithadam, Kasargod |  |  |  | 8.32 | 3.10 | 2.58 | 8.32 | 3.10 | 2.58 |
| Floating solar at Cheruthoni |  |  |  |  |  |  | 96.36 | 3.16 | 30.45 |
| Floating solar at Anchuruli |  |  |  | 0.00 | 3.16 | 0.00 | 96.10 | 3.16 | 30.37 |
| Floating solar at Kulamavu |  |  |  | 0.00 | 3.16 | 0.00 | 96.10 | 3.16 | 30.37 |
| Floating solar at Banasurasagar |  |  |  | 0.00 | 3.16 | 0.00 | 96.10 | 3.16 | 30.37 |
| PM KUSUM |  |  |  | 8.30 | 3.50 | 2.90 | 16.64 | 3.50 | 5.83 |
| Power procurement through reverse bid process |  |  |  | 0.00 | 3.00 | 0.00 | 82.08 | 3.00 | 24.62 |
| PSA with SECI for purchase of Wind Power |  |  |  | 136.38 | 2.90 | 39.55 | 613.16 | 2.90 | 177.82 |
| PSA with SECI for purchase of Wind Power |  |  |  | 0.00 | 0.00 | 0.00 | 192.18 | 2.88 | 55.35 |
| Additional non solar power procurement |  |  |  | 464.92 | 2.90 | 134.83 |  |  |  |
| Additional solar power procurement |  |  |  | 228.24 | 3.00 | 68.47 |  |  |  |
| Total | 281.18 | 3.71 | 104.34 | 1331.03 | 3.14 | 417.86 | 1947.15 | 3.14 | 610.74 |

10.7.58	Sourcewise energy availability for 3 years

| Table 10. 64 Summary of energy availailty for remaining control period |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Financial Year | Generation(MU) |  |  |  |  | Power Purchase(MU) |  |  |  | Total Availability (MU) |
|  | Hydro | Thermal | Wind | Solar | Sub Total | Small IPPs (existing) | CGS | IPPs outside the State | RE contracted /roof top solar |  |
| 2019-20 | 6449.13 | 11.92 | 1.33 | 22.57 | 6484.95 | 281.18 | 10517.24 | 8111.05 | 0.00 | 25023.37 |
| 2020-21 | 6638.85 |  | 2.08 | 47.6 | 6688.53 | 293.23 | 11326.55 | 8648.62 | 498.65 | 27455.58 |
| 2021-22 | 6799.36 |  | 2.08 | 47.6 | 6849.04 | 293.23 | 11326.55 | 8648.62 | 2180.23 | 29297.67 |
| Above availability is exclusive of RE energy through banking |  |  |  |  |  |  |  |  |  |  |

Summary of monthwise energy availability

10.7.59	Based on the above the summary of the month-wise energy balance statement for the remaining part of FY-20, is as shown below

| Table 10. 65 : Summary of the total energy availability for remaining period of FY 2019-20 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Month | Demand | Generation (MU) |  |  | Power Purchase MU) |  |  | Credible outage on LTA and CGS | Total availability | Surplus/ Deficit |
|  |  | Hydro | Wind | Solar | IPPS | CGS | LTA(net) |  |  |  |
|  | MU | MU | MU | MU | MU | MU | MU | MU | MU | MU |
| Oct-19 | 2043.00 | 501.45 | 0.07 | 1.93 | 25.75 | 874.57 | 719.32 | 79.69 | 2043.39 | 0.39 |
| Nov-19 | 2080.00 | 479.92 | 0.06 | 1.76 | 19.24 | 966.01 | 696.12 | 83.11 | 2080.00 | 0.00 |
| Dec-19 | 2157.00 | 570.70 | 0.09 | 1.77 | 17.08 | 930.53 | 719.32 | 82.49 | 2157.00 | 0.00 |
| Jan-20 | 2119.53 | 474.60 | 0.12 | 2.21 | 16.67 | 992.20 | 719.32 | 85.58 | 2119.54 | 0.00 |
| Feb-20 | 2154.70 | 555.19 | 0.11 | 2.17 | 15.45 | 905.70 | 672.91 | 78.93 | 2072.60 | -82.10 |
| Mar-20 | 2555.95 | 834.65 | 0.13 | 2.68 | 19.97 | 912.83 | 719.32 | 81.61 | 2407.98 | -147.96 |
| Total | 13110.18 | 3416.51 | 0.58 | 12.51 | 114.15 | 5581.84 | 4246.32 | 491.41 | 12880.51 |  |

10.7.60	The month wise peak demand for the second half of FY 20 is given below.

|  | Table 10. 66 : Summary of Peak time availability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Month |  | Demand (MW) | Hydro (MW) | Outages (MW) | Small IPPs (MW) | CGS (MW) | LTA (MW) | UI (MW) | Total (MW) | Deficit(-)/Surplus(+) (MW) |
| Oct-19 |  | 3582 | 1448 | -100 | 60 | 1160.00 | 1000 | 50 | 3618.00 | 36 |
| Nov-19 |  | 3800 | 1448 | -100 | 60 | 1166.00 | 1000 | 50 | 3624.00 | -176 |
| Dec-19 |  | 3850 | 1389 | -100 | 60 | 1186.00 | 1000 | 50 | 3585.00 | -265 |
| Jan-20 |  | 3900 | 1489 | -100 | 60 | 1176.00 | 1000 | 50 | 3675.00 | -225 |
| Feb-20 |  | 4000 | 1480 | -100 | 50 | 1184.00 | 1000 | 50 | 3664.00 | -336 |
| Mar-20 |  | 4400 | 1743 | -100 | 30 | 1184.00 | 1000 | 50 | 3907.00 | -493 |

10.7.61	The month wise energy requirement and availability estimated for FY 21 is given below.

| Table 10. 67 : Summary of the total energy availability for FY 2020-21 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Month | Demand | Hydro | Wind | Solar incl Soura | Total Generation | Renewable energy existing &contracted | CGS | IPPs | Total availability | Deficit(-)/Surplus(+) |
| April | 2502.44 | 696.14 | 0.09 | 3.97 | 700.20 | 27.90 | 847.95 | 776.01 | 2352.06 | -150.38 |
| May | 2635.93 | 669.16 | 0.12 | 3.97 | 673.24 | 32.76 | 892.60 | 801.88 | 2400.48 | -235.45 |
| June | 2018.00 | 480.87 | 0.13 | 3.97 | 484.96 | 42.35 | 838.94 | 679.96 | 2046.20 | 28.21 |
| July | 2023.29 | 547.62 | 0.17 | 3.97 | 551.76 | 41.70 | 963.30 | 736.81 | 2293.56 | 270.27 |
| August | 1930.99 | 566.78 | 0.49 | 3.97 | 571.24 | 43.61 | 982.89 | 625.51 | 2223.26 | 292.27 |
| September | 2056.24 | 504.28 | 0.31 | 3.97 | 508.55 | 41.43 | 978.24 | 672.18 | 2200.41 | 144.17 |
| October | 2188.86 | 501.27 | 0.31 | 5.38 | 506.95 | 72.42 | 935.86 | 679.03 | 2194.26 | 5.40 |
| November | 2194.82 | 493.23 | 0.06 | 35.93 | 529.23 | 64.43 | 937.10 | 704.63 | 2235.38 | 40.56 |
| December | 2174.02 | 423.27 | 0.11 | 37.00 | 460.38 | 78.88 | 992.03 | 743.84 | 2275.12 | 101.10 |
| January | 2181.16 | 459.92 | 0.10 | 37.00 | 497.02 | 62.17 | 961.12 | 702.62 | 2222.93 | 41.77 |
| February | 2165.64 | 588.45 | 0.14 | 33.80 | 622.39 | 56.57 | 932.00 | 724.28 | 2335.24 | 169.60 |
| March | 2602.62 | 707.87 | 0.07 | 37.00 | 744.93 | 65.34 | 1064.53 | 801.88 | 2676.68 | 74.06 |
| Total | 26673.99 | 6638.85 | 2.08 | 209.91 | 6850.85 | 629.56 | 11326.55 | 8648.62 | 27455.58 | 781.59 |

10.7.62	The monthwise peak demand estimated for FY 21 is given below.

| Table 10. 68 : Summary of the peakdemand balance statement for FY 2020-21 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2020-21 | Demand expected | Availability(MW) |  |  |  |  |  | Shortage |
|  |  | Hydro | Small IPPs | CGS | LTA | Unforeseen outages | Total |  |
|  | MW | MW | MW | MW | MW | MW | MW | MW |
| April | 4445 | 1692 | 35 | 1403 | 1,103 | 125 | 4,108 | -338 |
| May | 4346 | 1661 | 35 | 1332 | 1,103 | 122 | 4,009 | -336 |
| June | 3925 | 1593 | 40 | 1288 | 968 | 113 | 3,776 | -149 |
| July | 3916 | 1548 | 50 | 1461 | 1,013 | 124 | 3,948 | 32 |
| August | 3812 | 1530 | 50 | 1486 | 862 | 117 | 3,810 | -2 |
| September | 3852 | 1548 | 50 | 1431 | 955 | 119 | 3,865 | 13 |
| October | 3836 | 1548 | 50 | 1403 | 933 | 117 | 3,817 | -20 |
| November | 3863 | 1498 | 50 | 1439 | 1,001 | 122 | 3,866 | 3 |
| December | 3916 | 1615 | 50 | 1497 | 1,024 | 126 | 4,059 | 143 |
| January | 3915 | 1596 | 115 | 1413 | 968 | 119 | 3,973 | 58 |
| February | 4007 | 1593 | 115 | 1384 | 1,103 | 124 | 4,071 | 64 |
| March | 4551 | 1694 | 105 | 1434 | 1,103 | 127 | 4,209 | -342 |

10.7.63	The month wise energy requirement and availability estimated for FY 22 is given below

| Table-10.69 Summary of energy availabiity for the year 2021-22 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2021-22 | Demand | Hydro | Wind | Solar* | Small IPPs/ RE purchase contracted | CGS | LTA | Total availability | Deficit (-) /Surplus(+) |
|  | MU |  |  |  |  |  |  |  |  |
| April | 2589.98 | 681.71 | 0.09 | 35.93 | 132.91 | 847.95 | 776.01 | 2474.60 | -115.38 |
| May | 2717.89 | 707.69 | 0.12 | 37.00 | 157.93 | 892.60 | 801.88 | 2597.21 | -120.67 |
| June | 2085.49 | 470.66 | 0.13 | 35.93 | 163.48 | 838.94 | 679.96 | 2189.10 | 103.61 |
| July | 2092.57 | 499.04 | 0.17 | 37.00 | 217.76 | 963.30 | 736.81 | 2454.07 | 361.50 |
| August | 1982.23 | 515.79 | 0.49 | 37.00 | 219.67 | 982.89 | 625.51 | 2381.36 | 399.13 |
| September | 2115.12 | 499.53 | 0.31 | 35.93 | 116.92 | 978.24 | 672.18 | 2303.11 | 188.00 |
| October | 2274.13 | 538.64 | 0.31 | 37.00 | 161.73 | 935.86 | 679.03 | 2352.56 | 78.44 |
| November | 2261.98 | 556.51 | 0.06 | 66.53 | 150.86 | 937.10 | 704.63 | 2415.68 | 153.70 |
| December | 2246.01 | 483.18 | 0.11 | 68.62 | 153.13 | 992.03 | 743.84 | 2440.90 | 194.89 |
| January | 2242.40 | 500.11 | 0.10 | 68.62 | 155.87 | 961.12 | 702.62 | 2388.44 | 146.04 |
| February | 2290.05 | 569.97 | 0.14 | 62.36 | 141.21 | 932.00 | 724.28 | 2429.95 | 139.90 |
| March | 2731.42 | 776.54 | 0.07 | 68.62 | 159.04 | 1064.53 | 801.88 | 2870.68 | 139.26 |
| Total | 27629.24 | 6799.36 | 2.08 | 590.56 | 1930.50 | 11326.55 | 8648.62 | 29297.67 | 1668.43 |
| * Solar incl rooftop under Saura scheme and PM KUSUM scheme |  |  |  |  |  |  |  |  |  |

10.7.61	The month wise peakdemand for FY22 is given below.

| Table 10.70 Summary of peak demand balance statement for for 2021-22 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2021-22 | Demand expected(MW) | Availability(MW) |  |  |  |  |  |  | Shortage |
|  |  | Hydro | Small IPPs/RE purchase* | Proposed wind and solar | CGS | LTA | Unforeseen outages | Total |  |
|  | MW | MW | MW | MW | MW | MW | MW | MW | MW |
| April | 4579 | 1646 | 105 | 35 | 1403 | 1,103 | 125.31 | 4131.89 | -446.95 |
| May | 4476 | 1631 | 105 | 35 | 1332 | 1,103 | 121.75 | 4049.17 | -426.76 |
| June | 4079 | 1500 | 120 | 105 | 1288 | 968 | 112.80 | 3763.29 | -315.81 |
| July | 4032 | 1500 | 165 | 105 | 1461 | 1,013 | 123.68 | 4014.96 | -17.52 |
| August | 3926 | 1530 | 165 | 105 | 1486 | 862 | 117.38 | 3925.19 | -1.20 |
| September | 3956 | 1548 | 165 | 105 | 1431 | 955 | 119.30 | 3979.79 | 24.02 |
| October | 3928 | 1548 | 165 | 105 | 1403 | 933 | 116.77 | 3931.56 | 3.58 |
| November | 3952 | 1498 | 165 | 105 | 1439 | 1,001 | 122.02 | 3981.34 | 29.25 |
| December | 4008 | 1615 | 165 | 105 | 1497 | 1,024 | 126.01 | 4174.13 | 166.44 |
| January | 4005 | 1596 | 160 | 105 | 1413 | 968 | 119.05 | 4017.92 | 13.29 |
| February | 4091 | 1593 | 160 | 105 | 1384 | 1,103 | 124.39 | 4116.34 | 25.03 |
| March | 4714 | 1694 | 150 | 105 | 1434 | 1,103 | 126.86 | 4254.25 | -459.75 |

Purchase from Short term market-Purchase for FY-20

10.7.62	The summary of energy balance statement for the first half of financial year is as follows.

| Table 10. 71 Energy Balance Statement during first half of FY-20 |  |  |  |
| --- | --- | --- | --- |
| Particulars | Approved | Actual | Deviation |
| Hydro | 4036.74 | 3002.50 | -1034.24 |
| Generation from other sources | 19.71 | 17.05 | -2.66 |
| Small IPP | 140.73 | 172.18 | 31.44 |
| Additional RE approved for meeting RPO(proportionate) | 275.18 | 0.00 | -275.18 |
| CGS(exbus) | 5729.70 | 5123.23 | -606.47 |
| IPPs outside State*(ex bus) | 4405.05 | 4030.38 | -374.67 |
| Demand | 12718.00 | 12928.08 | 210.08 |

10.7.62	It can be seen that during the first half of financial year, a reduction of 1034 MU in hydro, reduction of power purchase of 606.17 MU from CGS, reduction of 374 MU from LTA causes the reduction in availability by 2261 MU. Further the demand during April reaches 4316 MW and daily demand during April touches 88 MU.It may please be noted that the average daily consumption during April was 81 MU.There is an increase of 210 MU in enery demand during the first half of FY-2020.In order to meet the demand and to compensate the deficit KSEBLtd purchased 1225.27 MU through short term markets partially to meet the deficit in summer months of FY 2020, for a total cost of Rs 369.37Cr.

10.7.64	KSEBL has already filed a petition before the Honorable Commission to enter into banking arrangement with M/s PTC India Ltd and M/s BSES Yamuna Power Ltd, with delivery point as Regional Periphery of Exporting Utility as well as return period as given in Table below

| Table 10.72 Details of banking arrangements made for FY 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Supply thru PTC from TPDDL to KSEB |  |  |  |  |  | Return from KSEBL to PTC-TPDDL |  |  |  |  |  |  |
| Supply Period |  | Quantum(MW) | Duration(Hrs) |  |  | Return Period |  | Duration (Hrs) | Return (%) |  | Trading Margin(Ps/kWh) |  |
| 01.02.20 to 29.02.20 |  | 50 | RTC |  |  | 16.06.2020 to 30.09.2020 |  | 0-4 & 17-18.30 & 22.30 to 24.00 (uniformly distributed) | 102% |  | 2.98 |  |
|  |  | 50 | 0-6 |  |  |  |  |  |  |  |  |  |
|  |  | 100 | 20-24 |  |  |  |  |  |  |  |  |  |
| 01.03.2020 to 31.03.2020 |  | 50 | RTC |  |  |  |  |  |  |  |  |  |
|  |  | 100 | 0-6 |  |  |  |  |  |  |  |  |  |
|  |  | 50 | 18-20 |  |  |  |  |  |  |  |  |  |
|  |  | 100 | 20-24 |  |  |  |  |  |  |  |  |  |
| 01.04.2020 to 30.04.2020 |  | 50 | RTC |  |  |  |  |  |  |  |  |  |
|  |  | 100 | 0-6 |  |  |  |  |  |  |  |  |  |
|  |  | 25 | 18-23 |  |  |  |  |  |  |  |  |  |
| Supply from BYPL to KSEBL |  |  |  | Return from KSEBL to BYPL |  |  |  |  |  |  |  |
| Supply Period | Quantum (MW) |  | Duration (Hrs) | Return period |  | Duration (Hrs) |  |  | Return (%) |  | Trading Margin(Ps/kWh) |
| 16.02.20 to 29.02.20 | 75 |  | RTC | 16.06.2020 to 30.09.20 |  | 0-3 & 13-17 & 22-24.00(unifomly distributed) |  |  | 103% |  | NIL |
| 01.03.20 to 31.03.20 | 75 |  | RTC |  |  |  |  |  |  |  |  |

10.7.65	The quantum of power to be supplied to KSEBL as well as returned by KSEBL is given in table below:

| Table 10. 73 Quantum of power transactions thru banking |  |  |  |  |
| --- | --- | --- | --- | --- |
| Source | Supply |  | Return |  |
| PTC-TPDDL | Month | MU | Month | MU |
|  | Feb-20 | 55.1 | Jun-20 | 26.33 |
|  | Mar-20 | 71.3 | Jul-20 | 54.42 |
|  | Apr-20 | 57.75 | Aug-20 | 54.42 |
|  |  |  | Sep-20 | 52.66 |
| BYPL | Feb-20 | 27.00 | Jun-20 | 11.96 |
|  | Mar-20 | 55.80 | Jul-20 | 24.71 |
|  |  |  | Aug-20 | 24.71 |
|  |  |  | Sep-20 | 23.91 |

After considering the power through banking arrangements, there is a deficit of 20.86 MU. KSEBL is estimating power procurement of 20.86 MU @ Rs 4.24 per unit (Realised price of DB Power through DEEP portal) from day ahead market. KSEBL returned 286.78 MU through various banking arrangements approved by Honorble Commission and sold 55.93 MU for the first half of FY 20. The details are given in table below:

| Table 10. 74 Short term procurement for FY 2020 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Month | Peak demand |  |  | Energy availability |  |  | Short term | Power expected thru PTC_TPDDL & BYPL thru banking | External sale | Swap return |
|  | Demand | Peak Availability | Surplus(+) /Deficit(-)) | Demand | Availability after credible outages | Surplus(+) /Deficit(-)) |  |  |  |  |
|  | MW | MW | MW | MU | MU | MU | MU | MU | MU | MU |
| April -Sept |  |  |  | 13025.36 | 12,142.86 | -882.50 | 1,225.27 |  | 55.93 | 286.84 |
| Oct-19 | 3582 | 3618.00 | 36.34 | 2,043.00 | 2,043.39 | 0.39 |  |  | 0.39 |  |
| Nov-19 | 3800 | 3624.00 | -176.00 | 2,080.00 | 2,080.00 | 0.00 |  |  |  |  |
| Dec-19 | 3850 | 3585.00 | -265.00 | 2,157.00 | 2,157.00 | 0.00 |  |  |  |  |
| Jan-20 | 3900 | 3675.00 | -225.00 | 2,119.53 | 2,119.54 | 0.00 |  |  |  |  |
| Feb-20 | 4000 | 3664.00 | -336.00 | 2,154.70 | 2,072.60 | -82.10 |  | 82.1 |  |  |
| Mar-20 | 4400 | 3907.00 | -493.00 | 2,555.95 | 2,407.98 | -147.96 | 20.86 | 127.1 |  |  |
| Total |  |  |  | 26,135.54 | 25,023.37 | -1112.17 | 1,246.13 |  | 56.32 | 286.84 |
| Remaining power purchase through short term needed for period from October to March 2020 |  |  |  |  |  |  | 20.86 |  |  |  |
| External sale expected for period from October to March 2020 |  |  |  |  |  |  | 0.39 |  |  |  |

10.7.66	Thus the total power purchase through short term markets during FY 2020 quantifies to 1287.65 MU for a total amout of Rs 378.23 Cr.

| Table 10. 75 : Purchase from short term market |  |  |  |
| --- | --- | --- | --- |
| No | Particulars Purchase | Energy at Kerala periphery (MU) | Amount (Rs Cr.) |
| 1 | During first part of financial year | 1,225.27 | 369.38 |
| 2 | Remaining part of financial year | 20.86 | 8.85 |
|  | Total | 1,246.13 | 378.23 |

Short term/Medium Term Purchase for FY-21

10.7.68	The energy availability statement and the short term procurement for the year FY -21 is given in Table 10.67 above. It is seen that there is a shortage of about 600 MW during April and May 2020 and March 2021.  As stated in para above, KSEBL arranged 57.75 MU during the month of April 2020 through banking arrangements with PTC-TPDDL.

10.7.69	Considering the energy surplus during monsoon months KSEBL is planning to make similar banking arrangements done in FY 20, for the period from February to April 2021 also to the extent possible. It may be noted that surplus energy for return of banked energy is available mostly during off peak hours in monsoon months. Thus the surplus will be materializing only in case monsoon strengthens during this period. Also, there is only limited number of utilities in need of power during this period. Even these utilities are looking for RTC power, which may not be feasible for KSEBL due to inadequate availability during peak period.

10.7.70	Further inorder to meet the energy deficit during April and May 2020, KSEBL has already taken steps to avoid power crisis and to avoid scheduling of high cost internal thermal generating stations. KSEBL issued letter of award to M/s Adani enterprises and M/s DB Power Ltd purchase RTC and peak power from 01-04-2020 to 30.04.2020 from Adani Enterprises Ltd on firm and short term basis through DEEP portal as per the prices discovered as shown in table below.

| Table 10. 76 Details of power contracted through DEEP portal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Generator | Adani Enterprises Ltd |  |  |  |
| Period | Duration | Quantum(MW) on firm basis | Rate at delivery point ie; at Kerala periphery(Rs/unit) | Source of power |
| 01.04 to 30.04.2020 | 00.00 to 24.00 | 100 | 3.72 | Raipur Energen Ltd |
| 01.05. to 31.05.2020 | 00.00 to 24.00 | 150 | 3.76 |  |
| 01.05. to 31.05.2020 | 19.00 to 23.00 | 50 | 4.25 |  |
| Generator | DB Power Ltd(DBPL) |  |  |  |
| Period | Duration | Quantum(MW) on firm basis | Rate at delivery point ie; at Kerala periphery(Rs /unit) | Source of power |
| 01.04. to 30.04.2020 | 19.00 to 23.00 | 50 | 4.24 | DB Power Ltd |
| 01.05 to 31.05.2020 | 19.00 to 23.00 | 50 | 4.24 |  |

10.7.71. The energy expected from the above purchase arrangements already done is given in table below

| Table 10.77 Banking arrangements already done for FY 20-21 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Source | Supply to KSEBL |  | Return by KSEBL |  |
| PTC-TPDDL | Month | MU | Month | MU |
|  |  |  | Jun-20 | 26.33 |
|  |  |  | Jul-20 | 54.42 |
|  | Apr-20 | 57.75 | Aug-20 | 54.42 |
|  |  |  | Sep-20 | 52.66 |
| BYPL |  |  | Jun-20 | 11.96 |
|  |  |  | Jul-20 | 24.71 |
|  |  |  | Aug-20 | 24.71 |
|  |  |  | Sep-20 | 23.91 |
| Power procurement from M/Adani and M/s DB Power Ltd |  |  |  |  |
|  | Month | MU |  |  |
| Adani Enterprises Ltd | Apr-20 | 72 |  |  |
|  | May-20 | 117.8 |  |  |
| DB Power Ltd | Apr-20 | 6 |  |  |
|  | May-20 | 6.2 |  |  |
|  |  |  | Aug-21 | 24.71 |
|  |  |  | Sep-21 | 23.91 |

10.7.72	The energy balance statement of FY 21 is given in table below

| Table 10. 78 Energy balance statement for FY-21 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2020-21 | Demand expected | Availability | Deficit(-)/ Surplus(+) | Energy demand | Energy availability | Energy Deficit/surplus | Power availed through banking arrangement | Power purchase through contracts | Swap return | Total availability | Energy Deficit/surplus |
| Month | MW | MW | MW | MU | MU | MU | MU | MU | MU | MU | MU |
| April | 4445 | 4,108 | -338 | 2502.44 | 2352.06 | -150.38 | 57.75 | 78.00 |  | 2487.81 | -14.63 |
| May | 4400 | 4,009 | -336 | 2635.93 | 2400.48 | -235.45 |  | 124.00 |  | 2524.48 | -111.45 |
| June | 3925 | 3,776 | -149 | 2018.00 | 2046.20 | 28.21 |  |  | -38.29 | 2007.91 | -10.08 |
| July | 3916 | 3,948 | 32 | 2023.29 | 2293.56 | 270.27 |  |  | -79.13 | 2214.43 | 191.14 |
| August | 3812 | 3,810 | -2 | 1930.99 | 2223.26 | 292.27 |  |  | -79.13 | 2144.13 | 213.14 |
| Sept | 3852 | 3,865 | 13 | 2056.24 | 2200.41 | 144.17 |  |  | -76.57 | 2123.84 | 67.60 |
| Oct | 3836 | 3,817 | -20 | 2188.86 | 2194.26 | 5.40 |  |  |  | 2194.26 | 5.40 |
| Nov | 3863 | 3,866 | 3 | 2194.82 | 2235.38 | 40.56 |  |  |  | 2235.38 | 40.56 |
| Dec | 3916 | 4,059 | 143 | 2174.02 | 2275.12 | 101.10 |  |  |  | 2275.12 | 101.10 |
| Jan | 3915 | 3,973 | 58 | 2181.16 | 2222.93 | 41.77 |  |  |  | 2222.93 | 41.77 |
| Feb | 4007 | 4,071 | 64 | 2165.64 | 2335.24 | 169.60 |  |  |  | 2335.24 | 169.60 |
| March | 4551 | 4,209 | -342 | 2602.62 | 2676.68 | 74.06 |  |  |  | 2676.68 | 74.06 |
|  |  |  |  | 26673.99 | 27455.58 | 781.59 | 57.75 | 202.00 | -273.12 | 27442.21 | 768.22 |

10.7.73 It can be seen that there is a peak deficit during summer months even after the above arrangements and hence KSEBL is planning to meet this requirement through short term or medium term contracts. As the deficit during the period other than summer months is negligible, in this instant petition KSEBL is estimating the cost of the power purchase arrangements for summer months only. In order to meet the peak deficit and energy shortage KSEBL is planning to procure 250 MW for 18.00 to 24.00 hours during March 2021 (46.5 MU) and the corresponding power purchase cost is estimated @ clearing price of DB power Ltd ie Rs 4.24 per unit and to meet the energy deficit during April-June through short term/medium term contracts quantifying (14.63+111.45+10.08 MU=136.17 MU) @average rate of Rs 4.24/unit. Thus the energy procurement through short term/medium term contracts for the year FY 2020-21 is 384.67 MU (46.5 MU +136.17 MU+ 202 MU). The figures might change during course of time based on progress of monsoon, actual unit availability from various sources, demand variations etc. The corresponding power purchase cost amounts to Rs 154.01 Cr. The details are given in table below

| Table 10.79 Energy procurement through short term/medium term contracts for the year FY 2020-21 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Source | Month | Duration | Quantum (MW) on firm basis | Rate at Kerala periphery (Rs/unit) | Quantum (MU) | Power Purchase cost(Rs Cr) |
| Adani enterprises Ltd | Apr-20 | 00.00 to 24.00 | 100 | 3.72 | 72 | 26.78 |
|  | May-20 | 00.00 to 24.00 | 150 | 3.76 | 111.6 | 41.96 |
|  | May-20 | 19.00 to 23.00 | 50 | 4.25 | 6.2 | 2.64 |
| DB Power Ltd | Apr-20 | 19.00 to 23.00 | 50 | 4.24 | 6 | 2.54 |
|  | May-20 | 19.00 to 23.00 | 50 | 4.24 | 6.2 | 2.63 |
| Short term/ Medium Term | April-June |  |  | 4.24 | 136.17 | 57.74 |
|  | Mar-21 | 18.00 to 24.00 | 250 | 4.24 | 46.5 | 19.72 |
| Sub total |  |  |  |  | 384.67 | 154.01 |

10.7.74	It can be seen from the above table that there is a surplus energy availability of 1644.05 MU during the year 2020-21 as shown in table below. KSEBL is planning to sell the surplus energy only if the rate is more than the marginal cost of high cost generating stations such as Kudgi, NTPL,NTECL Vallur  etc. In case such opportunistic sale does not materializes in real time power from these stations with high variable cost will be surrendered which will reduce the power purchase cost. For estimation purpose this surplus energy is shown as sale at a rate of Rs 4/unit is considered through short term market. The corresponding expected revenue is Rs 657.62Cr.

| Table 10. 80 Surplus energy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2020-21 | Energy demand | Availability | Renewable energy proposed to procure through short term markets for meeting RPO | Swap /Short term/ medium term contracted/ proposed | Swap return | Total availability | Surplus sale |
| Month | MU | MU | MU | MU | MU | MU | MU |
| April | 2502.44 | 2352.06 |  |  |  |  |  |
| May | 2635.93 | 2400.48 |  |  |  |  |  |
| June | 2018.00 | 2046.20 |  |  |  |  |  |
| July | 2023.29 | 2293.56 |  |  |  |  |  |
| August | 1930.99 | 2223.26 |  |  |  |  |  |
| September | 2056.24 | 2200.41 |  |  |  |  |  |
| October | 2188.86 | 2194.26 |  |  |  |  |  |
| November | 2194.82 | 2235.38 |  |  |  |  |  |
| December | 2174.02 | 2275.12 |  |  |  |  |  |
| January | 2181.16 | 2222.93 |  |  |  |  |  |
| February | 2165.64 | 2335.24 |  |  |  |  |  |
| March | 2602.62 | 2676.68 |  |  |  |  |  |
| Total | 26673.99 | 27455.58 | 693.17 | 442.42 | -273.12 | 28318.043 | 1644.05 |

Short term/Medium Term Purchase for FY-22

10.7.75	The energy availability statement and the short term procurement for the year FY -22 is given in Table 10.67 above. It is seen that there is a deficit of 300 to 600 MW during summer months

10.7.76 Considering the energy surplus during monsoon months KSEBL will explore the possibilities of banking arrangements as done in FY 20, for the period from February 2021 to April 2022 also to the extent possible. It may be noted that surplus energy for return of banked energy is available mostly during off peak hours in monsoon months. Thus the surplus will be materializing only in case monsoon strengthens during this period. Also, there is only limited number of utilities in need of power during this period. Even these utilities are looking for RTC power, which may not be feasible for KSEBL due to inadequate availability during peak period. The energy balance statement for the year FY 22 is given in table below.

| Table 10. 81 Energy balance statement for FY -22 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2021-22 | Demand expected | Availability | Deficit(-)/ Surplus(+) | Energy demand | Energy availability | Deficit(-)/ Surplus(+) |
| Month | MW | MW | MW | MU | MU | MU |
| April | 4579 | 4131.89 | -446.95 | 2589.98 | 2,474.60 | -115.38 |
| May | 4476 | 4049.17 | -426.76 | 2717.89 | 2,577.21 | -140.67 |
| June | 4079 | 3763.29 | -315.81 | 2085.49 | 2,189.10 | 103.61 |
| July | 4032 | 4014.96 | -17.52 | 2092.57 | 2,474.07 | 381.50 |
| August | 3926 | 3925.19 | -1.20 | 1982.23 | 2,441.36 | 459.13 |
| September | 3956 | 3979.79 | 24.02 | 2115.12 | 2,303.11 | 188.00 |
| October | 3928 | 3931.56 | 3.58 | 2274.13 | 2,377.56 | 103.44 |
| November | 3952 | 3981.34 | 29.25 | 2261.98 | 2,415.68 | 153.70 |
| December | 4008 | 4174.13 | 166.44 | 2246.01 | 2,425.90 | 179.89 |
| January | 4005 | 4017.92 | 13.29 | 2242.40 | 2,383.44 | 141.04 |
| February | 4091 | 4116.34 | 25.03 | 2290.05 | 2,389.95 | 99.90 |
| March | 4714 | 4254.25 | -459.75 | 2731.42 | 2,845.68 | 114.26 |
|  |  |  |  | 27629.24 | 29297.67 | 1,668.43 |

10.7.77 It can be seen that there is a peak deficit during summer months and apart from banking KSEBL is planning to meet this requirement through short term or medium term contracts. As the deficit during the period other than summer months is negligible, in this instant petition KSEBL is estimating the cost of the power purchase arrangements for summer months only. In order to meet the peak deficit and energy shortage KSEBL is planning to procure 400 MW for for April-June  2021 and March 2022 from 18.00 to 24.00 hours amounting to 292.80 MU and the corresponding power purchase cost is estimated  @ clearing price of DB power Ltd for FY-20 ie Rs 4.24 per unit amounting to Rs 124.15 Cr. The remaining energy deficit (109.65 MU) will be met through swap arrangements or dayahead markets and for estimation purpose it is shown as a short term purchase at Rs 4.24 per unit. These figures are highly susceptible to monsoon pattern, unit availability at different generating stations and actual demand etc. Thus the energy procurement through short term/medium term contracts for the year FY 2021-22 is 402.45 MUas shown in table below and the corresponding power purchase cost amounts to Rs 170.64 Cr.

| Table 10. 82 Short term procurement for FY-22 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2021-22 | Demand expected | Availability | Deficit(-)/ Surplus(+) | Energy demand | Energy availability | Deficit(-)/ Surplus(+) | Short term 400 MW | Remaining deficit through short term/swap | Surplus energy |
| Month | MW | MW | MW | MU | MU | MU | MU | MU | MU |
| April | 4579 | 4131.89 | -446.95 | 2589.98 | 2,474.60 | -115.38 | 72 | 43.38 | 0 |
| May | 4476 | 4049.17 | -426.76 | 2717.89 | 2,577.21 | -140.67 | 74.4 | 66.27 | 0 |
| June | 4079 | 3763.29 | -315.81 | 2085.49 | 2,189.10 | 103.61 | 72 |  | 175.61 |
| July | 4032 | 4014.96 | -17.52 | 2092.57 | 2,474.07 | 381.50 |  |  | 381.50 |
| August | 3926 | 3925.19 | -1.20 | 1982.23 | 2,441.36 | 459.13 |  |  | 459.13 |
| September | 3956 | 3979.79 | 24.02 | 2115.12 | 2,303.11 | 188.00 |  |  | 188.00 |
| October | 3928 | 3931.56 | 3.58 | 2274.13 | 2,377.56 | 103.44 |  |  | 103.44 |
| November | 3952 | 3981.34 | 29.25 | 2261.98 | 2,415.68 | 153.70 |  |  | 153.70 |
| December | 4008 | 4174.13 | 166.44 | 2246.01 | 2,425.90 | 179.89 |  |  | 179.89 |
| January | 4005 | 4017.92 | 13.29 | 2242.40 | 2,383.44 | 141.04 |  |  | 141.04 |
| February | 4091 | 4116.34 | 25.03 | 2290.05 | 2,389.95 | 99.90 |  |  | 99.90 |
| March | 4714 | 4254.25 | -459.75 | 2731.42 | 2,845.68 | 114.26 | 74.4 |  | 188.66 |
|  |  |  |  | 27629.24 | 29297.67 | 1,668.43 | 292.8 | 109.65 | 2070.88 |
| 400 MW from 18.00 to 24.00 during April ,May ,June 2021 and d March 2022 |  |  |  |  |  |  |  |  |  |

10.7.78 It can be seen from the above table that there is a surplus energy of 2070.88 MU during the year 2021-22. KSEBL decided that the sale of surplus energy will be done only if the rate is more than the marginal cost of high cost generating stations such as Kudgi, NTPL, NLC II expansion etc. In case such opportunistic sale does not materializes in real time, power from these stations with high variable cost will be surrendered which will reduce the power purchase cost. For estimation purpose this surplus energy is shown as sale at a rate of Rs 4/unit through short term market. The corresponding expected revenue is Rs 828.35 Cr.

10.7.79	Surplus Energy Sale: The surplus energy sale approved by the Honorable Commission for the control period is given in table below.

| Table 10. 83 Surplus Energy Sales for the control period |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Approved by the Honorable Commission |  |  |  |  |  |
| Particulars | Unit | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| Surplus energy for external sales | MU | 1,513.27 | 1,885.40 | 1,231.36 | 811.21 |
| Rate | Rs./kWh | 5 | 5 | 5 | 5 |
| Total Revenue expected | Rs.Crore | 756.64 | 942.7 | 615.68 | 405.61 |

10.7.80	Based on the availability and demand there is a surplus energy as explained above. KSEBL has been taking earnest efforts to sell the excess power   whenever opportunity is available. The details of proposed energy sale is given in the table below

| Table 10. 84 Surplus Energy Sales for the control period |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Approved by the Honorable Commission |  |  |  |  |  |  |  |  |  |
| Particulars | Unit | 2018-19 |  | 2019-20 |  | 2020-21 |  |  | 2021-22 |
| Energy for external sales | MU | 1,513.27 |  | 1,885.40 |  | 1,231.36 |  |  | 811.21 |
| Rate | Rs./kWh | 5 |  | 5 |  | 5 |  |  | 5 |
| Total Revenue expected | Rs.Crore | 756.64 |  | 942.7 |  | 615.68 |  |  | 405.61 |
| Revised Estimate |  |  |  |  |  |  |  |  |  |
| Particulars | Unit | FY-19 | April-Sept 2019 |  | 2019-20 |  |  | 2020-21 | 2021-22 |
|  |  | Actuals |  |  | Estimate |  |  |  |  |
| Energy for external sales | MU | 824.78 | 55.93 |  | 56.32 |  | 1644.05 |  | 2,070.88 |
| Rate | Rs./kWh | 4.11 | 3.12 |  | 3.12 |  | 4.00 |  | 4.00 |
| Total Revenue expected | Rs.Crore | 339.34 | 17.45 |  | 17.56 |  | 657.62 |  | 828.35 |

Summary of cost of generation and power purchase for the control period

10.7.81	Approved power purchase cost for the control period is extracted below:

| Table 10. 85 Summay of Power Purchase cost for the control period |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Approved by Honorable Commission |  |  |  |  |  |  |  |  |
|  | ENERGY at KSEB end (MU) |  |  |  | COST (Rs.Crore) |  |  |  |
|  | 2018-19 | 2019-20 | 2020-21 | 2021-22 | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| CGS | 10,255.41 | 11,241.96 | 11,274.77 | 11,274.77 | 3,684.51 | 4,135.05 | 4,196.25 | 4,244.91 |
| LTA | 7,999.31 | 8,647.06 | 8,647.06 | 8,647.07 | 3,319.64 | 3,528.02 | 3,518.11 | 3,510.51 |
| IPPs | 280.7 | 280.7 | 280.7 | 280.7 | 103.78 | 103.78 | 103.78 | 103.78 |
| Additional RPO purchase -Non Solar | - | 503.35 | 691.04 | 787.71 | - | 145.97 | 200.4 | 228.44 |
| Additional RPO purchase -Solar | - | 485.51 | 740.5 | 1,065.21 | - | 145.65 | 222.15 | 319.56 |
| RGCCPP |  |  |  |  | 200 |  |  |  |
| Transmission charges |  |  |  |  |  |  |  |  |
| CGS |  |  |  |  | 419.72 | 435.34 | 437.12 | 437.12 |
| LTA |  |  |  |  | 111.96 | 111.96 | 111.96 | 111.96 |
| RGCCPP |  |  |  |  | 8.16 | 8.16 | 8.16 | 8.16 |
| Total | 18,535.42 | 21,158.58 | 21,634.07 | 22,055.46 | 7,847.77 | 8619.08 | 8747.85 | 9265.79 |

10.7.82	The revised estimate of power purchase cost for control period is given in table below:

| Table 10. 86 Summary of Power purchase Cost for the control period |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | Energy (MU) |  |  |  |  | Power Purchase cost(Rs Cr) |  |  |  |  |
|  | FY -19 | April-Sept 2019 | FY -20 | FY -21 | FY -22 | FY -19 | April-Sept 2019 | FY -20 | FY -21 | FY -22 |
| CGS* | 9241.09 | 4935.40 | 10517.24 | 11326.55 | 11326.55 | 3418.17 | 1890.34 | 4041.51 | 4526.22 | 4667.17 |
| LTA | 7652.05 | 4,246.32 | 8,111.05 | 8,648.62 | 8,648.62 | 3314.19 | 1668.18 | 3492.20 | 3737.65 | 3769.23 |
| IPPs inside the State | 268.48 | 167.03 | 281.18 | 293.23 | 293.23 | 101.08 | 60.06 | 104.34 | 108.13 | 108.13 |
| RE energy contracted for control period |  |  |  | 498.65 | 2180.23 |  |  |  | 103.52 | 496.79 |
| Additional RPO purchase -Non Solar |  |  |  | 464.92 | 0.00 |  |  |  | 134.83 | 0.00 |
| Additional RPO purchase -Solar |  |  |  | 228.24 | 0.00 |  |  |  | 68.47 |  |
| RGCCPP |  |  |  |  |  | 204.13 | 31.18 | 62.35 | 62.35 | 62.35 |
| Short term | 872.21 | 1225.27 | 1246.13 | 384.6692 | 402.45 | 235.84 | 369.38 | 379.81 | 154.01 | 170.64 |
| Transmission charges |  |  |  |  |  |  |  |  |  |  |
| CGS |  |  |  |  |  |  | 171.73 | 408.51 | 425.04 | 425.04 |
| LTA |  |  |  |  |  |  | 44.62 | 106.27 | 106.27 | 106.27 |
| RGCCPP |  |  |  |  |  |  | 4.20 | 8.16 | 8.16 | 8.16 |
| Subtotal |  |  |  |  |  | 513.24 | 220.55 | 522.94 | 539.47 | 539.47 |
| Other charges |  |  |  |  |  | 40.033 |  |  |  |  |
| Total | 18033.83 | 10574.02 | 20155.60 | 21844.88 | 22851.08 | 7826.68 | 4239.68 | 8603.15 | 9434.65 | 9813.79 |

10.7.83	Thus the revised generation and power purchase cost of SBU D for FY 20, FY-21 and FY-22 is given below

| Table 10. 87 Total Generation and Power Purchase-Revised estimate |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Financial Year | Net Generation | Cost of Generation | Quantum at KSEB end | Cost of Power Purchase | Total |  |
|  | MU | Rs Cr | MU | Rs.Cr. | MU | Rs. Cr. |
| 2018-19 | 7593.12 | 510.36 | 18046.53 | 7826.68 | 25639.65 | 8337.04 |
| April-Sept 2019 | 3901.58 | 322.34 | 10574.02 | 4386.83 | 14475.6 | 4709.17 |
| 2019-20 | 6471.48 | 480.25 | 20155.60 | 8603.15 | 26627.081 | 9083.40 |
| 2020-21 | 6688.53 | 534.47 | 21844.88 | 9434.65 | 28533.413 | 9969.12 |
| 2021-22 | 6849.04 | 659.44 | 22851.08 | 9813.79 | 29700.121 | 10473.23 |

10.8	Intra State Transmission Charges

10.8.1	The details of Intra State Transmission Charges are explained in Chapter 3 of this petition. A summary of revised estimate of intrastate transmission charges for next control period is given in table below:

| Table 10. 88 Intra State Transmission Charges |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Item | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| 1 | Energy injected into the system (MU) | 24897.52 | 26135.55 | 26673.99 | 27629.24 |
| 2 | Percentage of loss (%) | 3.74 | 3.65 | 3.55 | 3.45 |
| 3 | Loss of energy (MU) | 1008.35 | 953.9475 | 946.9266 | 953.2087 |
| 4 | Energy handled (MU) (1-3) | 24893.47 | 25181.6 | 25727.06 | 26676.03 |
| 5 | Transmission charges | 858.64 | 953.84 | 1190.45 | 1444.24 |
| 6 | Transmission Charges (Rs./unit) | 0.34 | 0.38 | 0.46 | 0.54 |

10.9	NLDC/RLDC/SLDC charges

10.9.1	An amount of 3 crores is approved in the MYT order towards NLDC/RLDC/SLDC charges vide MYT order dated 08.07.2019. KSEBLtd is claiming the same for the remaining part of the control period.

10.10	Operation & Maintenance Expenses

10.10.1	As per Regulation 79(1) of the tariff regulation, operation and maintenance expenses as per the norms specified in Annexure-IX are allowed to be recovered.Regulation allows recovery of the R&M component of O&M expenses viz the R& M expenses at 3 % of opening GFA exckluding land along with pro rata allowance for the asset addition during the year. Honorable Commission may please note that the land and land rights amounts to Rs 19.91 Crores as on 31.03.2019. The gross fixed assets of Distribution SBU as on 31.03.2019 is Rs 9450.31 crores.Thus land and land rights contributes to .21 % of total GFA for SBU-D. Hence KSEBL is taking 0.21 % of Gross fixed assets as assets of land and landrights for the remaining part of control period. The parameters used for the The Operation & Maintenance Expenses Claimed and approved for SBU-D of KSEBL for the control period from 2018-19 to 2021-22 is shown in Table below.

| Table 10. 89 O&M Expenses claimed vs approved |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Employee Cost and A&G cost projection-KSEBL claim in MYT petition |  |  |  |  |  |
| No | Parameters | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| 1 | No. of consumers (Rs.L/1000) | 4.8 | 5.03 | 5.27 | 5.53 |
| 2 | Dist Transformers (Rs. L/DTr) | 0.64 | 0.67 | 0.7 | 0.73 |
| 3 | Length of HT line (Rs. L/km) | 0.79 | 0.83 | 0.87 | 0.91 |
| 4 | Energy sales (Rs/unit) | 0.19 | 0.2 | 0.21 | 0.22 |
| 5 | No. of consumers | 12611100 | 12957080 | 13314526 | 13683932 |
| 6 | No. of Dist Transformers (Nos) | 80064 | 81537 | 82897 | 83950 |
| 7 | HT Line (Ckt-Km) | 66225 | 69686 | 72654 | 74729 |
| 8 | Energy sales in MU | 21647.29 | 22969.67 | 23957.26 | 24991.44 |
| 9 | Employee, A&G Cost (Rs Cr) | 1979.53 | 2153.38 | 2342.23 | 2529.65 |
|  | Approved by the Honorable commission |  |  |  |  |
| No | Parameters | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| 1 | No. of consumers (Rs.L/1000) | 4.8 | 5.03 | 5.27 | 5.53 |
| 2 | Dist Transformers (Rs. L/DTr) | 0.64 | 0.67 | 0.7 | 0.73 |
| 3 | Length of HT line (Rs. L/km) | 0.79 | 0.83 | 0.87 | 0.91 |
| 4 | Energy sales (Rs/unit) | 0.19 | 0.2 | 0.21 | 0.22 |
| 5 | No. of consumers | 12276321 | 12611100 | 12957080 | 13314526 |
| 6 | No. of Dist Transformers (Nos) | 77724 | 79674 | 81097 | 82502 |
| 7 | HT Line (Ckt-Km) | 62835 | 65236 | 67058 | 68613 |
| 8 | Energy sales in MU | 20880.7 | 21647.29 | 22969.67 | 23957.26 |
| 9 | Employee, A&G Cost (Rs Cr) | 1979.83 | 2142.56 | 2316.28 | 2490 |
| R&M estimation for SBU-D (Rs Cr)- Claimed by KSEBL in MYT pettition |  |  |  |  |  |
| No | Item | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| 1 | Opening GFA | 8390 | 11010.25 | 12531.99 | 13894.96 |
| 2 | GFA Addition | 2620.25 | 1521.74 | 1362.97 | 1270.24 |
| 3 | R&M expenses | 261.83 | 345.41 | 389.46 | 426.84 |
| Approved by Honorable Commission |  |  |  |  |  |
| No | Item | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| 1 | Opening GFA | 8390 | 11010.25 | 12531.99 | 13894.96 |
| 3 | R&M expenses | 246.25 | 316.39 | 342.46 | 358.95 |
| Total normative O&M expenses claimed by KSEBL |  |  |  |  |  |
| No | Item | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| 1 | Employee & A&G Expenses | 1979.53 | 2153.38 | 2342.23 | 2529.65 |
| 2 | R&M Expenses | 261.83 | 345.41 | 389.46 | 426.84 |
| 3 | Total | 2241.36 | 2498.79 | 2731.69 | 2956.49 |
| Approved by Honorable Commission |  |  |  |  |  |
| No | Item | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| 1 | Employee & A&G Expenses | 1979.83 | 2142.56 | 2316.28 | 2490 |
| 2 | R&M Expenses | 246.25 | 316.39 | 342.46 | 358.95 |
| 3 | Total | 2226.08 | 2458.95 | 2658.74 | 2848.95 |

10.10.2	The actual O&M expenditure upto 30.09.2019, revised parameters  and revised claim of O&M expenses from 2019-20 to 2021-22 based on revised parameters is furnished as follows

Actuals up to 30.09.2019:

The actual Employee expenses and A& G expenses upto September 2019 is Rs 1470.24 Cr. The actual R&M expenses upto September 2019 is Rs 128.53 Cr. Thus the actual O&M expenses upto September 2019 is Rs 1598.77 Cr. The revised forecast of Employee expenses & A&G expenses, R&M expenses and the revised parameters based on the revised estimations as explained earlier is given in table below

| Table 10. 90 O&M Expenses |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Employee Cost and A&G cost-Revised claim |  |  |  |  |  |  |  |  |  |  |
| No |  | Parameters |  | 2018-19 | 2019-20 |  | 2020-21 |  | 2021-22 |  |
| 1 |  | No. of consumers (Rs.L/1000) |  | 4.8 | 5.03 |  | 5.27 |  | 5.53 |  |
| 2 |  | Dist Transformers (Rs. L/DTr) |  | 0.64 | 0.67 |  | 0.7 |  | 0.73 |  |
| 3 |  | Length of HT line (Rs. L/km) |  | 0.79 | 0.83 |  | 0.87 |  | 0.91 |  |
| 4 |  | Energy sales (Rs/unit) |  | 0.19 | 0.2 |  | 0.21 |  | 0.22 |  |
| 5 |  | No. of consumers |  | 12552206 | 12845839 |  | 13146341 |  | 13453873 |  |
| 6 |  | No. of Dist Transformers (Nos) |  | 79876 | 81543 |  | 83912 |  | 85902 |  |
| 7 |  | HT Line (Ckt-Km) |  | 64568 | 66935 |  | 69990 |  | 72731 |  |
| 8 |  | Energy sales in MU |  | 22361.55 | 23022.04 |  | 23668.85 |  | 25117.67 |  |
| 9 |  | Employee, A&G Cost (Rs Cr) |  |  | 2149.69 |  | 2313.57 |  | 2497.17 |  |
| *Energy sales inside the State and Interstate sales |  |  |  |  |  |  |  |  |  |  |
| Revised R &M expenses |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 2019-20 |  | 2020-21 |  | 2021-22 |
| 1 |  | Opening GFA |  |  |  | 9450.31 |  | 11660.39 |  | 13284.06 |
| 2 |  | Less land |  |  |  | 19.91 |  | 24.57 |  | 27.99 |
| 3 |  | R& M @ 3% on opening GFA |  |  |  | 282.91 |  | 349.07 |  | 397.68 |
| 4 |  | R&M at prorata basis on assets added during the year |  |  |  |  |  |  |  |  |
| 5 |  | April |  |  |  | 1.97 |  | 1.45 |  | 1.78 |
| 6 |  | May |  |  |  | 4.01 |  | 2.95 |  | 3.62 |
| 7 |  | June |  |  |  | 2.96 |  | 2.17 |  | 2.67 |
| 8 |  | July |  |  |  | 1.18 |  | 0.87 |  | 1.07 |
| 9 |  | August |  |  |  | 1.71 |  | 1.25 |  | 1.54 |
| 10 |  | September |  |  |  | 1.19 |  | 0.88 |  | 1.08 |
| 11 |  | October |  |  |  | 0.87 |  | 0.64 |  | 0.79 |
| 12 |  | November |  |  |  | 0.75 |  | 0.55 |  | 0.67 |
| 13 |  | December |  |  |  | 0.82 |  | 0.61 |  | 0.74 |
| 14 |  | January |  |  |  | 1.10 |  | 0.81 |  | 1.00 |
| 15 |  | February |  |  |  | 0.98 |  | 0.72 |  | 0.88 |
| 16 |  | March |  |  |  | 0.00 |  | 0.00 |  | 0.00 |
| 17 |  | Sub Total R&M for assets added during the year |  |  |  | 17.55 |  | 12.89 |  | 15.82 |
| 18 |  | Total R&M |  |  |  | 300.46 |  | 361.97 |  | 413.50 |
| Total normative O&M expenses –Revised estimate |  |  |  |  |  |  |  |  |  |  |
| No | Item |  | April-Sept 2019 |  |  | 2019-20 |  | 2020-21 |  | 2021-22 |
|  |  |  | Actuals |  |  | Revised normative expenses |  |  |  |  |
| 1 |  | Employee & A&G Expenses | 1470.24 |  |  | 2149.69 |  | 2313.57 |  | 2497.17 |
| 2 |  | R&M Expenses | 128.53 |  |  | 300.46 |  | 361.97 |  | 413.50 |
| 3 |  | Total O& M expenses | 1598.77 |  |  | 2450.15 |  | 2675.54 |  | 2910.68 |

The details are furnished under D-2 of the Distribution tariff forms

10.11	Pay Revision Expenses

10.11.1	SBU-D is not claiming pay revision expenses along with this midterm review  petition. However, it is requested that pay revision expenses as and when arising in the control period may be allowed over and above the norms in line with Kerala State Electricity Regulatory Commission (Terms and Conditions for Determination of Tariff) Regulations, 2018. Details are furnished in Chapter 11. An estimate of pay revision expenses for the control period is submitted below.

| Table 10. 91 : Estimate of Pay revision expenses (Rs Cr) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Business Unit | Employee cost ratio | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| SBU D | 83.56 | 147.05 | 219.84 | 237.58 | 256.28 |

10.11.2	Actual provision made in accounts for 2018-19 has been Rs. 147.05 Cr. However, SBU-D has not revised the projection amount proposed in the MYT of SBU-D from 2019-20 to 2021-22. The Hon Commission may allow the said expense during the truing process.

10.12	Total O&M Expenses

10.12.1	Thus, the total Operation & Maintenance expenses of SBU-D projected for the control period as per the Regulation is summarised below:

| Table 10. 92 : SBU wise O&M expenses |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | April-Sept 2019 | 2019-20 | 2020-21 | 2021-22 |
| Normative O&M cost |  | 2450.15 | 2675.54 | 2910.68 |

10.12.2	The projection of the O&M expenses of SBU-D for the control period based on the past actuals is shown in the table below.

|  | Table 10. 93: Projection based on past actual (Rs Cr) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item |  | April-Sept 2019 | 2019-20 | 2020-21 | 2021-22 |
| O&M Cost of SBU-D |  | 1586.92 | 2744.48 | 2952.65 | 3174.64 |

10.12.3	It may kindly be noted that, the O&M expenses as per norms for the control period are below the actual expense projection. This will lead to under recovery of genuine O&M costs. However, KSEBL is taking earnest efforts to control the O&M expenses to the extent possible. If any variation is found beyond its control then it is humbly requested that the same may be allowed at actual during true up.

Interest and Finance Charges

10.13 Interest on Capital liabilities:

10.13.1	The Interest & Finance charges actually incurred by SBU-D for the year 2019-20 up to September 2019 have been Rs. 156.04 Cr. The Interest & Finance charges approved for the year 2019-20 by Hon’ble Commission vide MYT order dated 8-7-2019, actual incurred up to September 2019 and revised Interest & Finance charges for the years 2019-20, 2020-21 and 2021-22for SBU-D are submitted below. The revised Interest & Finance charges are arrived based on the opening loan, GFA addition during the year and normative repayment for the year, which is detailed under Chapter-12 of this petition.

| Table 10.94: Normative loan and interest for the remaining years (Rs Cr) |  |  |  |
| --- | --- | --- | --- |
|  | SBU-D |  |  |
|  | 2019-20 | 2020-21 | 2021-22 |
| Opening Normative loan | 2119.29 | 3277.83 | 4375.42 |
| GFA addition net of capital grants & subsidies-Addl normative loan | 1,310.11 | 1,299.99 | 1,719.86 |
| Normative repayment | 151.57 | 202.40 | 263.89 |
| Closing normative loan | 3277.83 | 4375.42 | 5831.39 |
| Av normative loan | 2698.56 | 3826.63 | 5103.40 |
| Average actual interest rate in 2018-19 | 10 | 10 | 10 |
| Normative interest for the year | 269.86 | 382.66 | 510.34 |

Note: Closing normative loan as per TU 2019 has been considered as opening normative loan for 2019-20. Actual rate of interest for 2018-19 has been considered for the remaining years of control period.

Interest on Security Deposit

10.13.2	The interest on security deposit claimed in the MYT petition and approved by the Honorable Commission and revised projection for the remaining years are detailed in Chapter 11. A summary of the revised claim is furnished below:

| Table 10. 95: Interest on Security Deposit (Rs Cr)- Revised claim |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Particulars | FY 19 | FY-20 | FY-21 | FY-22 |
| 1 | Opening: SD as on 1st April | 2836.96 | 3019.12 | 3319.12 | 3619.12 |
| 2 | Addition during the FY | 182.16 | 300.00 | 300.00 | 300.00 |
| 3 | Closing SD as on 31st March | 3019.12 | 3319.12 | 3619.12 | 3919.12 |
| 4 | Average SD during the FY | 2928.04 | 3169.12 | 3469.12 | 3769.12 |
| 5 | Average interest rate (%) | 6.25 | 6.25 | 6.25 | 6.25 |
| 6 | Interest Charges | 177.31 | 188.70 | 207.45 | 226.20 |
| 7 | Claim based on actual disbursement |  | 177.31 | 188.70 | 207.45 |

Interest on GPF

10.13.3	Details with regard to interest on GPF claimed in MYT petition, approval granted by the Hon’ble Commission and revised estimate for the remaining years of the control period are detailed in Chapter 11. A summary of the same is furnished in the Table Below:

| Table 10. 96 Interest on provident fund of SBU-D (Rs Cr) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No | Item | Claimed in ARR for 2019-20 | Approved for FY 2019-20 | Actual upto 30-9-19 | Revised estimate for 19-20 | Revised estimate for 20-21 | Revised estimate for 21-22 |
| 1 | Interest on Provident fund | 160.19 | 152.55 | 72.79 | 152.70 | 162.73 | 172.76 |

Interest on Master Trust bonds

10.13.4	SBU wise interest on Master Trust Bond and interest is calculated in Chapter 11. The interest for SBUD is reproduced in the Table below. This includes the interest contribution of existing bonds as well as additional contribution approved by the Hon’ble Commission.

| Table 10. 97 interest on Master Trust bonds (Rs Cr) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No | Item | Estimated in ARR | Approved vide order dtd.8-7-19 | Actual upto 30-9-19 | Revised estimate for 19-20 | Revised estimate for 20-21 | Revised estimate for 21-22 |
| 1 | Existing bond | 646.45 | 646.45 | 313.78 | 612.43 | 578.40 | 544.38 |
| 2 | Additional contribution to Master Trust | 311.58 | 167.10 |  | 167.10 | 167.10 | 167.10 |
|  | Total | 958.03 | 813.55 | 313.78 | 779.53 | 745.50 | 711.48 |

Interest on Working Capital

10.13.5	The Normative Working Capital for SBU-D, as per Regulation 32, includes O&M Cost for one month, cost of maintenance spares as one twelfth of average of the book value of stores for that FY, receivables for two months excluding security deposits.

10.13.6	The Interest on Working Capital in this petition is computed at a rate of 10.95% (8.95% as on 1-4-2019 + 2%) as per Regulation 32(2). The parameters adopted for computation of Interest on Working capital for the control period are furnished below.

| Table 10. 98: Parameters for Working Capital (Rs Cr) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Proposed by KSEBL in MYT Petition& Approved by the Commission |  |  |  |  |
| Item | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| Opening GFA | 8390 | 11010.25 | 12531.99 | 13894.96 |
| Inventory | 485.9 | 614.98 | 710.81 | 801.76 |
| O&M Cost | 2241.36 | 2487.63 | 2725.56 | 2958.09 |
| Expected Revenue | 12433.68 | 13143.48 | 13758.18 | 14402.37 |
| Revised parameters |  |  |  |  |
| Opening GFA | 8390 | 11010.25 | 12531.99 | 13894.96 |
| Inventory | 485.9 | 614.98 | 710.81 | 801.76 |
| O&M Cost | 2241.36 | 2487.63 | 2725.56 | 2958.09 |
| Expected Revenue | 12433.68 | 13143.48 | 13758.18 | 14402.37 |

10.13.7	The Interest on Working Capital projected and approved for the control period 2018-19 to 2021-22 for SBU-D is submitted below:

| Table 10. 99: Interest on Working capital (Rs.Cr.)- Proposed by KSEBL in MYT Petition |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Item | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| 1 | O&M expenses | 186.78 | 207.30 | 227.13 | 246.51 |
| 2 | Cost of Mace spares | 4.86 | 6.15 | 7.11 | 8.02 |
| 3 | Receivables for two months | 2072.28 | 2190.58 | 2293.03 | 2400.395 |
| 4 | Total Working Capital | 2263.92 | 2404.03 | 2527.27 | 2654.92 |
| 4 | Less: Security Deposits | 2836.96 | 3086.96 | 3386.96 | 3686.96 |
| 5 | Net Working Capital | -573.04 | -682.93 | -859.69 | -1032.04 |
| Approved by the Commission |  |  |  |  |  |
| No | Item | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| 1 | O&M expenses | 185.51 | 204.91 | 221.56 | 237.41 |
| 2 | Cost of Mace spares | 4.86 | 6.15 | 7.11 | 8.02 |
| 3 | Receivables for two months | 2074.97 | 2209.11 | 2312.36 | 2420.56 |
| 4 | Total Working Capital | 2265.34 | 2420.17 | 2541.03 | 2665.99 |
| 4 | Less: Security Deposits | 2836.96 | 3086.96 | 3386.96 | 3686.96 |
| 5 | Net Working Capital | -571.62 | -666.79 | -845.93 | -1020.97 |

10.13.8	Since the working Capital requirement is less than the security deposit held at the beginning of the year, no interest is claimed for working capital for the remaining years of the control period as well.

Carrying cost on approved as well as projected revenue gap .

  - 10.13.9	Carrying cost on revenue gap sought in MYT petion, approval and revised estimate based on the approved gap till 2016-17 and revenue gap sought for true up till 2018-19 are detailed in Chapter 11. Hon’ble Commission may kindly note that in addition to the approved revenue gap till FY 17 of Rs.6739.13 Cr, the additional revenue deficit sought for True up for 2016-17 and true up sought for 2017-18 and 2018-19 amounted to Rs.63.76 Cr, Rs.1331.81 Cr and Rs.759.88 Cr respectively. In addition, KSEBL has claimed carrying cost for the revenue gap estimated for the remaining years also. Hon’ble Commission may kindly approve towards carrying cost for the remaining years under SBU D detailed in Chapter 11 as extracted below:

| Table 10. 100 Carrying cost (Rs Cr) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Item | 2018-19 TU | 2019-20 | 2020-21 | 2021-22 |
| 1 | As per MYT petition | 281.17 | 200.32 | 104.68 | 9.03 |
| 2 | Approved as per order | 304.08 | 223.59 | 121.78 | 27.08 |
| 3 | Revised estimate | 629.11 | 688.90 | 737.61 | 814.47 |

10.13.10	Summary of interest and finance charges estimated for SBU-D, approved by the Hon’ble Commission and the revised estimate for the remaining years of the control period is furnished below:

| Table 10. 101: Summary of Interest & Finance Charges (Rs Cr ) for SBU-D- Proposed by KSEBL in MYT petition |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Item | FY-19 | FY-20 | FY-21 | FY-22 |
| 1 | Interest on capital liabilities | 304.86 | 486.34 | 594.91 | 688.82 |
| 2 | Interest on security Deposit | 199.93 | 218.49 | 238.74 | 258.99 |
| 3 | Interest on GPF | 160.19 | 170.72 | 181.24 | 191.77 |
| 4 | Interest on MT Bonds | 646.45 | 612.43 | 578.40 | 544.38 |
| 5 | Carrying cost | 281.17 | 200.32 | 104.68 | 9.03 |
| 6 | Additional interest to Master Trust | 311.58 | 311.58 | 311.58 | 311.58 |
| 7 | Total Interest & Finance Charges | 1934.59 | 2799.73 | 3148.71 | 3433.69 |
| Approved by the Commission |  |  |  |  |  |
| 1 | Interest on capital liabilities | 149.90 | 222.94 | 281.40 | 329.80 |
| 2 | Interest on GPF | 152.55 | 162.58 | 172.60 | 182.63 |
| 3 | Interest on Master Trust | 646.45 | 612.43 | 578.40 | 544.38 |
| 4 | Carrying cost | 304.08 | 223.59 | 121.78 | 27.08 |
| 5 | Interest on security Deposit | 175.33 | 199.93 | 218.49 | 238.74 |
| 6 | Additional interest to Master Trust | 167.10 | 167.10 | 167.10 | 167.10 |
| 7 | Total Interest & Finance Charges | 1595.41 | 1421.47 | 1372.67 | 1322.63 |
| Revised estimate |  |  |  |  |  |
| 1 | Interest on capital liabilities | 180.27 | 269.86 | 382.66 | 510.34 |
| 2 | Interest on security Deposit | 170.75 | 177.31 | 188.70 | 207.45 |
| 3 | Interest on GPF | 136.68 | 152.70 | 162.73 | 172.76 |
| 4 | Interest on MT Bonds | 652.08 | 612.43 | 578.40 | 544.38 |
| 6 | Carrying cost | 629.11 | 688.90 | 737.61 | 814.47 |
| 7 | Other interest | 18.41 |  |  |  |
| 8 | Additional interest to Master Trust | 167.10 | 167.10 | 167.10 | 167.10 |
| 9 | Total Interest & Finance Charges | 1954.40 | 2068.30 | 2217.20 | 2416.50 |

Note: RE for 2018-19 represent amount sought for truing up as per TU 19.

10.14 Depreciation

10.14.1	The depreciation in line with Tariff Regulation, 2018 for the control period as per MYT petition and approved by the Hon’ble Commission is detailed below:

| Table 10. 102 Depreciation for control period (Rs Cr) |  |  |
| --- | --- | --- |
| Year | As per MYT petition | As per MYT order |
| 2018-19 | 109.72 | 72.24. |
| 2019-20 | 143.99 | 93.47 |
| 2020-21 | 163.89 | 121.53 |
| 2021-22 | 181.71 | 154.91 |

10.14.2	The methodology for calculation of depreciation is detailed under Chapter-12.  The Depreciation approved for the year 2019-20 by Hon’ble Commission vide MYT order dated 8-7-2019, actual incurred up to September 2019 and revised Depreciation for the years 2019-20,2020-21 and 2021-22 are submitted below.

| Table 10. 103:Depreciation for SBU-D (Rs.Cr.) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item | Estimated in ARR for 2019-20 | Approved vide order dtd. 8-7-2019 | Actual upto Sept-2019 | Revised estimate for FY-20 | Revised estimate for FY-21 | Revised estimate for FY-22 |
| Depreciation | 143.99 | 93.47 | 223.11 | 151.57 | 202.40 | 263.89 |

10.15 Return on Equity

10.15.1	The ROE for SBU-D claimed, approved revised estimate for the future years of the control period is submitted below.

| Table 10. 104 ROE OF SBU-D Return on Equity (Rs.Cr.) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Claimed in ARR for the all years | Approved vide order dtd.8-7-2019 | Actual upto 30-9-2019 | Revised estimate for 2019-20 | Revised estimate for 2020-21 | Revised estimate for 2021-22 |
| No | Item |  |  |  |  |  |  |
| 1 | Return on Equity | 253.50 | 253.50 |  | 253.50 | 253.50 | 253.50 |

10.16	Bad debts to be written off

10.16.1	A provision of Rs 15 Cr is estimated for the control period. Honorable Commission disallowed the same by stating that the same will be examined at the time of truing up. Hence KSEBL is not claiming the same in this petition. Honble Commission may kindly allow the same at the time of truing up.

10.17	Aggregate Revenue Requirement

10.17.1	The SBU-D handles the distribution assets of KSEB Ltd and manages retail supply of power within the State for supply to 99% of total consumers in the State.  The Aggregate Revenue Requirement of SBU-D for the control period claimed by KSEBL and approved by Honorable Commission is given in table below:

| Table 10. 105ARR of SBUD (Rs Cr.) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Claimed in MYT Petition |  |  |  |  |  |
| No | Particulars | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| 1 | Cost of Generation | 522.94 | 560.87 | 657.72 | 792.44 |
| 2 | Cost of Power Purchase | 7942.01 | 8672.27 | 8884.51 | 9184.58 |
| 3 | Cost of Intra State Transmission | 951.47 | 1106.58 | 1394.65 | 1637.52 |
| 4 | NLDC /RLDC charges | 3 | 3 | 3 | 3 |
| 5 | O&M Expenses | 2241.36 | 2499 | 2731.69 | 2956.49 |
| 6 | I&F charges on long-term loans | 304.86 | 486.34 | 594.91 | 688.82 |
| 7 | Interest on Master Trust Bonds | 646.45 | 612.43 | 578.4 | 544.38 |
| 8 | Interest on additional bonds | 311.58 | 311.58 | 311.58 | 311.58 |
| 9 | Interest on Provident Fund | 160.19 | 170.72 | 181.24 | 191.77 |
| 10 | Interest on Security Deposit | 199.93 | 218.49 | 238.74 | 258.99 |
| 11 | Interest on WC and deposits | 0 | 0 | 0 | 0 |
| 12 | Carrying Cost on Gap | 281.17 | 200.32 | 104.68 | 9.03 |
| 13 | Depreciation | 109.72 | 143.99 | 163.89 | 181.71 |
| 14 | One Time expenses | 29.24 |  |  |  |
| 15 | Bad debts written off | 15 | 15 | 15 | 15 |
| 16 | Recovery of previous gap | 806.47 | 806.47 | 806.47 | 806.47 |
| 17 | Total Revenue Expenditure | 14525.39 | 15807.05 | 16666.49 | 17581.79 |
| 18 | Return on Equity | 253.5 | 253.5 | 253.5 | 253.5 |
| 19 | Aggregate Revenue Requirement | 14778.89 | 16060.35 | 16919.99 | 17835.29 |
| Approved by Honorable Commission |  |  |  |  |  |
| 1 | Cost of Generation | 481.41 | 505.93 | 586.34 | 639.86 |
| 2 | Cost of Power Purchase | 7,847.77 | 8,613.94 | 8,797.94 | 8,964.44 |
| 3 | Cost of Intra State Transmission | 874.6 | 983.69 | 1,162.47 | 1,283.53 |
| 4 | NLDC /RLDC charges | 3 | 3 | 3 | 3 |
| 5 | O&M Expenses | 2,226.08 | 2,458.95 | 2,658.75 | 2,848.95 |
| 6 | Interest on long-term loans | 149.9 | 222.94 | 281.4 | 329.8 |
| 7 | Interest on Master Trust Bonds | 646.45 | 612.43 | 578.4 | 544.38 |
| 8 | GPF Interest | 152.55 | 162.58 | 172.6 | 182.63 |
| 9 | Interest on WC | - | - | - | - |
| 10 | Charges for proposed addn to Master Trust | 167.1 | 167.1 | 167.1 | 167.1 |
| 11 | Interest on Security Deposit | 175.33 | 199.93 | 218.49 | 238.74 |
| 12 | Carrying Cost on Gap | 304.08 | 223.59 | 121.78 | 27.08 |
| 13 | Depreciation | 72.24 | 93.47 | 121.53 | 154.91 |
| 14 | One Time Expenses CMDRF | - | - | - | - |
| 15 | Bad debts written off | - | - | - | - |
| 16 | Recovery of previous gap | 400 | 1,000.00 | 850 | 850 |
| 17 | Total Revenue Expenditure | 13,500.52 | 15,247.53 | 15,719.81 | 16,234.41 |
| 18 | Return on Equity | 253.5 | 253.5 | 253.5 | 253.5 |
| 19 | Aggregate Revenue Requirement | 13,754.02 | 15,501.03 | 15,973.31 | 16,487.91 |

10.17.2	The revised ARR based on revised expenses for the remaining years of second control period is given in table below:

| Table 10. 106 Aggregate Revenue Requirement of SBU-D (Rs Cr.)-Revised Claim |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No |  | Particulars |  |  |  |  | 2018-19 |  |  | April to Sept-19 |  |  | FY-20 |  |  | FY-21 |  |  | FY-22 |  |  |
| 1 |  | Cost of Generation |  |  | 506.79 |  |  | 322.37 |  |  | 482.31 |  |  | 533.36 |  |  | 676.09 |  |  |
| 2 |  | Cost of Power Purchase |  |  | 7826.68 |  |  | 4386.83 |  |  | 8603.15 |  |  | 9434.65 |  |  | 9813.79 |  |  |
| 3 |  | Cost of Intra State Transmission |  |  | 858.64 |  |  | 536.03 |  |  | 953.84 |  |  | 1190.45 |  |  | 1444.24 |  |  |
| 4 |  | O&M Expenses |  |  | 2379.04 |  |  | 1586.92 |  |  | 2450.15 |  |  | 2675.54 |  |  | 2910.68 |  |  |
| 5 | Interest on capital liabilities | 180.27 | 76.00 | 269.86 | 382.66 | 510.34 |
| 6 | Interest on Master Trust Bonds | 652.08 | 313.78 | 612.43 | 578.4 | 544.38 |
| 7 | Interest on Provident Fund | 136.68 | 72.89 | 152.70 | 162.73 | 172.76 |
| 8 | Interest on Security Deposit | 170.75 | 172.79 | 177.31 | 188.7 | 207.45 |
| 9 | Interest on Working Capital |  | 82.81 | 0 | 0 | 0 |
| 10 | Carrying Cost on revenue gap | 629.11 |  | 688.90 | 737.61 | 814.47 |
| 11 | Other interest | 18.41 | 0.04 |  |  |  |
| 12 | Depreciation | 107.98 | 223.11 | 151.57 | 202.40 | 263.89 |
| 13 | One Time expenses | 29.25 | 11.84 |  |  |  |
| 14 |  | Other exp |  |  | 291.38 |  |  | 10.21 |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  | T&D achievement claim |  |  | 28.59 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  | Additional Interest on Master trust |  |  | 167.1 |  |  |  |  |  | 167.10 |  |  | 167.10 |  |  | 167.10 |  |  |
| 17 |  | Total Revenue Expenditure |  |  |  |  | 13982.75 |  |  | 7795.62 |  |  | 14709.32 |  |  | 16253.60 |  |  | 17525.18 |  |  |
| 18 |  | Return on Equity |  |  | 253.5 |  |  | 89.43 |  |  | 253.50 |  |  | 253.50 |  |  | 253.50 |  |  |
| 19 |  | Aggregate Revenue Requirement |  |  |  |  | 14236.25 |  |  | 7885.04 |  |  | 14962.82 |  |  | 16507.10 |  |  | 17778.68 |  |  |

10.18	Other Income

10.18.1	The other income of SBU-D includes income from sale of scrap, interest on advances made to contractors, interest on staff loans and advances, Rent from buildings etc. KSEBLtd is claiming the the other income claimed in MYT petition for future years of control period. The other income of SBU-D for the remaining period of control period is summarized below:

| Table 10. 107 : Other Income |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Other Income (Rs. Cr.) | FY-20 | FY-21 | FY-22 |
| 1 | Interest on staff loans etc | 0.10 | 0.09 | 0.07 |
| 2 | Income from statutory investments |  |  |  |
| 3 | Income from rent of land etc | 4.06 | 4.59 | 5.12 |
| 4 | Income from sale of scrap | 22.00 | 24.75 | 27.51 |
| 5 | Income from staff welfare activities |  |  |  |
| 6 | Rental from staff quarters | 0.25 | 0.28 | 0.31 |
| 7 | Excess found on physical verification | 0.02 | 0.02 | 0.03 |
| 8 | Interest on investments, fixed and call deposits and bank balances | 2.78 | 2.78 | 2.78 |
| 9 | Interest on advances to suppliers/contractors | 14.52 | 15.38 | 16.23 |
| 10 | Income from hire charges from contractors and others | 0.00 | 0.00 | 0.00 |
| 11 | Income due to right of way granted for paying fibre optic cables/co-axial cables | 43.05 | 48.65 | 54.24 |
| 12 | Miscellaneous receipts | 118.66 | 126.33 | 134.03 |
| 13 | Commission for collection of electricity duty | 9.54 | 10.78 | 12.02 |
| 14 | Interest on delayed or deferred payment on bills | 20.01 | 20.35 | 20.69 |
| 15 | Rebate from NTPC & PFC | 137.00 | 139.00 | 141.00 |
|  | Total Other Income (Rs.Cr.) | 371.99 | 393.00 | 414.03 |

10.19	Non Tariff Income

10.19.1	The non tariff income of regulated business includes income other than those obtained from tariff such as income from wheeling, receipts on account of cross-subsidy surcharge and additional surcharge on charges of wheeling ,reactive energy charges ,meter rent, rental from electric plants or lines, testing fee, late payment surcharge, prompt payment incentives, recovery from theft and pilferage of energy or such other charges.KSEBL is claiming the same nontariff income as claimed in MYT petition for future years of second controlperiod

| Table 10. 108 : Non-Tariff Income(Rs Cr) |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Other Income | 2019-20 | 2020-21 | 2021-22 |
| 1 | Income from wheeling | 0.10 | 0.10 | 0.10 |
| 2 | Reactive energy charges | 50.00 | 50.00 | 50.00 |
| 3 | Receipts : cross-subsidy surcharge |  |  |  |
| 4 | Receipts on additional surcharge |  |  |  |
| 5 | Meter Rent | 96.37 | 97.97 | 99.57 |
| 6 | Rental from Electrical plants etc |  |  |  |
| 7 | Testing Fee |  |  |  |
| 8 | Late payment from surcharge |  |  |  |
| 9 | Prompt payment incentives |  |  |  |
| 10 | Recovery on Theft and Pilferage | 6.36 | 6.47 | 6.58 |
| 11 | Power factor Penalty | 23.28 | 23.69 | 24.08 |
|  | Total Non Tariff Income (Rs.Cr.) | 176.11 | 178.23 | 180.33 |

10.20	Net ARR of SBU-D

10.20.1	The net ARR of SBU-D as per MYT petition and approval for the control period is submitted below:

| Table 10. 109Aggregate Revenue Requirement of SBU-D (Rs Cr.) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Claimed in MYT petition |  |  |  |  |  |
| No | Particulars | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| 1 | Aggregate Revenue Requirement | 13960.51 | 15160.62 | 15916.69 | 16695.22 |
| 2 | Less Other Income | 357.8 | 371.99 | 393 | 414.03 |
| 3 | Less Non Tariff Income | 173.75 | 176.11 | 178.23 | 180.33 |
| 4 | Net ARR | 13428.96 | 14612.52 | 15345.46 | 16100.86 |
| Approved by the Honorable Commission |  |  |  |  |  |
| 1 | Aggregate Revenue Requirement | 13754.02 | 15501.03 | 15973.31 | 16487.91 |
| 2 | Less Other Income/Non Tariff Income | 531.55 | 548.10 | 571.23 | 594.36 |
| 3 | Net ARR | 13222.47 | 14952.93 | 15402.08 | 15893.55 |

10.20.2	The revised claim of net ARR based on revised expenses is given in table below

| Table 10. 110 Net ARR of SBU-D (Rs Cr.)-Revised projection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Particulars | Apr-Sept-19 | FY-20 | FY-21 | FY-22 |
| 1 | Aggregate Revenue Requirement (Table 10.106) | 7885.04 | 14962.82 | 16507.10 | 17778.68 |
| 2 | Less Other Income (Table 10.107) | 174.06 | 371.99 | 393 | 414.03 |
| 3 | Less Non Tariff Income (Table 10.108) | 100.98 | 176.11 | 178.23 | 180.33 |
| 4 | Net ARR (1-2-3) | 7610.00 | 14414.72 | 15935.87 | 17184.32 |

10.21	Wheeling charges

10.21.1	SBU-D handles the entire distribution network of Kerala. The revised projection of the energy input to the Distribution network, energy handled, losses and the wheeling charges estimated for the control period are submitted below:

| Table 10. 111 : Wheeling charges for control period |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Particulars | FY-20 | FY-21 | FY-22 |
| 1 | Energy injected into the system (MU) | 26135.55 | 26673.99 | 27629.24 |
| 2 | Percentage of transmission loss (%) | 3.65 | 3.55 | 3.45 |
| 3 | Loss of energy (MU) | 953.95 | 946.93 | 953.21 |
| 4 | Energy handled by Distribution(MU) (1-3) | 25181.60 | 25727.06 | 26676.03 |
| 5 | Sale of energy at 66 KV and above(MU) | 2091.84 | 2156.49 | 2225.90 |
| 6 | Less auxiliary consumption of substation(MU) | 16.92 | 17.76 | 18.65 |
| 7 | Energy transmitted to HT system(MU) | 23072.85 | 23552.82 | 24431.48 |
| 8 | HT loss(MU) | 1211.32 | 1212.97 | 1233.79 |
| 9 | Energy handled at HT level (MU) (1-3) | 21861.52 | 22339.85 | 23197.69 |
| 10 | Distribution ARR excl Gen PP trn (Rs Cr) | 4375.42 | 4777.41 | 5250.20 |
| 11 | Distribution ARR(at HT level)(25 % )(Rs Cr.) | 1093.86 | 1194.35 | 1312.55 |
| 12 | Wheeling Charges(Rs/unit) | 0.50 | 0.53 | 0.57 |

  - 10.22.	Average pooled Power Purchase cost for FY-21 and FY 22

  - 10.22.1Based on the approved quantum and cost in ARR of 2018-19 vide MYT order 08.07.2019 , KSEBL is considering the  average pooled purchase cost for FY-20 as Rs 2.85 per unit for settling the net banked energy. The average pooled cost of power purchase for FY-21 and FY-22 based on revised estimate is given below

10.22.2	Honorable CERC vide Regulation 5(1) (c) of Central Electricity Regulatory Commission (Terms and Conditions for recognition and issuance of Renewable Energy Certificate for Renewable Energy Generation) Regulations, 2010 defines Average Power Purchase Cost (APPC) as follows:

“for the purpose of these regulations „Pooled Cost of Purchase‟ means the weighted average pooled price at which the distribution licensee has purchased the electricity including cost of self generation, if any, in the previous year from all the energy suppliers long-term and short-term,but excluding those based on renewable energy sources, as the case may be.”

10.22.3As per said regulation, the average pooled purchase cost of KSEBL for the year 2020-21 and 2021-22 are tabulated in the tables given below

| Table 10.112(a) Parameters for determination of average pooled cost of power purchase |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Particulars | 2019-20 |  | 2020-21 |  | 2021-22 |  |
|  | Quantity (MU) | Cost (Rs.Cr.) | Quantity (MU) | Cost (Rs.Cr.) | Quantity (MU) | Cost (Rs.Cr.) |
| Own generation less RE | 6079.86 | 452.16 | 6122.07 | 477.01 | 6179.86 | 590.80 |
| CGS (KSEB Ltd.periphery) | 10517.24 | 4041.51 | 11326.55 | 4526.22 | 11326.55 | 4667.17 |
| Traders (Long Term & Medium Term) | 8111.05 | 3492.2 | 8648.62 | 3737.65 | 8648.62 | 3769.23 |
| short term | 1287.65 | 379.81 | 384.67 | 154.01 | 314.61 | 170.64 |
| RGCCPP | 0 | 62.35 | 0 | 62.35 | 0 | 62.35 |
| Less surplus sale excl from renewable energy* | 54.76 | 17.07 | 1522.168 | 608.87 | 1847.35 | 738.94 |
| Total | 25941.04 | 8410.96 | 24959.74 | 8348.37 | 24622.29 | 8521.25 |
| *apportioned based on energy from conventional and renewable sources |  |  |  |  |  |  |

| Table 10.112(b) Average Pooled Cost of Power Purchase (Rs /unit) |  |  |
| --- | --- | --- |
| Particulars | 2020-21 | 2021-22 |
| Total Generation and Power Purchase except from Renewable Sources of previous year(MU) | 25941.04 | 24,959.74 |
| Total Cost of Generation and Power Purchase excluding cost from Renewable Sources& Inter/Intrastate transmission charges of previous year | 8410.964 | 8348.37 |
| Average Pooled Cost of Power Purchase (Rs /unit) | 3.24 | 3.34 |

10.22.3	The total cost of power purchase considered for computation of APPC excludes cost of generation and procurement from renewable energy sources and transmission charges. Exclusion of cost of generation and procurement from renewable sources is in alignment with the objective of determining APPC, which is to ascertain the average cost of power from conventional sources of electricity. The cost of generation from conventional sources is arrived by apportioning generation ARR on the basis of MW. Interstate as well as intrastate transmission charges have been excluded from these calculations.

  - 10.22.4	As seen in the table above KSEBL estimates the average pooled purchase cost of KSEBL for the year 2020-21 and 2021-22 as Rs 3.24 per unit and Rs 3.34 per unit respectively. Honorable Commission may kindly approve the same.

###### 10.23	Expected Revenue from Charges

10.23.1	The revenue from sale of power is estimated on the basis of following.

- Actual revenue for the year 2018-19.

- Actual revenue  is taken for the first half of FY 2019-20 and revenue for the second half of FY 2020 is estimated based on the prevailing Schedule of Tariff and terms and conditions for retail supply of electricity dated 08.07.2019.

- The revenue for FY 2021and 2022is estimated based on the slabwise consumption of FY 19 and the prevailing Schedule of Tariff and terms and conditions for retail supply of electricity dated 08.07.2019.

10.23.2	Honorable Commission vide order dated 08.07.2019 approved the revenue from tariff for the control period as given in table below.

| Table 10.113 Revenue for the control period |  |  |  |  |
| --- | --- | --- | --- | --- |
| As per Petition |  |  |  |  |
| Item | 2018-19 | 2019-20 | 2010-21 | 2021-22 |
|  | Rs.Crore | Rs.Crore | Rs.Crore | Rs.Crore |
| Net Revenue from Sale of power | 12,352.03 | 13,150.46 | 13,765.29 | 14,409.63 |
| Fuel Surcharge | 81.65 |  |  |  |
| Revenue from external sales | 712.96 | 962.74 | 518.19 | 312.38 |
| Total revenue | 13,146.64 | 14,113.20 | 14,283.48 | 14,722.01 |
| Approved by Honorable Commission |  |  |  |  |
| Item | 2018-19 | 2019-20 | 2010-21 | 2021-22 |
|  | Rs.Crore | Rs.Crore | Rs.Crore | Rs.Crore |
| Net Revenue from Sale of power | 12,352.03 | 13,209.68 | 13,841.64 | 14,489.41 |
| Fuel Surcharge | 81.65 |  |  |  |
| Revenue from external sales | 756.64 | 942.7 | 615.68 | 405.61 |
| Total revenue | 13,190.32 | 14,152.38 | 14,457.33 | 14,895.02 |

Revenue from tariff for the year 2018-19

10.23.3	The summary of the KSEBL’s revenue from sale as per provisional accounts for the year 2018-19 is given in the table below:

| Table 10. 114 : Revenue from sale of power for the year 2018-19 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Category | Consumer Strength (31-3-19) |  | Energy sale (18-19) |  | Revenue (18-19) |  |
|  | (nos) | % | (MU) | % | (Rs. Cr) | % Total |
| Domestic | 9705347 | 77.32 | 10864.34 | 50.45 | 4621.04 | 36.35 |
| Commercial | 2196919 | 17.50 | 3221 | 14.96 | 3062.80 | 24.09 |
| Industrial | 138834.00 | 1.11 | 1111.67 | 5.16 | 792.76 | 6.24 |
| Agriculture | 470517 | 3.75 | 338 | 1.57 | 86.78 | 0.68 |
| Public Lighting | 34653 | 0.28 | 378 | 1.75 | 172.77 | 1.36 |
| HT & EHT | 5913 | 0.05 | 4724 | 21.93 | 3422.07 | 26.92 |
| Railway Traction | 12 | 0.00 | 303 | 1.41 | 186.38 | 1.47 |
| Bulk Supply | 11 | 0.00 | 597.04 | 2.92 | 369.13 | 2.90 |
| Total | 12552206 | 100 | 21536.77 | 100 | 12713.72 | 100 |
| Misc |  |  |  |  | 0.20 |  |
| Sale outside State |  |  | 824.78 |  | 339.34 |  |
| Total | 12552206 |  | 22361.55 |  | 13053.26 |  |

10.23.4	The revenue from tariff for the year 2018-19 amounts to Rs.  12713.72 Cr (Rs.12599.96 Cr after adjusting PFI Rs.114.06 Cr) corresponding to the sale of 21536.77 MU. 824.78 MU was sold outside the State. The total revenue from sale of power including interstate sales amounts to Rs13053.26Cr (Rs.12939.20 Cr net of PFI).

Estimation of revenue from sale of power for the control period

10.23.5	The revised estimate of the revenue based on revised energy sales and revised Tariff and considering the   slabwise consumption and consumerwise details. The slab wise data is used for arriving at the slab wise consumption details of consumers and by applying corresponding energy charges and fixed charges revenue is estimated.

10.23.6	The revised revenue estimate of the next control period is given in table below:

| Table 10. 115 Revised Estimate of revenue projection for next control period |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Category | April-Sept 19(Provisional) |  | 2019-20(Estimate) |  | 2020-21(Estimate) |  | 2021-22(Estimate) |  |
|  |  | MU | Rs Cr | MU | Rs Cr | MU | Rs Cr | MU | Rs Cr |
| 1 | LT I Domestic | 5988.15 | 2674.47 | 11584.02 | 5272.77 | 11680.62 | 5437.09 | 12087.61 | 5635.06 |
| 2 | LT II Colony | 2.00 | 1.32 | 4.11 | 2.79 | 4.23 | 2.94 | 4.27 | 2.96 |
| 3 | LT IV A Industrial | 551.99 | 394.72 | 1120.40 | 811.12 | 1123.28 | 826.99 | 1134.51 | 835.67 |
| 4 | LT IV B Industrial | 5.28 | 3.84 | 10.71 | 7.86 | 10.74 | 7.99 | 10.84 | 8.04 |
| 5 | LT V A Agriculture | 125.47 | 32.32 | 299.31 | 77.96 | 300.51 | 80.79 | 311.78 | 83.69 |
| 6 | LT V B Agriculture | 28.73 | 8.07 | 65.15 | 18.67 | 62.94 | 18.46 | 65.30 | 19.14 |
| 7 | LT VI A General | 108.37 | 78.22 | 216.32 | 157.83 | 222.66 | 164.80 | 235.13 | 173.94 |
| 8 | LT VI B General | 62.82 | 48.94 | 126.06 | 99.60 | 129.39 | 104.12 | 136.64 | 109.90 |
| 9 | LT VI C General | 125.04 | 129.06 | 248.91 | 260.98 | 259.87 | 280.40 | 274.44 | 296.09 |
| 10 | LT VI D General | 11.40 | 3.28 | 22.69 | 6.93 | 23.31 | 6.24 | 24.61 | 6.58 |
| 11 | LT VI E General | 1.52 | 0.87 | 3.03 | 1.82 | 3.12 | 1.76 | 3.30 | 1.84 |
| 12 | LT VI F General | 477.46 | 454.04 | 945.50 | 912.15 | 979.66 | 958.52 | 1034.55 | 1013.05 |
| 13 | LT VI G General | 39.28 | 38.29 | 77.96 | 75.55 | 81.05 | 78.44 | 85.59 | 82.85 |
| 14 | LT VII A Commercial | 816.99 | 794.01 | 1620.82 | 1607.85 | 1678.50 | 1707.80 | 1772.55 | 1802.37 |
| 15 | LT VII B Commercial | 91.23 | 63.55 | 181.42 | 127.91 | 188.33 | 134.58 | 198.88 | 141.82 |
| 16 | LT VII C Commercial | 12.11 | 9.68 | 23.79 | 19.37 | 24.39 | 20.14 | 25.76 | 21.26 |
| 17 | LT VIII A Pub Lighting | 135.88 | 64.76 | 281.84 | 135.99 | 289.53 | 141.29 | 295.86 | 144.38 |
| 18 | LT VIII B Pub Lighting | 49.08 | 20.94 | 102.00 | 44.54 | 104.97 | 46.81 | 107.27 | 47.80 |
| 19 | LT IX Ad & Hoardings | 1.01 | 2.03 | 2.06 | 4.27 | 2.15 | 4.55 | 2.21 | 4.67 |
| 20 | LT X Electrical vehicles | 0.01 | 0.01 | 0.62 | 0.32 | 0.66 | 0.34 | 0.70 | 0.36 |
|  | LT Subtotal | 8633.82 | 4822.40 | 16936.75 | 9646.30 | 17169.90 | 10024.05 | 17811.80 | 10431.47 |
| 1 | HT I A Industrial | 1068.35 | 731.36 | 2177.86 | 1514.52 | 2336.33 | 1649.19 | 2433.30 | 1722.13 |
| 2 | HT I B Industrial | 7.06 | 5.20 | 14.29 | 10.75 | 15.24 | 11.74 | 15.87 | 12.26 |
| 3 | HT II A General | 101.52 | 71.75 | 201.46 | 144.64 | 210.53 | 153.61 | 222.33 | 162.20 |
| 4 | HT II B General | 315.78 | 266.82 | 634.33 | 538.70 | 671.05 | 576.10 | 708.67 | 606.09 |
| 5 | HT III A Agriculture | 3.51 | 2.01 | 7.31 | 4.27 | 7.76 | 4.72 | 8.28 | 4.93 |
| 6 | HT III B Agriculture | 1.09 | 0.49 | 2.27 | 1.05 | 2.41 | 1.19 | 2.58 | 1.31 |
| 7 | HT IV A Commercial | 279.52 | 258.65 | 452.29 | 426.54 | 340.87 | 332.27 | 359.98 | 349.82 |
| 8 | HT IV B Commercial | 99.13 | 94.01 | 297.21 | 314.73 | 435.78 | 414.56 | 460.21 | 439.25 |
| 9 | HT V Domestic | 8.67 | 6.94 | 17.50 | 14.18 | 17.98 | 14.82 | 18.88 | 15.52 |
| 10 | HT VI | 0.00 | 0.00 | 0.00 | 0.00 | 45.00 | 27.00 | 90.00 | 54.00 |
|  | HT Sub Total | 1884.62 | 1437.24 | 3804.53 | 2969.36 | 4082.96 | 3185.19 | 4320.11 | 3367.49 |
| 1 | EHT 66 KV Industrial | 104.74 | 64.02 | 264.92 | 161.67 | 352.38 | 214.57 | 361.19 | 222.49 |
| 2 | EHT 110KV Industrial | 246.56 | 145.23 | 613.10 | 361.82 | 751.99 | 436.50 | 770.79 | 446.17 |
| 3 | EHT 220 KV Industrial | 52.47 | 30.59 | 110.71 | 64.98 | 122.73 | 70.23 | 126.41 | 72.22 |
| 4 | EHT General A | 7.29 | 4.27 | 15.93 | 9.39 | 15.61 | 9.34 | 16.08 | 9.63 |
| 5 | EHT General B | 25.68 | 22.52 | 56.10 | 48.12 | 54.99 | 47.32 | 56.64 | 48.83 |
| 6 | EHT Railway Traction | 146.96 | 90.95 | 304.21 | 189.24 | 334.20 | 208.04 | 350.91 | 217.69 |
| 7 | KMRL | 8.07 | 4.69 | 15.81 | 9.00 | 15.82 | 9.34 | 16.29 | 9.60 |
|  | EHT Sub Total | 591.78 | 362.26 | 1380.78 | 844.23 | 1647.71 | 995.34 | 1698.30 | 1026.65 |
| 1 | KPUPL | 47.79 | 31.02 | 95.01 | 62.44 | 96.65 | 64.14 | 101.22 | 66.81 |
| 2 | CSEZ | 29.10 | 18.09 | 58.63 | 36.86 | 59.03 | 37.54 | 59.46 | 37.78 |
| 3 | RPL | 13.43 | 7.26 | 31.84 | 17.35 | 35.17 | 19.41 | 37.43 | 20.48 |
| 4 | Technopark | 46.62 | 28.98 | 93.90 | 55.22 | 99.31 | 55.10 | 104.74 | 58.08 |
| 5 | Cochin Port trust | 18.92 | 12.94 | 36.77 | 25.50 | 36.99 | 25.89 | 37.80 | 26.38 |
| 6 | Thrissur Corporation | 81.96 | 55.06 | 163.06 | 110.99 | 165.28 | 113.72 | 169.82 | 116.47 |
| 7 | KDHPCL | 29.40 | 13.26 | 61.25 | 30.52 | 64.04 | 34.68 | 67.88 | 36.52 |
| 8 | MES | 35.90 | 24.20 | 71.74 | 49.48 | 72.07 | 50.79 | 73.18 | 51.44 |
| 9 | Karnataka | 0.37 | 0.26 | 0.79 | 0.58 | 0.80 | 0.60 | 0.82 | 0.61 |
| 10 | Infopark | 5.14 | 3.21 | 10.51 | 6.82 | 11.04 | 7.39 | 11.59 | 7.70 |
| 11 | Smart City | 2.16 | 1.47 | 4.29 | 2.91 | 4.50 | 3.03 | 4.73 | 3.16 |
|  | Subtotal | 310.79 | 195.75 | 627.80 | 398.68 | 644.88 | 412.28 | 668.66 | 425.43 |
|  | Grand Total | 11421.01 | 6817.65 | 22749.86 | 13858.57 | 23545.45 | 14616.86 | 24498.87 | 15251.04 |
|  | Open Access | 216.26 |  | 216.26 |  |  |  |  |  |
|  | Total | 11637.26 |  | 22966.11 |  |  |  |  |  |

Impact of revision in power factor incentive.

10.23.8	Honorable Commission vide order dated 08.07.2019 refixed the power factor incentive and approved the power factor incentive and approved the net revenue for the control period as in the following table:

| Table 10. 116 : Revenue from sale of power after deducting power factor incentive (Rs Cr) |  |  |  |
| --- | --- | --- | --- |
| Item | FY-20 | FY-21 | FY-22 |
| Revenue | 13254.67 | 13874.17 | 14523.36 |
| Power factor incentive | 44.99 | 32.5 | 33.95 |
| Net Revenue from Tariff | 13209.68 | 13841.67 | 14489.41 |

10.23.9	The actual power factor incentive disbursed during the year 2018-19 was Rs 114.07Cr. As per the incentive disbursed for September 2019, the average incentive works out to 6 paise per unit. Based on the same the power factor incentive and net revenue for the FY-20, FY-21 and FY-22 are computed and is furnished in the table below

| Table 10. 117 Revised Net Revenue for the control period (Rs Cr.) |  |  |  |
| --- | --- | --- | --- |
| Item | 2019-20 | 2010-21 | 2021-22 |
| Revenue | 13858.57 | 14616.86 | 15251.04 |
| Power factor incentive | 46.96 | 32.47 | 34.10 |
| Net Revenue from Tariff | 13811.61 | 14584.39 | 15216.94 |

Revenue from external sales

10.23.10	KSEBL has been taking earnest efforts to sell the excess power   whenever opportunity available at better rates. Honorable Commission vide MYT order dated 08.07.2019 approved the revenue from external sales for the second control period as per table given below.

| Table 10. 118 Revenue from External sales of surplus energy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Claimed in MYT petition |  |  |  |  |  |
|  | Unit | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| Energy for external sales | MU | 1,598.63 | 1,925.48 | 1,036.37 | 624.75 |
| Rate | Rs./kWh | 4.46 | 5 | 5 | 5 |
| Total Revenue expected | Rs.Crore | 712.97 | 962.74 | 518.19 | 312.38 |
| Approved by Honorable Commission |  |  |  |  |  |
|  | Unit | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| Energy for external sales | MU | 1,513.27 | 1,885.40 | 1,231.36 | 811.21 |
| Rate | Rs./kWh | 5 | 5 | 5 | 5 |
| Total Revenue expected | Rs.Crore | 756.64 | 942.7 | 615.68 | 405.61 |

10.23.11	Based on the revised energy availability and energy requirement the revised quantum of surplus energy and the revenue from external sales @ Rs 4 per unit is given in table below

| Table 10. 119 Revenue from external sales of surplus energy-Revised claim |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Unit | April-Sept 2019 | FY-20 | FY-21 | FY-22 |
| Energy for external sales | MU | 55.93 | 56.32 | 1644.05 | 2,070.88 |
| Rate | Rs./kWh | 3.12 | 3.12 | 4.00 | 4.00 |
| Total Revenue expected | Rs.Crore | 17.45 | 17.56 | 657.62 | 828.35 |

10.23.12	Total estimate of revenue from sale of power during the Control period

The total net revenue estimated for the next control period is given in the table below

| Table 10.120 Total Revenue for the control period (Rs Cr) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Revised claim |  |  |  |  |
| Item | April-Sept 2019 | FY-20 | FY-21 | FY-22 |
| Net Revenue from Sale of power | 7025.98 | 13,811.61 | 14,584.39 | 15,216.94 |
| Revenue from external sales | 17.45 | 17.56 | 657.62 | 828.35 |
| Total revenue | 7043.43 | 13829.17 | 15242.01 | 16045.29 |

10.24 REVENUE GAP

10.24.1	The revenue gap proposed and approved for the control period is given below:

| Table 10. 121 Revenue Gap (Rs.Cr) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Claimed in the petition |  |  |  |  |
| Particulars | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| Net ARR | 14247.34 | 15512.26 | 16348.76 | 17240.92 |
| Net Revenue | 13146.64 | 14113.20 | 14283.48 | 14722.01 |
| Revenue Surplus (Gap) | -1100.71 | -1399.05 | -2065.28 | -2518.91 |
| Approved by Honorable Commission |  |  |  |  |
| Net ARR | 13,222.47 | 14,952.93 | 15,402.08 | 15,893.55 |
| Net Revenue | 13,190.32 | 14,152.38 | 14,457.33 | 14,895.02 |
| Revenue Gap | -32.15 | -800.55 | -944.75 | -998.53 |

10.24.2	The revised revenue gap for remaining part of the control period is given below:

| Table 10. 122 Revenue Gap (Rs.Cr) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Revised Claim |  |  |  |  |
| Particulars | April-Sept 2019 | FY-20 | FY-21 | FY-22 |
| Net ARR (Table 10. 110) | 7610.00 | 14414.72 | 15935.87 | 17184.32 |
| Revenue from SoP (Table 10. 120) | 7043.43 | 13829.17 | 15242.01 | 16045.29 |
| Revenue Surplus(+)/ Gap(-) | -566.57 | -585.55 | -693.86 | -1139.03 |

***

| ANNEXURE I |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DYUTHI PROGRESS AS ON 31-03.2019 for 2018-19 WORK ( Under 21 work heads) |  |  |  |  |  |  |  |  |
| Sl.No. | Master Group |  | Work Scheduled |  | Work In Progress |  | Work Completed |  |
|  |  |  | Qty | Amt(in Cr.) | Qty | Amt(in Cr.) | Qty | Amt(in Cr.) |
| 1 | Construction - HT line | Km | 1064 | 98.79 | 642 | 47.26 | 119 | 10.07 |
| 2 | Construction - LT Line | Km | 539 | 32.37 | 112 | 5.57 | 45 | 1.81 |
| 3 | Construction – DTR | No | 416 | 14.98 | 370 | 13.55 | 140 | 5.12 |
| 4 | Replacement DTR | No | 87 | 1.98 | 86 | 1.90 | 259 | 4.80 |
| 5 | Construction - FPI | No | 2366 | 2.84 | 663 | 0.80 | 42 | 0.05 |
| 6 | Construction – Switches | No | 5138 | 24.08 | 3923 | 17.53 | 1082 | 4.69 |
| 7 | Conversion – HT Line | Km | 179 | 26.55 | 29 | 4.27 | 1 | 0.09 |
| 8 | Conversion -LT Line | Km | 1491 | 41.27 | 336 | 9.29 | 181 | 4.75 |
| 9 | Reconductoring - HT line | Km | 802 | 6.49 | 400 | 3.24 | 107 | 0.87 |
| 10 | Reconductoring - LT Line | Km | 15416 | 66.02 | 2623 | 11.18 | 693 | 2.97 |
| 11 | Standardisation - HT line | Km | 84 | 2.69 | 32 | 1.04 | 3 | 0.08 |
| 12 | Standardisation -Other Structures | No | 10374 | 27.76 | 9408 | 11.75 | 4572 | 4.34 |
| 13 | Metering - Boarder Meter | No | 353 | 1.95 | 198 | 1.09 | 18 | 0.10 |
| 14 | Metering - DTR Meter | No | 2777 | 2.74 | 745 | 0.73 | 30 | 0.03 |
| 15 | Faulty Meter Changing | No | 329153 | 27.19 | 94307 | 7.89 | 18443 | 1.58 |
| 16 | Shifting DTR | No | 105 | 0.76 | 92 | 0.67 | 23 | 0.16 |
| 17 | Shifting of lines | Km | 19 | 0.53 | 14 | 0.39 | 2 | 0.07 |
| 18 | Earthing | No | 56642 | 5.65 | 15670 | 2.23 | 1375 | 0.25 |
| 19 | Dismantling - Lines | Km | 172 | 1.36 | 46 | 0.37 | 14 | 0.11 |
| 20 | Dismantling - DTR/ Structure | No | 7 | 0.00 | 8 | 0.01 | 2 | 0.00 |
| 21 | Other Works |  | 47944 | 23.12 | 10147 | 5.35 | 2206 | 1.54 |
| Amount (in Cr. Lakhs) |  |  |  | 409.13 |  | 146.09 |  | 43.48 |
| System Generated Progress-Work Scheduled will be changed day by day. |  |  |  |  |  |  |  |  |

| ANNEXURE II |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DYUTHI PROGRESS AS ON 11.03.2020 for 2018-19 WORK ( Under 21 work heads) |  |  |  |  |  |  |  |  |
| Sl.No. | Master Group |  | Work Scheduled |  | Work In Progress |  | Work Completed |  |
|  |  |  | Qty | Amt(in Cr.) | Qty | Amt(in Cr.) | Qty | Amt(in Cr.) |
| 1 | Construction - HT line | Km | 137.24 | 13.06 | 614.00 | 59.39 | 1091.87 | 85.16 |
| 2 | Construction - LT Line | Km | 104.20 | 7.84 | 289.62 | 21.07 | 662.73 | 39.42 |
| 3 | Construction – DTR | No | 49.00 | 1.78 | 199.00 | 7.31 | 920.00 | 33.70 |
| 4 | Replacement DTR | No | 19.00 | 0.37 | 23.00 | 0.52 | 853.00 | 16.92 |
| 5 | Construction - FPI | No | 537.00 | 0.64 | 1349.00 | 1.62 | 1246.00 | 1.50 |
| 6 | Construction – Switches | No | 465.00 | 2.73 | 2065.00 | 9.82 | 8237.00 | 35.95 |
| 7 | Conversion – HT Line | Km | 14.60 | 2.16 | 117.96 | 17.46 | 51.12 | 7.56 |
| 8 | Conversion -LT Line | Km | 156.33 | 4.26 | 405.03 | 11.45 | 1853.30 | 51.08 |
| 9 | Reconductoring - HT line | Km | 305.91 | 2.48 | 406.18 | 3.29 | 533.94 | 4.32 |
| 10 | Reconductoring - LT Line | Km | 1775.41 | 7.63 | 5994.26 | 25.83 | 14757.12 | 63.18 |
| 11 | Standardisation - HT line | Km | 13.35 | 0.43 | 53.02 | 1.72 | 33.44 | 1.05 |
| 12 | Standardisation - LT line | Km | 28.98 | 0.45 | 197.52 | 3.05 | 691.23 | 10.05 |
| 13 | Standardisation -Other Structures | No | 972.00 | 1.92 | 5247.20 | 11.36 | 30121.26 | 36.33 |
| 14 | Metering - Boarder Meter | No | 62.00 | 0.34 | 154.00 | 0.85 | 212.00 | 1.17 |
| 15 | Metering - DTR Meter | No | 380.00 | 0.38 | 829.00 | 0.82 | 1382.00 | 1.36 |
| 16 | Faulty Meter Changing | No | 9526.00 | 0.97 | 30916.00 | 2.67 | 610194.00 | 50.39 |
| 17 | Shifting DTR | No | 20.00 | 0.14 | 43.00 | 0.31 | 199.00 | 1.46 |
| 18 | Loss Reduction – Shifting line | Km | 0.00 | 0.00 | 8.00 | 0.22 | 29.51 | 0.83 |
| 19 | Earthing | No | 5907.00 | 0.66 | 13886.00 | 1.63 | 38012.00 | 4.76 |
| 20 | Dismantling - Lines | Km | 6.77 | 0.05 | 58.56 | 0.46 | 155.66 | 1.23 |
| 21 | Dismantling - DTR/ Structure | No | 0.00 | 0.00 | 5.00 | 0.00 | 26.00 | 0.02 |
| 22 | Construction - Others | No | 7841.00 | 4.15 | 21783.20 | 11.67 | 46014.84 | 23.48 |
| Total |  |  |  | 52.45 |  | 192.51 |  | 470.93 |
| System Generated Progress-Work Scheduled will be changed day by day. |  |  |  |  |  |  |  |  |

| ANNEXURE III |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DYUTHI PROGRESS AS ON 11.03.2020 for 2019-20 WORK ( Under 21 work heads) |  |  |  |  |  |  |  |  |
| Sl.No. | Master Group |  | Work Scheduled |  | Work In Progress |  | Work Completed |  |
|  |  |  | Qty | Amt(in Cr.) | Qty | Amt(in Cr.) | Qty | Amt(in Cr.) |
| 1 | Construction - HT line | Km | 1193.07 | 108.70 | 344.20 | 28.64 | 28.08 | 2.30 |
| 2 | Construction - LT Line | Km | 1073.58 | 80.92 | 92.74 | 5.75 | 28.70 | 1.51 |
| 3 | Construction – DTR | No | 560 | 20.14 | 200 | 7.34 | 36 | 1.29 |
| 4 | Replacement DTR | No | 165 | 3.56 | 68 | 1.41 | 84 | 1.81 |
| 5 | Construction - FPI | No | 1858 | 2.23 | 1570 | 1.88 | 145 | 0.17 |
| 6 | Construction – Switches | No | 2914 | 14.27 | 1358 | 5.97 | 354 | 1.42 |
| 7 | Conversion – HT Line | Km | 96.71 | 14.31 | 22.05 | 3.26 | 5.48 | 0.81 |
| 8 | Conversion -LT Line | Km | 1828.90 | 51.43 | 192.85 | 5.15 | 111.29 | 3.09 |
| 9 | Reconductoring - HT line | Km | 778.40 | 6.30 | 143.59 | 1.16 | 7.53 | 0.06 |
| 10 | Reconductoring - LT Line | Km | 21225.82 | 90.85 | 2701.72 | 11.54 | 636.56 | 2.73 |
| 11 | Standardisation - HT/LT line | Km | 714.26 | 11.53 | 89.42 | 1.61 | 62.11 | 0.89 |
| 12 | Standardisation -Other Structures | No | 21260 | 37.59 | 6933.30 | 10.48 | 2676.00 | 3.39 |
| 13 | Metering - Boarder Meter | No | 67 | 0.37 | 16 | 0.09 | 3 | 0.02 |
| 14 | Metering - DTR Meter | No | 1215 | 1.20 | 239 | 0.24 | 44 | 0.04 |
| 15 | Faulty Meter Changing | No | 235776 | 19.75 | 179987 | 14.64 | 151793 | 12.28 |
| 16 | Shifting DTR | No | 140 | 1.01 | 32 | 0.23 | 4 | 0.03 |
| 17 | Loss Reduction – Shifting line | Km | 34.73 | 0.99 | 10.57 | 0.30 | 0.71 | 0.02 |
| 18 | Earthing | No | 40017 | 4.38 | 9301 | 1.34 | 733 | 0.10 |
| 19 | Dismantling - Lines | Km | 134.13 | 1.06 | 52.64 | 0.42 | 7.18 | 0.06 |
| 20 | Dismantling - DTR/ Structure | No | 6 | 0.00 | 0 | 0.00 | 1 | 0.00 |
| 21 | Construction - Others | No | 58106 | 28.85 | 12698 | 7.55 | 3408 | 1.58 |
| Total Amount (Rs Cr) |  |  |  | 499.44 |  | 109.00 |  | 33.60 |
| System Generated Progress-Work Scheduled will be changed day by day. |  |  |  |  |  |  |  |  |

Annexure IV

Status of central aided   system strengthening projects as on March & Sept 2019

Deena DayalUpadhyaya Grama JyothiYojana (DDUGJY ): Distribution Strengthening for Rural Areas including service connections in Rural areas

- Sanctioned Project Cost 	: Rs. 485.37Cr

- Date of Sanction	: 05.01.2016

- GOI Fund received	: Rs.238.63 Cr

- Own Funds infused 	: Rs 209.63 Cr

- Total Funds	: Rs 448.26 Cr

- Total Disbursements under scheme as on 30.09-2019 is Rs. 448.26 Cr.(92.35%)

- Overall Financial Progress as per Physical Achievement as on 30.09-2019 is 99.5%

- Target date of completion: Physical by 30.09.2019, Submission of Closures by 31.03.2020

- Service connections completed (Given-122727/target-98527)

- State is 100% Electrified as on today.

SansadAdarshGramYojana( SAGY)

- Sagy is one component under DDUGJY aiming holistic development of village sector by MPs.

- Work in all selected villages (27/27) completed.

- Funding pattern is same as in DDUGJY

- Revised DPR after incorporating BOQ changes submitted to REC along with SLSC recommendations.

- Field Inspection by REC Quality Monitor (RQM): Inspection completed in all Districts Defects reported in these districts are being rectified.

SAUBHAGYA : Scheme for 100% electrification of House holds : Free connection including one point wiring to certain categories of households as per scheme guidelines

- Sanction was received for “the project for re-connection of 3 Lakh Rural Households de-electrified in the flood, under SAUBHAGYA” for a total project cost of Rs. 90.00 Crore.  Upon submitting required details to REC,MOP released Rs 15.2 Cr as advance.Rs.26.12 Cr was released on   30-09-2019 . Total  fund received from GoI under Soubhagya is  Rs.41.32 Cr. Funding pattern under Soubhagya is Grant 60%, Loan 30% & State share 10% (Additional 15% grant if project is completed within timeline).

IPDS:System Strengthening for Urban & Semi urban areas

- Sanctioned Project Cost                    	:   Rs.592.07Cr

- Date of sanction                                 	:  15.6.2016

- Extended Target Date of completion  	:  30.09.2019

- GoI Fund received	:   Rs.107.41Cr

- Total disbursement through PFMS  	:   Rs.  373.98 Cr–     63%

- Own Funds infused	:   Rs. 266.57 Cr    (Rs. 59.207 Cr is 10% - utility contribution as per guidelines)

- Overall Financial Progress as per Physical Achievement   :   Rs. 489.23 Cr               –  82.63%

R-APDRP-PART B Distribution strengthening of 43 RAPDRP Towns

(This scheme is now subsumed under IPDS - 50% grant, if losses are brought to 15% or below)

- Total Project Cost sanctioned for the  State 		 :	Rs:1078.25Cr

- Date of commencement			 : 	08.06.2010

- Original date of completion 			 :	 31.03.2017

- Extended Target Date of Completion :-	-31st  March 2018 (for 8 schemes)

- KSEBLtd  has implemented this project through departmental execution mode.All Towns work completed and closed and reported to PFC

- Financial Progress as per Closure		: Rs.1115.18 Cr (103.42% - City schemes have

more than 100%)

As per closure proposal submitted to PFC balance fund (10%) of Rs. 96.584Cr  is released by PFC. Thus total amount received from PFC is Rs. 258.324 Cr (25%)

This scheme has 50% grant, if losses are brought to 15% or below.

Project stand completed with financial closures submitted for 43 schemes.

| DDUGJY- status as on 31-03-2019 and as on 30-09-2019 (Rs Cr.) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl No: | Particulars | Achievement as on 30-09-19 |  | Achievement as on 31-03-19 |  | Achievement for the FY 18-19 |  | Achievement for the FY 19-20 up to 30.09.19 |  |
|  |  | Qty | Amt | Qty | Amt | Qty | Amt | Qty | Amt |
| 1 | Service connections (No:s) | 122727 | 36.82 | 118324 | 35.50 | 7752 | 2.35 | 4403 | 1.32 |
| 2 | Construction of 11kV lines(ckt km) | 1257.45 | 97.52 | 1245.2 | 96.71 | 662.78 | 63.39 | 12.246 | 0.81 |
| 3 | Construction of LT lines(ckt km) | 2793.51 | 102.04 | 2722.86 | 99.76 | 564.57 | 26.59 | 70.648 | 2.28 |
| 4 | Installation of distribution transformers (No:s) | 549 | 16.19 | 544 | 16.05 | 198 | 5.86 | 5 | 0.15 |
| 5 | Enhancement of distribution transformers (No:s) | 50 | 1.27 | 50 | 1.27 | 28 | 0.79 | 0 | 0.00 |
| 6 | LT Reconductoring (ckt. km) | 670.54 | 15.17 | 666.23 | 15.05 | 281.65 | 7.34 | 4.31 | 0.12 |
| 7 | HT Reconductoring ckt. km) | 370.789 | 9.45 | 349.499 | 8.94 | 141.9 | 4.18 | 21.29 | 0.51 |
| 8 | Others |  | 80.37 |  | 72.97 |  | 63.45 |  | 7.41 |
|  | Total |  | 484.48 |  | 465.26 |  | 224.39 |  | 19.22 |

| IPDS –status of works as on 31-03-2019 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Particular | Unit | Revised Sanctioned Qty | Physical Achievement | Revised Project Cost | Financial achievement |
|  |  |  |  |  | (Rs. In Cr.) | as on 31-03-2019 |
| 1 | Status of New Substation: | Nos | 3 | * | 29.62 | 17.05 |
|  | 66/11kV & 33/11 KV S/S : New |  |  |  |  |  |
|  | 1. 33kV Pandalam S/s - 2*5 MVA => 10MVA |  |  |  |  |  |
|  | 2. 66kV GIS S/s - 2*10 MVA => 20MVA, including 1.5 km |  |  |  |  |  |
|  | 66kV UG feeder |  |  |  |  |  |
|  | 3. 33kV Containerised S/s, VB, Tvpm- 2*5MVA => 10 MVA |  |  |  |  |  |
|  | including 2.5km 33kV UG Cable feeder |  |  |  |  |  |
| 2 | 110/33 ; 33/11 KV S/S : Additional Transformer | Nos. | 1 | 0.75 | 2.47 | 1.53 |
| 3 | 66/11kV & 33/11 KV S/S : Transformer capacity enhancement | Nos. | 14 | 8 | 14.93 | 7.91 |
| 4 | Renovation & Modernisation of 110, 66 & 33/11 kV SS | Nos. | 95 | 86 | 24.33 | 21.78 |
| 5 | New 33 KV new OH feeder | km | 26.87 | 7.78 | 9.74 | 2.04 |
| 6 | 66 KV feeders Reconductoring | km | 27.7 | 26.56 | 7.5 | 7.21 |
|  | Kalamassery - Vytila 66 kV feeder - 5.7 km - HTLS |  |  |  |  |  |
|  | 66kV CHKD feeder - 22 km - AL 59 conductor |  |  |  |  |  |
| 7 | 33 kV Line Bay Extension at EHV station | Nos | 6 | 3.035 | 2.53 | 1.12 |
| 8 | 11 kV Line : New Feeder/ Feeder Bifurcation | km | 326.69 | 256.707 | 21.31 | 16.75 |
| 9 | 11 kV Line : Augmentation/Reconductoring | km | 184.97 | 101.34 | 6.26 | 3.38 |
| 10 | Arial Bunched Cable (HT & LT) | km | 1152.393 | 343.574 | 178.89 | 58.84 |
| 11 | 11 kV UG Cable | km | 192.19 | 50.407 | 60.07 | 15.75 |
| 12 | 11 KV Bay Extension | km | 0 | 0 | 0 | 0 |
| 13 | Installation of Distribution Transformer | Nos. | 829 | 745 | 25.89 | 22.98 |
| 14 | Capacity enhancement of LT sub-station | Nos. | 384 | 363 | 10.52 | 9.84 |
| 15 | LT Line : New Feeder/ Feeder Bifurcation | km | 218.015 | 128.061 | 10.57 | 6.03 |
| 16 | LT Line : Augmentation/Reconductoring | km | 2416.49 | 1790.94 | 61.85 | 51 |
| 17 | Capacitor Bank | Nos. | 0 | 0 | 0 | 0 |
| 18 | HVDS | Nos. | 107 | 8 | 1.07 | 0.08 |
| 19 | Metering | Nos. | 655393 | 459030 | 79.55 | 38.9 |
|  | Consumer meter |  |  | 457959 | 0 | 35.19 |
|  |  | 1 ph | 583808 | 428318 | 0 | 29.77 |
|  |  | 3 ph | 48113 | 29222 | 0 | 5.35 |
|  |  | CT | 273 | 101 | 0 | 0.07 |
|  | Smart Meter |  | 20000 | 0 | 0 | 0 |
|  |  |  | (ES/Ksdpm) |  |  |  |
|  | Boundary Meter |  | 288 | 136 | 0 | 2.04 |
|  | Feeder Meter |  | 250 | 40 | 0 | 0.16 |
|  | DTR Meter |  | 2661 | 895 | 0 | 1.5 |
| 20 | Provisioning of solar panel | kWp | 5465 | 4245 | 38.26 | 29.72 |
| 21 | RMU, Sectionaliser, Auto reclosures, FPI etc. | E | 58 | 3 | 2.78 | 0.14 |
| 22 | Others |  |  |  | 3.93 | 2.46 |
|  | GRAND TOTAL |  |  |  | 592.07 | 388.6 |
|  | % overall Progress |  |  |  |  | 53.23% |

| IPDS –Status of works as on 30-09-2019 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Particular | Unit | Revised Sanctioned Qty | Physical Achievement | Revised Project Cost | Financial achievement |
|  |  |  |  |  | (Rs. Lac ) | as on 30-09-2019 (Total) |
| 1 | Status of New Substation: | Nos | 3 | * | 29.83 | 28.67 |
|  | 66/11kV & 33/11 KV S/S : New |  |  |  |  |  |
|  | 1. 33kV Pandalam S/s - 2*5 MVA => 10MVA |  |  |  |  |  |
|  | 2. 66kV GIS S/s - 2*10 MVA => 20MVA, including 1.5 km |  |  |  |  |  |
|  | 66kV UG feeder |  |  |  |  |  |
|  | 3. 33kV Containerised S/s, VB, Tvpm- 2*5MVA => 10 MVA |  |  |  |  |  |
|  | including 2.5km 33kV UG Cable feeder |  |  |  |  |  |
| 2 | 110/33 ; 33/11 KV S/S : Additional Transformer | Nos. | 1 | 1 | 2.47 | 2.01 |
| 3 | 66/11kV & 33/11 KV S/S : Transformer capacity enhancement | Nos. | 11 | 11 | 12.82 | 12.65 |
| 4 | Renovation & Modernisation of 110, 66 & 33/11 kV SS | Nos. | 171 | 119 | 29.25 | 27.47 |
| 5 | New 33 KV & 66 kV new OH feeder | km | 45.62 | 29.4 | 23.69 | 17.96 |
| 6 | 66 KV feeders Reconductoring | km | 27.7 | 27.025 | 8.55 | 8.45 |
|  | Kalamassery - Vytila 66 kV feeder - 5.7 km - HTLS |  |  |  |  |  |
|  | 66kV CHKD feeder - 22 km - AL 59 conductor |  |  |  |  |  |
| 7 | 33 kV Line Bay Extension at EHV station | Nos | 6 | 5 | 2.52 | 1.85 |
| 8 | 11 kV Line : New Feeder/ Feeder Bifurcation | km | 331.06 | 311.021 | 21.81 | 20.29 |
| 9 | 11 kV Line : Augmentation/Reconductoring | km | 163.97 | 140.002 | 5.47 | 4.67 |
| 10 | Arial Bunched Cable (HT & LT) | km | 1162.85 | 837.216 | 180.67 | 145.88 |
| 11 | 11 kV UG Cable | km | 203.74 | 85.045 | 64.27 | 26.58 |
| 12 | 11 KV Bay Extension | km | 0 | 0 | 0 | 0 |
| 13 | Installation of Distribution Transformer | Nos. | 829 | 818 | 26.18 | 25.58 |
| 14 | Capacity enhancement of LT sub-station | Nos. | 384 | 378 | 10.52 | 10.32 |
| 15 | LT Line : New Feeder/ Feeder Bifurcation | km | 187.05 | 165.426 | 9 | 7.82 |
| 16 | LT Line : Augmentation/Reconductoring | km | 2354.21 | 2193.709 | 59.36 | 62.3 |
| 17 | Capacitor Bank | Nos. | 0 | 0 | 0 | 0 |
| 18 | HVDS | Nos. | 107 | 107 | 1.07 | 1.07 |
| 19 | Metering | Nos. | 632649 | 580527 | 58.8 | 52.1 |
|  | Consumer meter |  |  | 577997 | 0 | 44.8 |
|  |  | 1 ph | 583808 | 537261 | 0 | 37.34 |
|  |  | 3 ph | 48113 | 40289 | 0 | 7.37 |
|  |  | CT | 273 | 129 | 0 | 0.09 |
|  | Smart Meter |  | 0 | 0 | 0 | 0 |
|  | Boundary Meter |  | 288 | 234 | 0 | 3.75 |
|  | Feeder Meter |  | 250 | 108 | 0 | 0.15 |
|  | DTR Meter |  | 2661 | 2188 | 0 | 3.39 |
| 20 | Provisioning of solar panel | kWp | 5465 | 4215 | 38.26 | 29.51 |
| 21 | RMU,Sectionaliser, Auto reclosures, FPI etc. | E | 68 | 27 | 3.26 | 0.53 |
| 22 | Others |  |  |  | 4.28 | 3.53 |
|  | GRAND TOTAL |  |  |  | 592.07 | 586.14 |
|  | % overall Progress |  |  |  |  | 82.83% |
|  | Pandalam 33 kV -Substation commissioned on 28.3.19 |  |  |  |  |  |
|  | 66kV Trippunithura GIS -S/s completed - commisioned on 17.12.18 |  |  |  |  |  |
|  | 33 kV Vydyuthi BhavanTvpm - Containerised S/s - Commissioned on 21.6.2019 |  |  |  |  |  |

Annexure  V

Project Status of Centrally Aided new /ongoing  IT projects as on September 2019

IPDS – Real Time Data Acquisition System (RT-DAS) for Non-SCADA Towns

The objective of this scheme is to implement Feeder Remote Terminal Units (FRTUs) and Real Time Data Acquisition System in the substations within the Non-SCADA towns for the automated measurement of SAIDI/SAIFI (Reliability Indices) to assess the reliability of power  As per the direction of PFC, a DPR amounting to Rs.5.25 Crore was prepared with an estimated cost of Rs.2.75 Lakhs per substation having 5 or lesser number of feeders and Rs. 3 Lakhs per substation having 5 or more number of feeders for setting up FRTU and Rs. 1.5 Cr for setting up RT-DAS at Data center. Approval of the DPR is awaited.

Towards the objective of reducing the AT&C loss and to attain better power distribution reliability, accurate measurement, diagnosis and remedial actions are essential.  The reliability of power is measured in the terms of SAIFI and SAIDI which requires solutions based on real time. A Real Time Data Acquisition System (RT-DAS) using FRTU (Feeder Remote Terminal Unit) at substations can be implemented to measure the reliability of power. PFC has sanctioned the implementation of RT-DAS in KSEBL to cover non-SCADA RAPDRP and IPDS towns with eligible approved project cost of Rs. 5.25 Crore having a grant component of Rs. 3.15 Crore, being 60% of approved project cost. 40 towns under RAPDRP are eligible for the scheme (excluding 3 SCADA towns) along with 23 exclusive IPDS towns. Subsequently, the RFP was published on 07.08.2018 vide E-Tender No. CE (IT, CR & CAPs)/05/2019-20.

The technical bid was opened on 07.09.2019.  The following 3 bidders participated in the tender: Dongfang Electronics Pvt Ltd., SCOPE TNM Pvt Ltd. and SUN Industrial automation and solutions Pvt Ltd. The technical evaluation is progressing.

IPDS-Phase 2 : Incremental IT for 21 Towns/municipalities.

The project is for IT enabling of 21 Towns / municipalities as Phase II for facilitating Energy Audit. The work includes supply and installation of additional hardware in the towns (such as router, switches, PC, printer etc), GPS based GIS survey of assets, data migration and project management. The project also covers preparation of Feeder and Town wise consumer database / bill book and linking with existing GIS Application, so that Feeder wise AT&C loss and Town wise AT&C loss can be calculated. An amount of Rs 22.86 Cr for 21 towns has been approved. About 60 % of the project cost (ie Rs 13.72 Cr) would be GOI grant. The project was sanctioned on 20.02.2017 and to be completed within 30 months.

The DPR for Rs 22.86 Crore, covering 21 Towns was sanctioned on 20-02-2017. M/s. KPMG Advisory Service Private Limited was appointed as the Project Management Agency. M/s. BCITS, Bangalore (LoA issued on 17.11.2018) was appointed as the IT Implementation Agency.

The Letter of Award for the procurement of server, storage etc. has been issued and their installation is expected to be completed within few weeks.

Implementation related activities like ring fencing has been completed in 21 towns. ITIA installed about 367 modems in border meters and feeder meters of all 21 towns. Modems are communicating with central server. Kottarakkara town has been selected as pilot town. DTR metering has been completed in Kottarakkara town and activities for asset surveying and DTR modem installation have been progressing.189/198 DTR meters have been installed in the pilot town of Kottarakara. AoI of Vaikom, Chengannur , Mavellikara, Pandalam and Adoor towns have been finalised. AoI surveying is progressing in Pala. DTR meter installation has been completed in Nileshwaram, Nilambur, Chavakkad, Tripunithara, Kottakal, Manjeri, Koothuparambu, Chengannur, Chottanikara and is progressing in remaining towns.

IPDS-ERP Implementation

The Enterprise Resource Planning implementation under IPDS covers the area of Human Resource Management, Financial Accounting and Supply Chain Management system. The project was sanctioned on 07.02.2018 and expected to be completed within 30 months from date of sanction.

M/s. KPMG Advisory Service Private Limited was appointed as the Project Management Agency (PMA) to conduct an As-Is-Study and Gap-Analysis of business processes & work flow applications, to find which activities are yet to be re-engineered and automated from the perspective of an integrated ERP solution. DPR for Rs. 42.64 crore was approved vide letter dated 7.2.2018 of PFC. NIT & RFP were published on 27.11.2018. No bidders participated even after re-tendering. On 09.05.2019,  Board  decided to carry out customisation/development and implementation of ERP solution with the in-house development team under IT wing as part of partial turnkey basis execution sanctioned by PFC and supply, installation and commissioning of server, storage etc through open tender process. The processes towards in-house development are in progress.GoI Grant would be Rs. 25.58 Crores (60% of the Project Cost).

IPDS-Smart Meter Implementation

The objectives of this scheme is to convert Energy meters of (a)all consumers under Electrical Section, Kesavadasapuram to smart meters at a project cost Rs 19.5251 Cr (approval awaited) and (b) another 3,21,800 energy meters with per month consumption more than 230 units from all categories for UDAY participating states. M/s KPMG Advisory Service Private Limited has been appointed as Project Management Agency. The Approved Amount of the project is Rs. 64.36 Cr (for 3,21,800 nodes) with  60% GoI Grant amounting to  Rs. 38.62 Cr. The project was sanctioned on 28.03.2018 and to be commissioned by 30 months from date of sanction.

PFC has sanctioned DPR for Rs. 64.36 crore towards implementation of 3,21,800 smart meters for consumers within IPDS towns. M/s KPMG has been appointed as the PMA. NIT for appointing implementation agency was published on 25.07.2018. It has been decided to select the implementation agency through reverse bidding process. The RFP document was published on 17.06.2019 with last date of bid submission on 5.08.2019. Pre-bid meeting was convened on 27.06.2018. 10 firms participated in the Pre-bid meeting. Based on the request from various Smart meter manufacturers, the date of bid submission extended to 26.08.2019. The technical bids were opened on 30.08.2019. Only M/s. United Electrical Industries Ltd. participated in the tender. The Pre-Qualification Committee Meeting held on 31.08.2019 decided to proceed with re-tendering. The details are being submitted to the Board for deciding future action plan.

Under IPDS (System strengthening) about 16,000 smart meters are to be installed for all consumers under Electrical Section, Kesavadasapuram. NIT was published on 12.07.2018 and the RFP was released on 23.07.2018. Technical bid opened on 31.10.2018. Two bidders M/s L&T Ltd and M/s United Electricals have submitted bids. As the latter failed to meet PQ criteria it was re-tendered with last date of submission as 23.11.2018.  Again M/s L&T Ltd and M/s United Electricals Ltd. submitted bids. Technical bids opened on 27.11.18.  On completion of Technical evaluation, only M/s. L&T Ltd. qualified. Financial bid of the qualified single bidder (M/s. L&T Ltd) was opened on 01.02.2019. On 25.04.2019, Board decided to re-tender the work on the basis of financial bid evaluation. Re-tendered with last date of bid submission as 24.06.2019. Technical bid was opened on 27.06.2019 and the following 4 bidders participated: Larsen and Toubro Limited, Genus Power Infrastructures Limited, KSEDC Ltd- Keltron Controls Division and United Electrical Industries Ltd. The Pre-Qualification Committee Meeting held on 31.08.2019 decided to technically qualify Larsen and Toubro Limited and Genus Power Infrastructures Limited to proceed further (to open financial bids after after successful completion of testing). M/s. L&T Ltd. had requested to consider their earlier test results, which was carried out as part of the cancelled tender. The Technical evaluation of the sample meters submitted by M/s. Genus is progressing.

In the meantime, PFC has directed for the closure of System strengthening projects by September 2019. Hence, new work was included under IPDS, instead of implementation of 16000 Smart meters.

Smart Grid Kochi

The Scope of Work and proposed features of implementation of Smart Grid in Kochi Smart City covering five electrical sections includes the following:

| - AMI (Approx One Lakh Nos) | - EV Charging Stations. |
| --- | --- |
| - Peak Load Management. | - Estimated cost:- Rs 103.35 Cr |
| - Distribution Trans Monitoring (240) | - Approved cost: Rs 90.87 |
| - Facilitate Distr Gen (Roof top PV). | - GoI Grant:- 30% |
| - Energy Storage Systems (500 kWh x 2nos). | - Project to be completed in 18 months |

Ministry of Power (MoP) has directed the utilities to formulate Detailed Project Reports (DPRs) for implementation of Smart grid. As part of Smart City Kochi project, Smart Grid implementations for section offices under the Smart City area were preferred. A Project proposal including an estimate cost of       Rs.103.35Crore for the Smart Grid implementation at smart city Kochi is prepared. The State Level Project Monitoring Unit had finalized the Detailed Project Reports (DPR) for implementation of Smart Grid in the State and recommended the DPR to the National Smart Grid Mission (NSGM). The Kochi Smart Grid DPR has been submitted to National Smart Grid Mission (NSGM) by KSEBL on 16-2-2018. The NSGM technical committee held on 28-4-2018 approved the project for a moderated cost of Rs.90.7Crore which includes Rs. 29.31Crore as NSGM grant portion (30%) and Rs.68.39 Crore as Utility Share (70%). The DPR had been revised based on the decision of the SLPMU committee held on 03.05.2019. The Scope of Work & Proposed feature of implementation of Smart Grid in Kochi includes the following:

• Advance Metering Infrastructure

• Peak Load Management.

• DT Monitoring

• Facilitate Distributed Generation (Roof top PV).

• Electric Vehicle Charging Stations.

The Project has to be commissioned within 18 months post award. The Smart Grid project involves component cost of Smart Meter, EV Charging Stations which are the same involved in developing Smart City. The Smart Grid implementation will advance the Smart City features and helps in conserve the energy using advance metering infrastructure and control systems. Also KSEBL requested necessary financial aid from Cochin Corporation and Cochin Smart Mission Limited d towards the implementation of Smart Grid implementation in Kochi Smart City.

CYBER SECURITY

The request for Proposal for Appointment of Consultant for Cyber Security Assessment, Preparation of Cyber Security Policy & Security audit for KSEBL is in progress. Rfp floated and tender opened on 15-09-2018. PQ evaluation is in progress. Estimated cost is Rs. 50 Lakhs. Project implementation is planned for the control period 2018-19.

CENTRALLY AIDED NEW – ONGOING IT PROJECTS

R – APDRP (Part A) Scheme- Status of implementation

Government of India sanctioned R-APDRP scheme during 2010, which focuses on actual demonstrable performance in terms of sustained loss reduction. In Kerala, 43 schemes were sanctioned under Part A and B. The total outlay for Part A schemes comes to Rs.214.32 Crore towards IT and Rs 83.15 Cr towards SCADA scheme

Part-A of R-APDRP – IT:

This scheme includes works related for IT enabling of 43 Town schemes for installation of Data Centre, Customer Care Centre and related applications for facilitating Energy Audit, IT based consumer indexing GIS mapping etc. with 100 % grant. The details of projects are given below.

- Total Project Cost Sanctioned by PFC 			: Rs. 214.38 Cr

- IT Implementing Agency				: M/s. KEPCO KDN

- LoA for IT Implementation issued on  			: 07.11.2012

- Date of Completion of Implementation 			: 31st March 2018

- Financial Closure				   : 30th April 2018

- Estimated Project cost at completion			 : Rs 175.23 Cr

- Amount Received so far from PFC 			: Rs. 141.23Cr.

- TPIEA verification by PGCIL completed and reports submitted to PFC.

The project stand closed and financial closure is submitted. Release of balance tranche Rs 34.03 Cr is pending from PFC for which letter already issued to Nodal Officer, PFC.

PART A of R-APDRP- SCADA (Ongoing Project): This scheme envisages the advanced distribution automation, loss reduction and reliability improvement in three cities of Trivandrum, Kochi & Kozhikode. For this scheme, 100 % grant is provided by Government of India. The details of project are as follows:

- Implementation Agency 			: M/s Schneider Electric India Pvt. Ltd.

- Total Project cost as Approved by PFC 	: Rs. 83.15Cr

- Awarded cost to Schneider Electric (SIA)	: Rs. 56.7Cr

- Target date of Completion			:30.09.2019- project completed

- Status:

PART A SCADA is implemented in Three cities of Tvpm, Kochi & Kozhikode for advanced distribution automation, loss reduction and reliability .The SCADA/DMS system is functional with 50 RTUs (at substations) and 2865 FRTUs (at RMU locations) installed and integrated to the three  SCADA/DMS Control Centres TVM,EKM,KZD with counts of RTU 14,25 & 11 and FRTU counts 1450,644 & 771 respectively. The integration of GIS with SCADA/DMS is implemented and Distribution Network developed in SCADA/DMS system for Trivandrum, Ernakulam& Kozhikode towns using CIM XML from GIS. Final tests such as SAT, SAVT for three cities completed and 11kV Control room operations for the cities shifted to the SCADA/DMS control room. Third Party Independent Evaluation Agency (TPIEA) Audit completed. Documentation is in progress. All the required milestones have been reached within the Target date of completion sanctioned by PFC, ie, 30th September 2019.The project is fully funded by PFC.

The sanctioned cost of the project is Rs. 83.15Cr and award cost is Rs.61.86 Cr.

Annexure VI

EV charging infrastructure proposal and mode of implementation

Kerala aims to be one of the front-runners in the adoption of Electric Mobility and has been among the earliest States in India to adopt an approved Electric Vehicles Policy and tariff. The Policy targets to have 1 million EVs on the road by 2022 with special emphasis to public transport. As part of the Policy, the Kerala State Electricity Board Ltd. (KSEB) has been appointed as the State Nodal Agency for establishing a charging infrastructure for electric vehicles (EVs).

As articulated in the Central and State policies, there is an urgent need to promote electric mobility. A critical requirement for accelerating EV adoption is the development of an appropriate charging infrastructure for EVs. In this scenario, KSEB with the support of the Government of Kerala, is committed to take a proactive stance in promoting both public and private investment in charging infrastructure in the State.

# EV charging infrastructure proposal

EV Charging Infrastructure development plan focus on the following

- Setting up Energy availability and Grid capability:

KSEB has already initiated steps to ensure power availability and to enhance the transmission capacity for bringing more power from the national grid foreseeing future demand. Steps are being taken by KSEB to increase the Renewable Energy content in its energy portfolio, thereby increasing the clean component of the State’s energy basket. It is expected that during the pilot stage energy requirement for meeting the eV demand will be around 1.5 Million Units (MU) per day, which is less than the current daily fluctuation in energy demand of KSEB and can easily be met.

However, there can be local stresses in the distribution grid to cater to the power requirements of EV charging, especially during peak hours. In the long run, requirement in transport sector will play a significant role in the total power demand. It is expected that in a scenario when the population of eV in the State matches with existing number of ICE vehicles, the daily energy requirement for eV will be approximately 50 MU. KSEB will be taking steps to closely monitor the available resources and will make adequate investments to cater to the emerging needs.

- Setting up Charging Pyramid:

EV Charging Ecosystem is expected in three tiers; at (1) homes & offices, (2) other slow charging stations and (3) fast charging stations. It is expected that the vast majority of energy consumed in EV charging will be delivered through overnight charging at the owners’ homes or office (in case of official vehicles) using slow AC chargers. Home charging is expected to be the predominant mode of charging for auto-rickshaws as well as private two & four - wheelers in Kerala. Dependence on interim charging arises when a user has to travel beyond the range offered by the vehicle battery. Even in such cases, slow AC charging will be sufficient for most situations. Slow Charging is expected to play a major role in public charging considering the vehicle mix which is likely to be predominated by two- and three-wheelers which do not usually feature fast charging technologies. Public DC fast chargers enable users to quickly charge their vehicles and resume their journey during long trips. Establishments like government offices, malls, hotels etc may install fast chargers for the convenience of visitors or to attract customers.

KSEB will partner with the Government of Kerala, PSUs and other private investors to create an adequate network of fast charging stations in Kerala, in the following categories.

| Category A | At public places for commercial purpose and will be made available to any individual without any restrictions (e.g., EV Charging station established in at Municipal Parking Lots, Petrol Stations, Streets, Malls, and Market Complexes etc.) |
| --- | --- |
| Category B | Within the premises of a State or Central Government Office Complexes, Hospitals, educational Institutions or any other Public Office for non-commercial use. (e.g., EV Charging station established in KSEB complexes, Public Office Building, Govt Hospital complex etc.) |
| Category C | Within the semi-restricted premises for commercial or non-commercial purpose wand will be made available to any individual for charging of eVs without any restrictions. (e.g., EV Charging stations established by Taxi Car / Auto Operators, Co-operative housing societies) |

Swappable batteries on a pay-as-you-go model which eliminates the need to pay upfront for the battery, thus potentially making EVs cheaper than conventional vehicles is another business model which KSEB foresee in near future. From utility perspective, swappable batteries also supports grid balancing as they can be charged during low demand period.

Development Phases

The development of a charging ecosystem is envisaged in three stages i.e. Pilot, Coverage and Scaling – each with distinct objectives, investment requirements, risk and role of various stakeholders. During the Pilot stage, the first few charging stations will be up in each city wherein the new technology is demonstrated and awareness is created. The chargers installed in the initial period will generate only minimal revenue.

During the second phase, KSEB, with the support of Government plan to provide adequate support to companies who plan to invest in establishing a state-wide coverage for its charging station network. KSEB proposes to establish more (i) Public Charging Stations – PCS (with AC as well as DC fast, moderate & slow chargers) catering to different vehicle segments in same location and (ii) Mini Charging Stations (with AC slow charges necessitating minimum investment) which give more focus on two & three wheelers, taking the vehicle population in to account. If required, KSEB may also partner with potential investors through various business models.

Scaling Stage begins once a critical mass of vehicle adoption has been reached and the utilization of charging stations begins to increase. The focus will then shift to waiting time and effective utilization of chargers. State may take few more years to reach this stage.

Road map for rollout

- KSEB has already established the first fast charging station in the State at Government Secretariat, Trivandrum. KSEB is now in to installation of Charging Stations in Board’s own land along the National Highway in six Corporation areas viz. Thiruvananthapuram, Kollam, Kochi, Thrissur, Kozhikode & Kannur. These locations will have multiple charging points Tender for the same is already floated by KSEB. The work is expected to be completed within two months’ from the date of award of work.

- In appreciation of the efforts being taken by KSEB to support eMobility initiatives of KSEB, Government of Kerala was kind enough to allocate ₹ 8.2 Crore from its e-mobility promotion fund. This amount will be utilized for installation of Charging Stations in 32 locations in Thiruvananthapuram District; the work is expected to be commenced by May, 2020 and completed by September,2020.

- The Government of Kerala is gearing up to increase the adoption of EVs for its fleet of official vehicles. KSEB will work with the Government to quickly set up charging stations in all District Collectorates as well as other office complexes with more than 20 government vehicles. These infrastructures can be utilized by the public as well. DC fast chargers can also be installed based on need.

- KSEB has invited an Expression of Interest (EoI) for short listing firms for setting up Public Charging Stations in Kerala, based on which the detailed technical specification and business model will be finalized. About 70 locations are already identified across Trivandrum, Kochi and Kozhikode districts, where PCS will be installed this year itself. Further rollout to other Districts will follow. Slow charging kiosks as well as fast charging coverage are proposed along the National Highway. KSEB has set an internal target of March 2021 for completing these installations.

- Phasing out of existing vehicles of KSEB: As a matter of policy, KSEB will scrap the own old vehicles plying in the Corporate offices and other field Offices and replace it with EVs in a phased manner. In future, petrol and diesel cars hired on contract for use of field officers will also be replaced with EVs, as far as possible. If required, the services of EESL / NTPC / Government / Private Agencies for possible leasing of EVs will be explored for this.

- Utilization of FAME-II incentives: The Department of Heavy Industries, Government of India has invited applications for availing incentives for the deployment of charging infrastructure in cities where funds are being offered for setting up 25 charging stations having 7 or 8 chargers each under each city. The incentives ranges from 50% to 100% of capital cost depending on the category, specifically defined in this respect in the EOI. With Kerala having 7 one million plus population cities, KSEB has already applied under this scheme. 131 EV charging stations were approved in four cities  as phase I, remaining will be approved in phase II

- Backend support: KSEB is taking proactive role in ensuring backend support to stakeholders. A web based application will soon be developed for demand aggregation. This will not only help land / building owners who are willing to utilize their available space for eV charging infrastructure and Investors to identify the available properties, but also enable KSEB to plan ahead for developing its infrastructure. A mobile app is also being developed for capturing characteristics of such properties – which is now in testing phase before final rollout. A mobile App required for the EVSE will also be developed by our team with the support of Kerala State IT mission for enabling eV users to locate and navigate to available charging points in their location, advance booking of slots, Integrated Payment Gateways, Authentication and Remote Monitoring by eVSE (Electric Vehicle Supply Equipment) owners etc.

- E Mobility for Public Transport: The State’s eV Policy is developed with a key focus on Public Transport. Transformation of conventional Three Wheelers and fleets including those owned by KSRTC may be gradual, but will eventually happen. KSEB is gearing up to support these initiatives. Mini Charging Stations exclusively for 2 & 3 Wheelers will be a cost effective option for Auto Stands. It is expected that large fleet operators, including KSRTC, will be setting up their own (Captive) charging stations, but will require support of KSEB for network adequacy. During the last pilgrim season at the request of KSRTC, KSEB has developed the backend infrastructure to set up a charging station at Nilakkal base camp, which helped KSRTC to successfully operate eBuses between Nilakkal & Pampa. Now KSRTC is operating 3 charging stations, one each at Thiruvananthapuram, Haripad & Aluva. KSEB has expressed its support to KSRTC and is now helping them in selection of charging locations.

KSEB is committed to support the rapid adoption of electric vehicles in Kerala. It recognizes that Electric Mobility is still in its early stages with rapid changes in technologies, standards, Government policies and consumer preferences. KSEB will continue its efforts to understand the emerging needs and come up with proactive plans to satisfy the ambitions of the State in Electric Mobility and a clean future.

****************************************************************

****************************************************************