MID TERM PERFORMANCE REVIEW OF SBU-G FOR FY-20 & REVISED ESTIMATE FOR FY-21 & FY-22

CHAPTER – 7   SBU – GENERATION

7.1. INTRODUCTION

7.1.1 The total installed capacity of KSEBL as on 31.03.2019 is 2237.59 MW of which 92.01 % of the installed capacity is Hydel, 7.15% is Thermal, 0.75 % from solar and rest from the wind. The total installed capacity became 2238.035 MW as on 30.09.2019.

7.1.2. There are altogether 38 hydroelectric generating stations owned by KSEBL (as on 30.09.2019) of   which 15 are Major Hydel Stations and the rest are Small Hydro Plants. The Major Hydel stations contribute to a total capacity of 1938.75MW whereas the small Hydro stations contribute to 120MW. The lists of all the Hydro stations are shown in Annexure G1.

7.1.3. There are two thermal stations owned by KSEBL which have a total installed capacity of 159.96MW with a firm annual generation capability of 1035.6 MU. However, due to high variable cost of thermal stations, presently KSEBL is limiting the generation from these liquid fuel stations to the extent essential. The total generation from these stations up to 30.09.2019 in the current FY was 11.87 MU only ie. 11.77 MU from KDPP and that of BDPP was 0.10 MU.

7.1.4. KSEBL presently owns one wind farm at Kanjikode which is having an installed capacity of 2.025 MW and an annual generation capability of 4MU. KSEBL has an installed capacity of 16.85 MW on solar projects as on 31.03.2019.  Further, KSEBL has targeted to add around 8.6 MW (excluding soura) during the remaining period of current control period on own land and buildings. The Soura scheme launched by the Government of Kerala aims at developing solar PV roof top/Ground mounted/floating solar plants aggregating to 1000MWp in the state by 2021. The list of solar projects completed and planned on own land and building during the control period is shown in Annexure G2.

7.1.5. The gross energy generated from the KSEBL owned stations during the FY 2019-20 up to 30.09.2019 was 3059.09MU, out of which 3036.97MU (99.27%) was from own hydro electric stations and the remaining 0.73% was generated from thermal, wind and solar stations. The source wise capacity and generation details are given in the Table 7.1below.

| Table 7.1 : Source wise Generation |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Installed Capacity (MW) |  | Generation (MU) |  |
| Source | As on 31.03.2019 | As on 30.09.2019 | FY 2018-19 | Till 30.09.2019 in FY 2019-20 |
| Hydel | 2058.75 | 2058.75 | 7602.41 | 3036.97 |
| Thermal | 159.96 | 159.96 | 4.089 | 11.87 |
| Wind | 2.03 | 2.03 | 1.33 | 0.75 |
| Solar | 16.85 | 17.295 | 18.54 | 9.5 |
| Total | 2237.59 | 2238.035 | 7626.37 | 3059.09 |

  - Capital Investment for the control period

7.2.1. As per the MYT Petition, KSEBL has proposed for a capital addition of Rs 2341.67 Cr for SBU-G during the control period. However, the Hon’ble Commission has given provisional approval to projects accounting to Rs 1261.22Cr. It was specifically stated in the order that the GFA addition is provisional and doesn’t mean that the Commission has approved the said figures nor has disallowed the balance portion of the GFA proposed by KSEBL. The provisionally approved figure with respect to the proposal given by KSEBL is shown below in Table 7.2.

| Table 7. 2 : Proposed/Approved Capital Addition of SBU-G |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Particulars | 2018-19 |  | 2019-20 |  | 2020-21 |  | 2021-22 |  | Proposed Total | Approved Total |
|  |  | Proposed | Approved | proposed | Approved | proposed | Approved | proposed | Approved |  |  |
| 1 | New Hydel Projects |  |  |  |  | 87.99 | 53.03 | 87.23 | 42.42 | 175.22 | 95.45 |
| 2 | R & M Projects |  |  | 12.22 | 12.23 | 327.20 |  |  |  | 339.42 | 12.23 |
| 3 | Solar New |  |  | 57.51 | 57.51 |  |  |  |  | 57.51 | 57.51 |
| 4 | Ongoing Hydel Projects | 99.47 | 65.79 | 384.5 | 354.94 | 331.41 | 278.30 | 578.48 | 111.34 | 1393.86 | 810.37 |
| 5 | RMU - Ongoing |  |  | 20.40 | 20.40 | 252.26 | 252.26 |  |  | 272.66 | 272.66 |
| 6 | Others –DRIP etc |  |  | 3 | 3 | 100 | 10 |  |  | 103 | 13 |
|  | Total | 99.47 | 65.79 | 477.63 | 448.07 | 1098.86 | 593.59 | 665.71 | 153.76 | 2341.67 | 1261.22 |

7.2.2  New Hydel Projects

To meet the increasing energy demand of the State, KSEBL has proposed new hydro projects during the control period from 2018-19 to 2021-22. However, due to various reasons explained below the completion date of Pazhassi Sagar and Annakayam projects has been forced to be extended.  The Pazhassi Sagar work was re-tendered twice due to lack of participation of suitable qualified bidders whereas the Anakkayam work needs to have sanction from the Ministry of Enviorment, Forest and Climate Change. These factors has affected severely in completing the above projects within the time frame. Further, based on the actual site conditions revision has also been accorded to the above projects by the Board. The details of the new hydel projects with the expected period of completion are shown below.

| Table 7.3 : New Hydel Projects for the control period |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No | Name of Scheme | Gen Capacity (MW) | Design Energy (MU) | Project Cost (Cr.) | Expected year of Completion |  | Status of the Project |
|  |  |  |  |  | MYT | REVISED |  |
| 1 | Pazhassi Sagar | 7.5 | 25.8 | 104.37 | 2020-21 | 2021-22 | Civil work under progress |
| 2 | Peruvanamuzhi | 6 | 24.70 | 84.11 | 2021-22 | 2021-22 | Civil work under progress |
| 3 | Chinnar | 24 | 76.45 | 269.87 | 2022-23 | 2022-23 | Civil work under progress |
| 4 | Anakkayam | 7.5 | 22.83 | 139.64 | 2022-23 | 2023-24 | Tendering of work in progress |

7.2.3  R&M Projects

R&M works have been proposed at Switch Yard of Idukki Power Station and at Kuttiyadi HEP during the control period. The work at Idukki involves renovation & upgradation of switch yard equipments, implementation of Automatic generation control system and automated metering system whereas in Kuttiyadi the work involves enhancing the capacity of 3 units of Kuttiyadi HEP from 25MW to 27.5MW and also laying a separate penstock for the KES unit of 50MW. The works were originally scheduled for completion during the FY 2019-20 and 2020-21 respectively; but in consideration of more time required for dismantling the existing equipments the same has been rescheduled to 2020-21 and 2022-23. The details of the project along with the revised year of completion are shown below.

| Table 7.4 : New R&M projects for the Control period |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No | Name of Scheme | Capacity (MW) | Annual Energy (MU) | Project Cost (Rs. Cr.) | Expected year of Completion |  | Status of the Project |
|  |  |  |  |  | ARR | REVISED |  |
| 1 | Idukki 220 kV Yard | -- | -- | 12.23 | 2019-20 | 2020-21 | Tendering of work in progress |
| 2 | Kuttiyadi | 3x27.5* 3x50 | 530.8 | 377.41 | 2020-21 | 2022-23 | Work to commence |

* Enhanced capacity

Further to the above, 4 new R&M works on Generating station switchyards are proposed and planned to be completed during the control period. The work involves replacing the damaged equipments in the switch yard of Sabarigiri, Lower Periyar, Sengulam and Kakkayam power stations. The details of the works are shown in the table below. All the works are currently under progress and is expected to be completed in the FY 2020-21.

| Table 7. 5 : New R& M Hydel projects |  |  |  |
| --- | --- | --- | --- |
| No | Name of Scheme | Expected year of Completion | Project cost (Cr.) |
| 1 | Switchyard work at Sabarigiri | 2020-21 | 3.19 |
| 2 | Switchyard work at Lower Periyar | 2020-21 | 5.21 |
| 3 | Switchyard work at Sengulam | 2020-21 | 1.87 |
| 4 | Switchyard work at Kakkayam | 2020-21 | 0.40 |

7.2.4. New Solar Projects

A total No of 4 Solar projects having a total capacity of around 10.1MW was proposed by KSEBL on own land and buildings during the control period 2018-19 to 2021-22. However due to the site conditions and technical constraints, the Brahamapuram  project has been revised from 6MWp to 4MWp whereas the Kanjikode project has been revised to 3MWp from 2MWp. Further, new projects are also proposed at Ettumanoor and Mylatty. The details of the new solar projects with the expected date of commissioning are shown in table below.

| Table 7.6 : New Major Solar projects for the control period |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Name of Scheme | Capacity (MWp) |  | Annual Energy (MU) | Project Cost (Rs. Cr.) | Expected year of Completion |  | Status of the project |
|  |  | ARR | REVISED |  |  | ARR | REVISED |  |
| 1 | Brahmapuram | 6.5 | 4.0 | 4.9 | 22.8 | 2019-20 | 2020-21 | Tendered. Work Order to be issued |
| 2 | Kottiyam | 0.6 | 0.6 | 0.84 | 3.75 | 2019-20 | 2019-20 | Work in progress |
| 3 | Kanjikode | 2 | 3 | 3.679 | 17.10 | 2019-20 | 2020-21 | Tendered. Work Order to be issued |
| 4 | Agali | 1 | 1 | 1.2 | 5.70 | 2019-20 | 2020-21 | Tendered. Work Order to be issued |
| 5 | Ettumanoor | - | 1 | 1.4 | 6.91 | - | 2022-23 | Approval sought of GoK |
| 6 | Mylatty | - | 1 | 1.44 | 5.82 | - | 2022-23 | Work awarded |

7.2.5. Ongoing Hydel Projects

Eight ongoing projects have been scheduled for completion during the control period.  Due to various reasons specified below, the completion of projects except for Kakkayam has been rescheduled. The Kakkayam project which was planned for commission on October, 2019 has been able to be completed before schedule date ie; on July, 2019. The details of the ongoing projects are furnished in the table 7.7 below:

| Table 7. 7 : Ongoing Hydel Projects for the control period |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No | Name of Scheme | Gen Capacity (MW) | Design Energy (MU) | Expected year of Completion |  | Project cost (Cr.) | Remarks on Revision |
|  |  |  |  | MYT petition | REVISED |  |  |
| 1 | Kakkayam | 3 | 10.39 | 2018-19 | COD ON 16.07.2018 | 43.2 | Completed earlier than scheduled |
| 2 | Boothathenkettu | 24 | 83.5 | 2019-20 | 2020-21 | 231.21 | Delayed due to flood and land acquisition issues. |
| 3 | Upper Kallar | 2 | 5.14 | 2019-20 | 2020-21 | 28.75 | Design revised due to geological factors |
| 4 | Porigalkuthu AES | 24 | 45.02 | 2019-20 | 2020-21 | 171.42 | Delay in getting the forest land |
| 5 | Chathankottunada | 6 | 14.76 | 2020-21 | 2021-22 | 79.00 | Delayed due to flood and protest of public against blasting |
| 6 | Thottiyar HES | 40 | 99 | 2020-21 | 2021-22 | 283.49 | Delayed due to flood |
| 7 | Sengulam Aug. Scheme |  | 85 | 2021-22 | 2021-22 | 113.00 | Due to problem encountered in tunnelling works |
| 8 | Pallivasal Ext Scheme | 60 | 153.9 | 2021-22 | 2021-22 | 550.00 | Due to retendering of works |

7.2.6. Ongoing RMU Projects

The following RMU works are also undergoing and planned to be completed during the control period. However due to soil conditions /natural calamity excess items of works were necessitated in the project which had incurred additional financial commitment. The details of the projects along with the present status of works are shown below.

| Table 7.8 : Ongoing RMU projects |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sl No | Name of the Scheme | Expected Year of Completion | Project Cost (Cr) | Status of work |
| 1 | Sholayar RMU | 2020-21 | 199.58 | 73% work completed) |
| 2 | Sengulam Pump House RMU | 2019-20 | 24.42 | 77% of Civil works completed |
| 3 | Idukki Stage I | 2020-21 | 89.9 | 60% of Physical work completed |

    - Review  of Capital Works (up to 30.09.2019) & Revised projection for 2019-20

Total no. of projects scheduled for completion during 2019-20 was nine, out of which only 2 projects namely; Sengulam PH work (RMU work) and the Kottiyam solar project is currently expected to be completed during the financial year 2019-20. All the remaining projects have been forced to be rescheduled due to various reasons. The main reason for rescheduling the projects are unexpected flood during 2019 & land acquisition problems experienced on acquiring project lands. However, with earnest efforts, KSEBL has been able to commission roof top solar projects ‘IPDS South’ (23 locations) having a total capacity of 0.445MW on 30.06.2019. The annual design energy expected from the above projects is 0.526MU and the total project cost comes to Rs 3.047Cr. The revised schedule of projects completing during the FY 2019-20 is shown below.

| Table 7. 9 : Projects scheduled & Revised for completion in the FY 2019-20 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No | Name of Scheme | Proposed completion in 2019-20 as per ARR |  |  | Revised Proposal for the FY 2019-20 |  |  |
|  |  | No of Projects | Name of Projects | Capital addition (Cr) | No of Projects | Name of the Project | Capital addition (Cr) |
| 1 | New Hydel Projects | - | - | -- | - | -- | - |
| 2 | R & M Projects | 1 | Idukki 220kV Switch Yard | 12.22 | -- |  |  |
| 3 | Solar New | 4 | Brahamapuram, Kottiyam, Kanjikode & Agali | 57.51 | 2 | Kottiyam, Roof top solar | (3.75+ 3.047) 6.797 |
| 4 | Ongoing Hydel Projects | 3 | Bhoothathenkettu, Upper Kallar, Poringalkuthu AES | 384.5 | -- |  |  |
| 5 | RMU - Ongoing | 1 | Sengulam PH | 20.40 | 1 | Sengulam PH | 24.42 |

    - Revised projection for 2020-21

As per the earlier schedule 6 projects has been expected to be completed by the FY 2020-21. However, due to various reasons like flood, delay in dismantling the existing equipments etc, the projects scheduled for completion has been forced to be extended. The Ongoing RMU projects at Idukki and sholayar are expected to be completed in the year as planned, whereas the rest of the work has been rescheduled.  The 7 projects which were to be completed in the FY 2019-20 have been rescheduled with commission date on 2020-21. New 4 R&M works started is expected to be completed by 2019-20. Accordingly, altogether there are 13 projects expected to be completed in 2020-21. The details of the same are shown in the table below.

| Table 7. 10 : Projects scheduled & Revised for completion in the FY 2020-21 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No | Name of Scheme | Proposed completion in 2020-21 as per ARR |  |  | Revised Proposal for the FY 2020-21 |  |  |
|  |  | No of Projects | Name of Projects | Capital addition (Cr) | No of Projects | Name of the Project | Capital addition (Cr) |
| 1 | New Hydel Projects | 1 | Pazhassi Sagar | 87.99 | - | -- | - |
| 2 | R & M Projects | 1 | Kuttiyadi | 327.20 | 5 | SY work at Idukki, Sabarigiri, Lower Periyar, Sengulam & Kakkayam | 22.90 |
| 3 | Solar New | -- | -- | -- | 3 | Brahamapuram Kanjikode & Agali -- | 45.6 |
| 4 | Ongoing Hydel Projects | 2 | Chathankottunada, Thottiyar | 331.41 | 3 | Bhoothathenkettu, Upper Kallar, Poringalkuthu AES | 431.38 |
| 5 | RMU - Ongoing | 2 | Idukki Stage 1, Sholayar | 252.26 | 2 | Idukki Stage 1, Sholayar | 289.48 |

    - Revised projection for 2021-22

As per the earlier schedule 3 projects has been expected to be completed by the FY 2021-22. The same is expected to be completed as planned only. Further, the new Pazhassisagar project and the ongoing Chanthankottunada and Thottiyar project is expected to be completed in the above FY.  Accordingly, altogether there are 6 projects expected to be completed in 2021-22. The details of the same are shown in the table below.

| Table 7. 11 : Projects scheduled & Revised for completion in the FY 2021-22 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No | Name of Scheme | Proposed completion in 2021-22 as per ARR |  |  | Revised Proposal for the FY 2021-22 |  |  |
|  |  | No of Projects | Name of Projects | Capital addition (Cr) | No of Projects | Name of the Project | Capital addition (Cr) |
| 1 | New Hydel Projects | 1 | Peruvanamoozhi | 87.23 | 2 | Pazhassi Sagar, Peruvanamoozhi | 188.48 |
| 2 | R & M Projects | -- | -- | -- | -- | -- | -- |
| 3 | Solar New | -- | -- | -- | -- | -- | -- |
| 4 | Ongoing Hydel Projects | 2 | Sengulam Aug, Pallivasal Ext. | 578.48 | 4 | Chathankottunada, Thottiyar Sengulam Aug, Pallivasal Ext. | 1025.49 |
| 5 | RMU - Ongoing | -- | -- | -- | -- | -- | -- |

7.2.10	The Hon’ble Commission in the MYT Order dated 08.07.2019 has only given provisional approval to the projects amounting to Rs 1261.21Cr for the control period. It has been stated in the order that the provisional approval does not mean that the Commission has approved the above GFA addition or dis allowed the balance portion of the GFA addition out of the total GFA addition proposed. The Commission has further stated that they will examine the capital investment separately and after going through public consultation process and prudence check will approve the same. The revised capital addition of SBU-G based on the progress noticed so far is shown in the table below.

| Table 7. 12 : Capital Addition of SBU-G |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No | Particulars | 2019-20 |  | 2020-21 |  | 2021-22 |  |
|  |  | Proposedin ARR | Revised | Proposed in ARR | Revised | Proposed in ARR | Revised |
| 1 | New Hydel Projects |  |  | 87.99 |  | 87.23 | 188.48 |
| 2 | R & M Projects | 12.22 |  | 327.20 | 22.90 |  |  |
| 3 | Solar New | 57.51 | 6.797 |  | 45.6 |  |  |
| 4 | Ongoing Hydel Projects | 384.5 |  | 331.41 | 431.38 | 578.48 | 1025.49 |
| 5 | RMU - Ongoing | 20.4 | 24.42 | 252.26 | 289.48 |  |  |
| 6 | Others –DRIP etc | 3 |  | 100 |  |  |  |
|  | Total | 477.63 | 31.217 | 1098.86 | 789.36 | 665.71 | 1213.97 |

    - For the above capital addition, the year wise expenditure outlay expected is shown in the table below.

| Table 7.13 Capital Expenditure Outlay of the Projects |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl No | Scheme | Total Project cost Cr | expense incurred upto 31.03.2018 | 2018-19 | 2019-20 | 2020-21 | 2021-22 | Expense after 2021-22 |
|  | New Projects |  |  |  |  |  |  |  |
| 1 | Pazhassi Sagar | 104.37 | 0.62 | 8.11 | 5.64 | 30 | 60 |  |
| 2 | Peruvananmuzhi | 84.11 | 2.51 | 3.6 | 10 | 30 | 38 |  |
| 3 | Chinnar | 269.87 | 21.74 | 6.35 | 20 | 35 | 110 | 76.78 |
| 4 | Anakkayam | 139.64 |  | 0.9 | 2 | 20 | 80 | 36.74 |
|  | R&M |  |  |  |  |  |  |  |
| 1 | Kuttiyadi | 377.41 |  |  | 1 | 35 | 40 | 301.41 |
| 2 | Idukki | 12.23 |  |  | 7.07 | 5.16 |  |  |
| 3 | Sabarigiri | 3.19 |  |  | 2.4 | 0.79 |  |  |
| 4 | Lower Periyar | 5.21 |  |  | 2.79 | 2.42 |  |  |
| 5 | Sengulam | 1.87 |  |  | 0.93 | 0.94 |  |  |
| 6 | Kakkayam | 0.4 |  |  | 0.4 |  |  |  |
|  | Ongoing R&M |  |  |  |  |  |  |  |
| 1 | Sengulam PH | 24.42 | 0.31 | 15.07 | 9.04 |  |  |  |
| 2 | Idukki Stage 1 | 89.9 | 26.844 | 35.92 | 3.762 | 23.374 |  |  |
| 3 | Sholayar | 199 |  | 54.84 | 6.57 | 137.59 |  |  |
|  | Ongoing Projects |  |  |  |  |  |  |  |
| 1 | Boothethankettu | 231.21 | 116.66 | 59.55 | 30 | 25 |  |  |
| 2 | Upper Kallar | 28.75 | 6.18 | 4.57 | 11 | 7 |  |  |
| 3 | Poringalkuthu AES | 171.42 | 109.49 | 26.6 | 24.95 | 10.38 |  |  |
| 4 | Chathankottunada | 79 | 5.51 | 9.49 | 16 | 35 | 13 |  |
| 5 | Thottiyar HES | 283.49 | 98.4114 | 6.68 | 30 | 80 | 23 | 45.398 |
| 6 | Sengulam Aug | 113 | 55.32 | 2.68 | 10 | 25 | 20 |  |
| 7 | Pallivasal Ext | 550 | 259.34 | 43.01 | 90 | 98 | 59.65 |  |
|  | Solar Projects |  |  |  |  |  |  |  |
| 1 | Brahmapuram | 22.8 |  |  |  | 22.8 |  |  |
| 2 | Kottiyam | 3.75 |  |  | 3.75 |  |  |  |
| 3 | Kanjikode | 17.1 |  |  |  | 17.1 |  |  |
| 4 | Agali | 5.7 |  |  |  | 5.7 |  |  |
|  | Total | 2817.84 | 702.9354 | 277.37 | 287.302 | 646.244 | 443.65 | 460.328 |

7.3   Estimation of Hydel Generation for the Control Period

7.3.1	Hydro availability estimation as per MYT order for the Control period

Taking into consideration the anticipated inflow proposed by KSEBL in the MYT petition, the estimated Hydro projection approved by the Hon’ble Commission from FY 2019-20 to FY 2021-22 is shown below in Table 7.14.

| Table7.14.Hydel Generation as per MYT Order |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Particulars | 2019-20 |  | 2020-21 |  | 2021-22 |  |
|  | Gross gen (MU) | Aux. Consp(MU) | Gross gen (MU) | Aux. Consp(M) | Gross gen (MU) | Aux. Consp(MU) |
| Existing Large Hydro | 6503.17 | 67.41 | 6294.28 | 65.03 | 6279.15 | 64.68 |
| Existing Small Hydro | 422.76 | 4.23 | 382.74 | 3.83 | 408.25 | 4.08 |
| Total Existing | 6925.93 | 71.64 | 6677.02 | 68.86 | 6687.35 | 68.76 |
| Ongoing and New Hydro |  |  |  |  |  |  |
| Kakkayam | 10.39 | 0.10 | 10.39 | 0.10 | 10.39 | 0.10 |
| Boothathenkettu | 48.71 | 0.49 | 83.50 | 0.83 | 83.50 | 0.83 |
| Upper Kallar | 2.14 | 0.02 | 5.14 | 0.05 | 5.14 | 0.05 |
| Poringalkuthu AES | 11.25 | 0.11 | 45.02 | 0.45 | 45.02 | 0.45 |
| Chathankottunada |  |  | 11.07 | 0.11 | 14.76 | 0.15 |
| Pazhassi Sagar |  |  | 9.51 | 0.10 | 22.83 | 0.23 |
| Thottiyar HES |  |  | 24.75 | 0.25 | 99.00 | 0.99 |
| Peruvanamuzhi |  |  |  |  | 21.48 | 0.21 |
| Sengulam Aug. Scheme |  |  |  |  | 85.00 | 0.85 |
| Sub Total New & Ongoing | 72.49 | 0.72 | 189.38 | 1.89 | 387.11 | 3.87 |
| Gross Generation | 6998.42 | 72.36 | 6866.40 | 70.75 | 7074.51 | 72.64 |
| Net Generation (less Aux. consumption.) | 6926.06 |  | 6795.65 |  | 7001.88 |  |

7.3.2 Review of Hydro–availability up to 30.09.2019.

The State of Kerala has witnessed heavy deficit in rainfall in the first half of southwest monsoon followed by heavy rainfall during the month of August, 2019. The overall net inflow (excluding spill) up to September, 2019 in the water year (2019-20) was less than the anticipated value projected in the ARR. As per the estimation, expected inflow was 4406.72MU from June to September, 2019 whereas the actual inflow was 3901.58MU only. The actual month wise net inflow of the stations is shown below. It may be noted that water stored in the Pambla dam has to be spilled in order to do maintenance work of the dam. Hence there is negative figure for inflow on Lower Periyar storage in the month of September, 2019.

| Table 7.15: Actual inflow from June 2019 to Sep 2019 (MU) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sl.no. | Station | Jun-19 | Jul-19 | Aug-19 | Sep-19 |
| 1 | KUTTIADI | 13.09 | 155.00 | 210.43 | 75.07 |
| 2 | PORINGAL | 0.00 | 24.10 | 28.71 | 15.49 |
| 3 | KUNDALA &MADUPETTY | 14.71 | 33.58 | 86.20 | 64.19 |
| 4 | ANAYIRANKAL &PONMUDI | 7.72 | 22.91 | 107.02 | 75.14 |
| 5 | NERIAMANGALAM | 11.62 | 32.61 | 50.24 | 53.48 |
| 6 | PAMBA&KAKKI | 34.39 | 118.51 | 405.35 | 265.30 |
| 7 | IDUKKI | 72.27 | 254.18 | 828.97 | 429.64 |
| 8 | EDAMALAYAR | 5.05 | 33.53 | 113.33 | 60.86 |
| 9 | LOWER PERIYAR | 0.81 | 14.83 | 19.09 | -27.70 |
| 10 | SHOLAYAR | 5.52 | 26.53 | 88.71 | 71.13 |
|  | TOTAL | 165.19 | 715.78 | 1938.03 | 1082.58 |

Considering the net available inflow for generation during the period upto September, 2019 for the water year 2019-20, the power availability from the generating stations for the control period needs to be revised.

7.3.3 Revised projection for 2019-20 from existing projects.

The inflow for the water year 2019-20 with actuals upto October, 2019 was found to be 5969.39MU against the 6487.41MU projected in the ARR. However, due to the floods in the year 2019,   Four nos. of major hydro stations and 14 Nos. of SHP’s have been badly affected thereby restricting generation from these stations. The stations affected by the flood and the details of restoration are attached as Annexure –G4. The reasons specified above have caused considerable reduction in the power availability from the hydro generating stations for the financial year 2019-20. Hence KSEBL was constrained to revise the hydel generation to 6510.91 against an approved value of 6925.93 MU.

7.3.4 Revised projection for 2020-21 from existing projects.

Due to reduction in net inflow available upto 30.09.2019, in the water year 2019-20, the inflow projected in the MYT petition for the remaining years of the control period needs revision. Taking into account the reduction of actual inflow in the water year 2019 (up to September, 2019), it is found that there will be a slight difference of 63.90 MU(reduction) from the  inflow projected for the financial year 2020-21 in the MYT petition. Hence, KSEBL has revised the generation for the financial year 2020-21 to 6614.23MU against the approved 6677.02MU.

7.3.5 Revised projection for 2021-22 from existing projects.

The reduction of average inflow experienced based on the reduction in the inflow for the year 2019-20 has also been taken into account for the financial year 2021-22. Accordingly, the generation has been revised to 6622.09MU against the approved value of 6687.40MU.

7.3.6 The revised inflow estimated based on the actual inflow upto October 2019 and thereafter 20 years average value is shown in Annexure-G3. The comparison statement of Inflow projected in the ARR with revised present projection for the coming years in the control period is shown below in Table 7.16.

| Table 7.16. Anticipated/ Revised estimate of Inflow |  |  |  |
| --- | --- | --- | --- |
| Water year | ARR | Revised | Difference |
| 2019-20 | 6487.41 | 5969.39 | (-)518.02 |
| 2020-21 | 6497.32 | 6433.42 | (-) 63.90 |
| 2021-22 | 6508.72 | 6441.63 | (-) 67.09 |

7.3.7 In tune to the above, the approved /revised projection of generation from the existing hydro stations is shown below in Table 7.17.

| Table 7.17 Comparison of Generation Proposed |  |  |  |  |
| --- | --- | --- | --- | --- |
| Year | ARR (A) | Approved (B) | Revised (C) | Difference D=C-B |
| 2019-20 | 6925.90 | 6925.93 | 6510.91 | (-)415.02 |
| 2020-21 | 6677.02 | 6677.02 | 6614.23 | (-) 62.79 |
| 2021-22 | 6687.40 | 6687.40 | 6622.09 | (-) 65.31 |

7.3.8 However, the  actual generation of hydro plants will have to be regulated based on the actual inflow, energy and peak demand, availability of power from CGS, traders, energy exchanges and short term markets etc. Accordingly, there may be changes in the estimated values based on the actual condition. The revised generation projection for the FY 2019-20 to 2021-22 from the generating stations is shown in Annexure – G5 (1) to G5 (3).

7.3.9 . Energy availability from New and Ongoing Hydel Projects

The Hon’ble Commission as per the MYT Order dated 08.07.2019 has provisionally approved generation from the new projects expected to be commissioned during the control period. However, due to uncontrollable events, the completion of certain projects has been rescheduled beyond the original CoD. Hence, the estimated generation proposed from the above stations has changed. Accordingly, the revised generation proposed from the New Hydel projects based on the current status for the coming years of the control period is shown below in Table- 7.18.

| Table 2.18.Hydel Generation as per MYT Order |  |  |  |
| --- | --- | --- | --- |
| Ongoing & New Hydro Projects | 2019-20 | 2020-21 | 2021-22 |
|  | Gross gen (MU) | Gross gen (MU) | Gross gen (MU) |
| Kakkayam | 5.612 | 5.612 | 5.612 |
| Boothathenkettu |  | 41.7 | 83.40 |
| Upper Kallar |  | 1.284 | 2.568 |
| Poringalkuthu AES |  | 45 | 45 |
| Chathankottunada |  |  | 7.38 |
| Pazhassi Sagar |  |  | 49.5 |
| Thottiyar HES |  |  | 12.6 |
| Sengulam Aug. Scheme |  |  | 42 |
| Total New & Ongoing | 5.612 | 93.596 | 248.06 |

7.3.10 In tune to the above generation proposed from the new Hydel projects, the total estimated hydel generation anticipated for the coming years of the control period will be as shown in table 7.19 below.

| Table 7.19: Revised Hydel Generation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Particulars | 2019-20 |  | 2020-21 |  | 2021-22 |  |
|  | Gross gen (MU) | Aux. Consp(MU) | Gross gen (MU) | Aux. Consp(M) | Gross gen (MU) | Aux. Consp(MU) |
| Total Existing | 6510.91 | 67.328 | 6614.23 | 68.043 | 6622.10 | 68.3194 |
| Ongoing and New Hydro | 5.612 | 0.056 | 93.60 | 0.936 | 248.06 | 2.4806 |
| Gross Generation | 6516.517 | 67.38426 | 6707.83 | 68.97981 | 6870.16 | 70.80 |
| Net Generation (less Aux. consp.) | 6449.133 |  | 6638.85 |  | 6799.36 |  |

7.4. Estimation of energy generation from Thermal Stations

7.4.1 As already submitted in MYT petition, because of the high variable cost of power from the two thermal stations, KDPP and BDPP, no generation has been proposed from these thermal generating stations throughout the control period. However, as part of giving quality and reliable power to the consumers and also part of having system stability, in contingency situations, the same had been used in the year 2018-19 and the quantum was 4.089 MU. For 2019-20 up to September, it was 11.87 MU only ie. 11.77 MU from KDPP and that of BDPP was 0.10 MU at a cost of Rs.5.46 Cr. The actual generation for October 2019 was 0.04 MU for KDPP. KSEBL was able to generate power at a rate of Rs.4/ unit from KDPP during 2019-20 using LSHS of BPCL stored in the premise of KDPP at a discounted rate of Rs.17240/MT. It may be noted that the market of LSHS is around Rs. 50,000/-(approx) including GST and transportation.

7.4.2. In future projections of the control period (from October 2019) no generation from the thermal stations has been accounted.  Hence in view of actual generation, the revised projection for FY 2019-20 is 11.91 only (KDPP-11.81 and BDPP - 0.1 MU).

7.4.3. The auxiliary consumption for BDPP is fixed at 5% and that of KDPP is 2.5% of gross generation as per the Tariff Regulation. Even though the thermal stations are not scheduled in the proposed generation plan for the control period, the auxiliary consumption needed for the stations are to be accounted as the station needs to be equipped for generation in any contingency situation. As per the past actual, a minimum of 1.5MU is needed annually for maintaining the above 2 stations. Hence, the same is been accounted in the estimation of energy.

7.5. Estimation of energy generation from Solar and Wind sources

7.5.1. Energy generation from solar projects:  KSEB Ltd has an installed solar capacity of 16.85 MW as on 31.03.2019 with an annual generation of about 18.54 MU. Further, KSEBL has added 0.445 MWp of solar projects up to 30.09.2019 thus totaling the capacity to 17.295 MW. As on 30.09.2019, the total generation achieved was 9.5 MU from the solar resources. The Kottiyam solar project with an installed capacity of 0.6 MWp is planned to be commission in the FY 2019-20. Accordingly, revised projection for FY 2019-20 is estimated as 22.57 MU.  The projected generation for the FY 2020-21 and 2021-22 is same as per the approval of the Hon’ble Commission in MYT order. The solar plants proposed to be commissioned during the control period are shown in Annexure G2. The expected generation based on solar projects for the years of the control period is shown in Annexure G6.

7.5.2. Energy generation from wind projects: The total wind energy capacity of KSEBL as on 30 .03.2019 is 2.025MW. At present there is no wind project planned by KSEBL during the control period. The generation received from the wind up to 30.09.2019 was 0.75 MU. Based on the current trends, it is expected to get a lesser generation than the figure approved by the Hon’ble Commission. However, if the approved generation is put for the months from Oct-2019, then also the generation for the FY 2019-20 will be 1.57MU. Since the generation from the wind source is meager and is highly depended on the climatic conditions, no revision from the approved generation is proposed by KSEBL for the FY 2020-21 and 2021-22.  The revised solar generation and the generation proposed from the wind is shown in table 7.20 below.

| Table 7.20 : Generation Plan of Solar & Wind |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Year | Solar (MU) |  | Wind (MU) |  |
|  | APPROVED | REVISED | APPROVED | REVISED |
| 2019-20 | 42.61 | 22.57 | 2.04 | 1.57 |
| 2020-21 | 47.60 | 47.60 | 2.08 | 2.08 |
| 2021-22 | 47.60 | 47.60 | 2.08 | 2.08 |

7.6. Auxiliary consumption of Generating Plants

7.6.1 The permissible auxiliary consumption of existing and new hydroelectric generating stations, as per regulation, ranges between 0.7 % to 1.2 % based on type of plants (Surface/underground) and excitation system. As per the new KSERC (Renewable Energy and Net Metering) Regulations, 2020, the auxiliary consumption of SHEPs is 1% and that of solar PV projects will be 0.25% of the gross generation.  Since there has been a revision in the commissioning dates of the projects planned, the auxiliary consumption estimated will also change depending on the same. Accordingly the revised auxiliary consumption of these hydro plants for the control period is given in Annexure G7. The actual auxiliary consumption for thermal stations upto 30.09.2019 was 0.985 MU and the revised projection for 2019 -20 is estimated as 1.5 MU and an auxiliary consumption of 1.5 MU is expected for the remaining period also.  The auxiliary of solar projects has been calculated as per the norms specified in the Regulation, ie; 0.25% of the gross generation.

7.7. Generation plan for the control period

7.7.1 A consolidated generation schedule of the Hydel stations including proposed projects is shown below. Net generation of KSEBL plants is calculated after deducting the auxiliary consumption from the gross generation.

| Table 7.21 : Hydel Generation Plan |  |  |  |
| --- | --- | --- | --- |
| Financial Year | Gross Hydel available(MU) | Aux. Consumption (MU) | Net Hydel Available(MU) |
| 2019-20 | 6516.517 | 67.38426 | 6449.133 |
| 2020-21 | 6707.83 | 68.97981 | 6638.85 |
| 2021-22 | 6870.16 | 70.80 | 6799.36 |

7.7.2 Even though, no generation proposed from the thermal stations of KSEBL for 2020-21 and 2021-22, a minimum auxiliary consumption needed for running the station, being 1.5 MU per year, based on past actual is taken into account in calculating energy availability. The auxiliary for the solar stations is taken as per the KSERC (Renewable energy and Net metering) Regulation, 2020.

|  |  | Table 7.22 : Total Generation Plan |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Financial Year | Net Hydel (MU) |  | Thermal (MU) | Solar (MU) | Wind (MU) | Aux. Cons Solar +thermal | Generation Available (MU) |
| 2019-20 | 6449.133 |  | 11.91 | 22.57 | 1.57 | 1.5564 | 6483.6266 |
| 2020-21 | 6638.85 |  | 0 | 47.60 | 2.08 | 1.619 | 6686.911 |
| 2021-22 | 6799.36 |  | 0 | 47.60 | 2.08 | 1.619 | 6847.421 |

7.7.3 The entire capacity and generation is allocated to the Distribution Business Unit of KSEBL.

7.8 Estimation of ARR of SBU-G for the control period

The estimation of ARR for Generation Business of KSEBL is prepared as per the KSERC (Terms and Conditions for determination of Tariff) Regulations, 2018, especially, Regulation 10 and 44.

7.8.1 Estimation of Operation & Maintenance Expenses

7.8.1.1 Regulation 45 of the tariff regulation specifies that, the generation business of KSEBL is allowed to recover the operation and maintenance expenses of existing stations for each financial year of the control period as per the norms specified in Annexure-VII of the Regulation. Further, annual pension contribution payable to Master Trust as well as expenses for the new generating stations is also allowable.  Accordingly, Operation & Maintenance Expenses Claimed and approved for SBU-G of KSEBL for the control period from 2018-19 to 2021-22 given in the table below.

| Table 7.23 : O&M cost for SBU-G |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Claimed in ARR |  |  |  |  |  |  |  |
| No | Item | 2018-19 | 2019-20 | 2020-21 |  | 2021-22 | Remarks |
| 1 | Existing Stations | 123.77 | 129.77 | 136.05 |  | 142.63 |  |
| 2 | New Stations | 4.01 | 22.67 | 40.54 |  | 69.14 |  |
| 3 | Total | 127.78 | 152.44 | 176.59 |  | 211.77 |  |
| Approved vide order dated 8-7-2019 |  |  |  |  |  |  |  |
| No | Item | 2018-19 | 2019-20 |  | 2020-21 | 2021-22 | Remarks |
| 1 | Existing Stations | 123.77 | 129.77 |  | 136.05 | 142.63 |  |
| 2 | New Stations | 0.38 | 5.27 |  | 17.32 | 29.38 |  |
| 3 | Total | 124.15 | 135.04 |  | 153.37 | 172.01 |  |

7.8.1.2. KSEB Ltd. has estimated the expense strictly in accordance with the Tariff Regulation , 2018, but Hon’ble Commission vide MYT order dated 08.07.2019 has adopted CERC norms in the absence of State Regulations in the case of Renewable energy projects. Now, as the KSERC (Renewable Energy and Net Metering) Regulations, 2020 has been notified and is in force from 07.02.2020 onwards, the same is taken for calculating the O&M Expense of the renewable energy stations. As per the above regulations, the O&M expense allowed for the SHEPs, wind and solar projects are shown below.

| Table 7.24 O&M Expense of new Renewable Sources |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sl No | Projects |  | O&M Expense 2019-20 (Cr)/MW | Escalation rate over previous year |
| 1 | Wind |  | 0.08 | 5.72% |
| 2 | SHEP | Below 5MW | 0.3241 | 5.72% |
|  |  | 5MW to 25MW | 0.2347 | 5.72% |
| 3 | Solar |  | 0.06 | 5.72% |

7.8.1.3. The list of new generating stations and units expected to be commissioned during the control period and their O&M Cost calculation are given in the Table below. The O&M has been calculated for new projects (including solar projects) based on the tariff regulation and also based on the KSERC (Renewable energy and Net Metering) Regulations, 2020. An escalation of 5.72% over the previous year is taken as envisaged in the KSERC (Renewable energy and Net Metering) Regulations, 2020.

| Table 7.25 : O&M expense for new Generating stations (Rs Cr) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New Stations | Installed Capacity (MW) | Project cost | Expected completion | O&M Exp Cr/MW | 2019-20 | 2020-21 | 2021-22 |
| Kakkayam HEP | 3 | 43.2 | 2018-19 | 0.3241 | 0.9723 | 1.027916 | 1.086712 |
| Boothathankettu SHEP | 24 | 231.21 | 2020-21 | 0.2347 |  | 5.954996 | 6.295622 |
| Uppar Kallar SHEP | 2 | 28.75 | 2020-21 | 0.3241 |  | 0.685277 | 0.724475 |
| Porigalkuthu AES | 24 | 171.42 | 2020-21 | 0.2347 |  | 5.954996 | 6.295622 |
| Chathankottunada SHEP | 6 | 79 | 2021-22 | 0.2347 |  |  | 1.573905 |
| Pazhassisagar SHEP | 7.5 | 104.37 | 2021-22 | 0.2347 |  |  | 1.967382 |
| Thottiyar HES | 40 | 283.49 | 2021-22 | 4%of PC |  |  | 11.3396 |
| Shengulam Aug. Scheme | - | 113 | 2021-22 |  |  |  |  |
| Peruvanamuzhi SHEP | 6 | 84.11 | 2021-22 | 0.2347 |  |  | 1.573905 |
| Pallivasal Extn | 60 | 550 | 2021-22 | 4%of PC |  |  | 22 |
| Solar 2018-19 | 2.143 | 14.999 | 2018-19 | 0.06 | 0.12858 | 0.135935 | 0.14371 |
| IPDS South RT Projects | 0.445 | 3.047 | 2019-20 | 0.06 | 0.02002 | 0.028227 | 0.029842 |
| Kottiyam Solar | 0.6 | 3.75 | 2019-20 | 0.06 | 0.003 | 0.038059 | 0.040236 |
| Kanjikode Solar | 3 | 17.1 | 2020-21 | 0.06 |  | 0.190296 | 0.201181 |
| Agali Solar | 1 | 5.7 | 2020-21 | 0.06 |  | 0.063432 | 0.06706 |
| Brahmapuram Solar | 4 | 22.8 | 2020-21 | 0.06 |  | 0.253728 | 0.268241 |
| Soura Phase 1 | 50 | 250 | 2020-21 | 0.06 |  | 3.1716 | 3.353016 |
| Soura Phase 1 | 50 | 125 | 2021-22 | 0.06 |  |  | 3.353016 |
| Total O&M Cost |  | -- | -- |  | 1.123905 | 17.50446 | 60.31353 |

7.8.1.4. Actuals up to 30.09.2019:

Though the approved O&M cost for the FY 2019- 20 is Rs.135 Cr. the actual O&M expenditure up to 30.09.2019 was Rs.134.83 Cr.  It is expected that, based on past actuals, the expense will become Rs.205.15 Cr.

      - Revised projection for 2019-20:

Revised O&M projection for 2019-20 is 130.89 Cr against approved figure of Rs.135.04 Cr. on normative basis.  Reduction in O& M cost is due to re-scheduling of new and ongoing hydel/solar projects. It may please be noted that the projected figure based on past actual is Rs.205.15 Cr.

      - Revised projection for 2020-21 :

Revised O&M projection for 2020-21 is 153.55Cr against approved figure of Rs.153.37 Cr. on normative basis.  The difference in O& M cost is due to re-scheduling of new and ongoing hydel/solar projects. It may please be noted that the projected figure based on past actual is Rs.221.72 Cr.

      - Revised projection for 2021-22 :

Revised O&M projection for 2021-22 is 202.99 Cr against approved figure of Rs.172.01 Cr. on normative basis.  Increase in O& M cost is due to re-scheduling of new and ongoing hydel/solar projects and to the addition of new projects within the control period. It may please be noted that the projected figure based on past actual is Rs.239.31 Cr.

      - Thus, the revised O&M cost as per norms for the control period from 2019-20 to 2021-22 for SBU-G is submitted below:

| Table 7.26 Revised O&M claim (Rs Cr) |  |  |  |
| --- | --- | --- | --- |
| Item | 2019-20 | 2020-21 | 2021-22 |
| Existing Stations | 129.77 | 136.05 | 142.63 |
| New Stations | 1.12 | 17.50 | 60.31 |
| Total | 130.89 | 153.55 | 202.99 |

7.8.1.9. Pay Revision Expenses

The Regulation 14 (3) provides for admission of expenses relating to pay revision during the control period for the same level of employees as admitted in the truing up of accounts for the year 2016-17. The pay revision of Officers/workmen of KSEBL is due from July / August 2018. The estimation of liability on this count and SBU wise breakup of the anticipated expenses are explained in Chapter 11.  Expenses related to SBU-G for the control period are summarized in the table below:

| Table 7.28 : Pay revision expenses for SBU G (Rs. Cr) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Business Unit | Employee cost ratio | 2018-19 TU 19 | 2019-20 | 2020-21 | 2021-22 |
| SBU G | 5.13 | 9.03 | 13.49 | 14.58 | 15.73 |

7.8.1.10. It is respectfully submitted that SBU-G is not including the projection against pay revision expenses of SBU-G for the years 2019-20 onwards. The Hon Commission may allow the said expense as part of truing up process, of respective years.  Thus, the total Operation & Maintenance expenses of SBU-G projected for the control period as per the Regulation is summarized below:

| Table 7.29 : Normative O&M cost (Rs Cr) |  |  |  |
| --- | --- | --- | --- |
| Item | 2019-20 | 2020-21 | 2021-22 |
| Normative O&M cost | 130.89 | 153.55 | 202.99 |
| Pay revision expenses | -- | -- | -- |
| Total O&M Cost | 130.89 | 153.55 | 202.99 |

7.8.1.11. Hon’ble Commission may kindly note that actual O&M expenses for 2018-19 has been Rs.197.32 Cr and the same for the half year ended 30.09.2019 amounts to Rs.134.83 Cr. The projection of the O&M expenses of SBU-G for the remaining years of the control period based on the past actuals are higher than the normative claim as shown in the table below.

| Table 7.30 : Projection of O&M of SBU (Rs Cr) |  |  |  |
| --- | --- | --- | --- |
| Item | 2019-20 | 2020-21 | 2021-22 |
| O&M Cost as per estimates | 205.15 | 221.72 | 239.31 |
| O&M cost as per norms | 130.89 | 153.55 | 202.99 |

7.8.1.12. It may kindly be noted that, the O&M expenses as per norms for the control period are below the actual expense projection. This will lead to under recovery of genuine O&M costs. However, KSEBL is taking earnest efforts to control the O&M expenses to the extent possible. If any variation is found beyond its control then it is humbly requested that the same may be allowed at actual during true up.

    - Depreciation

7.8.2.1. The depreciation claimed by KSEBL for the SBU G in line with Regulation 27 of the Tariff Regulations, 2018 and that approved by the Hon’ble Commission is summarized in Table below.

| Table 7.31 Depreciation for control period (Rs Cr) |  |  |
| --- | --- | --- |
| Year | As per MYT petition | As per MYT order |
| 2018-19 | 138.50 | 130.19. |
| 2019-20 | 159.32 | 141.38 |
| 2020-21 | 212.80 | 174.35 |
| 2021-22 | 235.78 | 196.26 |

7.8.2.2. As part of changes noticed in GFA, revised estimation of depreciation has been made for the control period, which is summarized below:

| Table 7.32: Computation of Normative depreciation for the remaining years of control period (Rs. Cr) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sl No | Particulars | TU | Revised estimate |  |  |
|  |  | 2018-19 | 2019-20 | 2020-21 | 2021-22 |
| 1 | GFA excluding revaluation | 4857.26 | 4904.21 | 4935.427 | 5724.787 |
| 2 | Contribution & grants | 75.31 | 88.66 | 88.66 | 135.66 |
| 3 | GFA net of contribution (1-2) | 4781.95 | 4815.55 | 4846.767 | 5589.127 |
| 4 | Addition during the year | 46.95 | 31.22 | 789.36 | 1213.97 |
| 5 | Contribution & grants | 13.35 | 0.00 | 47.00 | 44.50 |
| 6 | GFA addition net of contribution (4-5) | 33.60 | 31.217 | 742.36 | 1169.47 |
| 7 | Depreciation rate (%) on opening GFA (Actual for 2018-19) |  | 2.86 | 2.86 | 2.86 |
| 8 | Depreciation rate (%) for GFA addition at half the normal rate of 5.14%. |  | 2.57 | 2.57 | 2.57 |
| 9 | Depreciation for Op GFA (3x7) | 136.73 | 137.72 | 138.62 | 159.849 |
| 10 | Depreciation on current year addition (6X8) | 0.86 | 0.80 | 19.08 | 30.06 |
| 11 | Total depreciation for the year (9+10) | 137.60 | 138.53 | 157.70 | 189.90 |

    - Interest and Finance Charges

7.8.3.1. Interest on Capital Liabilities:

| Table7.33 Interest on capital liabilities (Rs Cr ) for SBU-G |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Item | Claimed in ARR 2019-20 | Approved for FY 2019-20 | 2018-19 TU | Actual upto (30.09. 2019) | Revised estimate 2019-20 | Appd for the FY 2020-21 | Revised estimate 2020-21 | Appd for the FY 2021-22 | Revised estimate 2021-22 |
| 1 | Opening Normative loan | 1104.44 | 730.2 | 889.69 | 846.98 | 785.70 | 1036.89 | 678.39 | 1456.13 | 1263.06 |
| 2 | GFA addition during the year | 457.23 | 448.07 | 46.95 | --- | 31.22 | 593.59 | 789.36 | 153.76 | 1213.97 |
| 3 | Less: Consumer contribution & Grants |  |  | 13.34 |  | 0.00 |  | 47.00 |  | 44.50 |
| 4 | Less: Allowable depreciation/repayment in case of actual | 125.01 | 141.38 | 137.60 | 68.1 | 138.53 | 174.35 | 157.70 | 196.26 | 189.90 |
| 5 | Normative loan during the year | 332.22 | 306.69 | 33.61 | -68.1 | 31.22 | 419.24 | 742.36 | -42.5 | 1169.47 |
| 6 | Closing normative loan | 1436.65 | 1036.89 | 785.70 | 778.88 | 678.39 | 1456.13 | 1263.06 | 1413.64 | 2242.62 |
| 7 | Average normative loan | 1270.54 | 883.55 | 837.69 | 812.93 | 732.05 | 1246.51 | 970.72 | 1434.88 | 1752.84 |
| 8 | Interest for the year | 127.05 | 82.45 | 69.88 | 43.78 | 61.05 | 116.32 | 80.96 | 133.90 | 146.19 |
| 9 | Interest rate (%) | 10 | 9.33 | 8.34 | 10.78 | 8.34 | 9.33 | 8.34 | 9.33 | 8.34 |

The Interest on Capital liabilities actually incurred as on 30.09.2019 is Rs 43.78 Cr. The revised Interest on Capital liabilities based on the revised GFA for the year 2019-20 and for the subsequent years in the Control period based on the GFA proposed is shown below.  Detailed explanation is furnished in     Chapter 11.

Note: Closing normative loan as per TU 2019 has been considered as opening normative loan for 2019-20. Actual interest for 2018-19 has been considered for estimating interest for the remaining years of control period.

7.8.3.2. Interest on Security Deposits:

SBU-G does not hold any security deposit and hence no amount is considered on this account.

7.8.3.3 Interest on GPF

The revised interest on GPF is computed in line with the methodology adopted by the Hon’ble Commission in the MYT order dated 08.07.2019 as detailed in Chapter 11. Accordingly, the figures arrived at for the rest of the years of the control period are furnished below:

| Table 7.34: Comparison of interest on GPF (Rs Cr) |  |  |  |
| --- | --- | --- | --- |
|  | As per MYT Petition | Approved | Revised Projections |
| Employee cost ratio (2017-18) | 5.13 | 5.13 | 5.13 |
| 2018-19-TU | 9.83 | 9.37 | 8.92 |
| 2019-20 | 10.48 | 9.98 | 9.37 |
| 2020-21 | 11.12 | 10.6 | 9.99 |
| 2021-22 | 11.77 | 11.21 | 10.60 |

7.8.3.4. Interest on Master Trust Bonds and additional contribution to Master Trust

Hon’ble Commission was pleased to approve the interest on Master Trust bonds along with additional contribution to Master Trust. No change is proposed in this petition as detailed below. Detailed explanation is furnished in Chapter 11.

| Table 7.35 Interest on Master Trust bonds of SBU-G (Rs Cr) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No | Item | Claimed for the year 2019-20 | Approved for FY 2019-20 | Revised estimate for 2019-20 | Revised estimate for 2020-21 | Revised estimate for 2021-22 |  |
| 1 | Existing bond | 37.58 | 37.58 | 37.58 | 35.50 | 33.41 |  |
| 2 | Additional contribution to Master Trust | 19.12 | 10.26 | 10.26 | 10.26 | 10.26 |  |
|  | Total | 56.70 | 47.84 | 47.84 | 47.84 | 47.84 |  |

7.8.3.5. Interest on Working Capital

The Interest on working capital for SBU-G is computed as per Regulation 32 on a normative basis. One month O&M cost, cost of maintenance spares at 1% historical cost and receivables for one month less security deposit held, if any.  The interest rate allowed, as per Regulation 32(2), is 2% higher than the Base rate on First of April of the ARR filing Year.  Therefore the Interest on Working Capital in this petition is computed at a rate of 10.95% (8.95% as on 1-4-2019 + 2%). The parameters adopted for computation of Interest on Working capital for the year 2019-20 is given below:

| Table 7.36 Parameters for Working Capital for 2019-20 (Rs Cr) |  |
| --- | --- |
| Item | 2019-20 |
| Opening GFA (Rs.Cr.) | 4904.21 |
| O&M Cost (Rs.Cr.) | 130.89 |

7.8.3.6 The Interest on Working Capital claimed in ARR for the year 2019-20, approved by Hon’ble Commission, actual for the period up to September 2019 and the revised estimate for the year 2019-20 is given below.

| Table 7.37 Interest on Working capital of SBU-G (Rs Cr) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Claimed in ARR for the year 2019-20 | Approved for FY 2019-20 vide order dtd. 8-7-2019 | Actual up to Sept-2019 | Revised for the year 2019-20 | Appd for the FY 2020-21 | Revised for the year 2020-21 | Appd for the FY 2021-22 | Revised for the year 2021-22 |
| No | Item |  |  |  |  |  |  |  |  |
| 1 | O&M Expenses (as per norms) | 12.70 | 11.25 | 22.78 | 10.91 | 12.78 | 12.80 | 14.33 | 16.92 |
| 2 | Maintenance Spares (as per norms) | 45.13 | 50.17 | 49.47 | 49.04 | 54.65 | 49.35 | 60.58 | 57.25 |
| 3 | Receivables (as per norms) | 0 | 0 | 0.84 | 0.84 | 0 | 0 | 0 | 0 |
| 4 | Less Security Deposits | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | Total working capital | 57.83 | 61.42 | 73.10 | 60.79 | 67.43 | 62.15 | 74.92 | 74.17 |
| 6 | Interest Rate (as per norms) | 10.70% | 10.15% | 10.15% | 10.15% | 10.15% | 10.15% | 10.15% | 10.15% |
| 7 | Interest on working capital | 6.19 | 6.23 | 7.42 | 6.17 | 6.84 | 6.31 | 7.60 | 7.53 |

7.8.3.7. The summary of interest and finance charges estimated for SBU-G for the control period is submitted below:

| Table 7.38 Summary of Interest & Finance Charges of SBU-G (Rs Cr) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Claimed in ARR for the year 2019-20 | Approved for FY 2019-20 vide order dtd. 8-7-2019 | Actual up to Sept-2019 | Revised for the year 2019-20 | Appd for the FY 2020-21 | Revised for the year 2020-21 | Appd for the FY 2021-22 | Revised for the year 2021-22 |
| No | Item |  |  |  |  |  |  |  |  |
| 1 | Interest on Capital Liabilities | 127.05 | 82.45 | 43.78 | 61.05 | 116.32 | 80.96 | 133.90 | 146.19 |
| 2 | Interest on GPF | 10.48 | 9.98 | 3.84 | 9.37 | 10.60 | 9.99 | 11.21 | 10.60 |
| 3 | Interest on Master Trust | 56.7 | 47.84 | 16.53 | 37.58 | 45.76 | 35.50 | 43.67 | 33.41 |
| 4 | Interest on Working Capital | 6.19 | 6.23 | 7.42 | 6.17 | 6.84 | 6.31 | 7.60 | 7.53 |
| 5 | Total Interest & Finance Charges | 200.42 | 146.50 | 68.51 | 114.18 | 179.53 | 132.75 | 196.39 | 197.73 |
| 6 | Less: Capitalized |  |  | 21.02 |  |  |  |  |  |
| 7 | Net I&FC |  |  | 47.49 |  |  |  |  |  |

    - Return on Equity

7.8.4.1. The ROE for SBU-G was proposed by KSEBL and approved by the Hon’ble Commission at Rs.116.38 Cr. No change is proposed in this filing as submitted below.

| Table 7.39 ROE OF SBU-G Return on Equity (Rs.Cr.) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Claimed in ARR for the year 2019-20 | Approved vide order dtd.8-7-2019 | Actual upto 30-9-2019 | Revised estimate for 2019-20 | Revised estimate for 2020-21 | Revised estimate for 2021-22 |
| No | Item |  |  |  |  |  |  |
| 1 | Return on Equity | 116.38 | 116.38 | 58.19 | 116.38 | 116.38 | 116.38 |

  - Aggregate Revenue Requirement

7.9.1 The SBU-G handles the generation assets of KSEB Ltd and manages bulk generation of power and supply to SBU-D. The revenue requirement of SBU-G is the transfer cost of internal generation to SBU-D.   The total Aggregate Revenue Requirement of SBU-G claimed in the original petition, approved, actual upto Sept- 2019 and revised estimate for the future years of the control period is submitted below:

    - Actuals up to 30.09.2019

Hon’ble Commission has approved aggregate revenue requirement of SBU-G for 2019-20 as 539.31 Cr. But, due to variation in actual O&M cost up to Sep 2019, the actuals up to Sept 2019 became 327.01 Cr. (about 61% of the approved value).

    - Revised estimate for 2019-20

Though the actual expenses are on the higher side during the 1st half of the year, a projection based on the norms specified in the regulation is taken for revised estimate for 2019-20 and it is estimated as Rs. 515.69 Cr. Reduction in ARR by Rs.23.62 Cr from the approved ARR of Rs.539.31 Cr is due to rescheduling of certain projects proposed in the MYT petition to ensuing financial years.

    - Revised estimate for 2020-21

Hon’ble Commission has approved aggregate revenue requirement of SBU-G for 2020-21 as 623.62 Cr.  But the revised ARR projection is Rs.570.64 Cr. Reduction in ARR by Rs.52.98 Cr is due to lower normative interest.

    - Revised estimate for 2021-22

Revised estimate for 2021-22 exceeded approval by Rs.36.23 Cr (Rs. 717.26 Cr against approval of Rs.681.03 Cr) mainly because of increase under O&M expenses.  A summary of revised estimate for the remaining years of the control period is furnished below:

| Table 7.40 Revised ARR of SBU-G (Rs.Cr) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Claimed for the year 2019-20 | Appvd. For FY 2019-20 | Actual up to Sept 2019 | Revised estimate for 2019-20 | Appd for the FY 2020-21 | Revised estimate for 2020-21 | Appd for the FY 2021-22 | Revised estimate for 2021-22 |
| No | Item |  |  |  |  |  |  |  |  |
| 1 | Generation of power |  |  | 5.46 | 5.46 |  |  |  |  |
| 2 | Operation & Maintenance Expenses | 152.44 | 135.04 | 134.83 | 130.89 | 153.37 | 153.55 | 172.01 | 202.99 |
| 3 | Interest on long term loans | 127.05 | 82.45 | 22.76 | 61.05 | 116.32 | 80.96 | 133.90 | 146.19 |
| 4 | Interest on Master Trust Bonds | 56.7 | 37.58 | 16.53 | 37.58 | 35.50 | 35.50 | 33.41 | 33.41 |
| 5 | GPF Interest | 10.48 | 9.98 | 3.84 | 9.37 | 10.60 | 9.99 | 11.21 | 10.60 |
| 6 | Depreciation | 125.01 | 141.38 | 68.16 | 138.53 | 174.35 | 157.70 | 196.26 | 189.90 |
| 7 | Interest on WC and deposits | 6.19 | 6.23 | 4.36 | 6.17 | 6.84 | 6.31 | 7.60 | 7.53 |
| 8 | Additional contribution to Master Trust | 0 | 10.26 |  | 10.26 | 10.26 | 10.26 | 10.26 | 10.26 |
| 9 | Other items |  |  | 2.02 |  |  |  |  |  |
| 10 | Total Revenue Expenditure (1 to 9) | 477.87 | 422.93 | 257.96 | 399.31 | 507.24 | 454.26 | 564.65 | 600.88 |
| 11 | Return on Equity | 116.38 | 116.38 | 69.05 | 116.38 | 116.38 | 116.38 | 116.38 | 116.38 |
| 12 | Tax on RoE | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 13 | Aggregate Revenue Requirement (11+12+13) | 594.25 | 539.31 | 327.01 | 515.69 | 623.62 | 570.64 | 681.03 | 717.26 |

  - Non-Tariff Income

The non-tariff income of SBU-G was fully approved by the Hon’ble Commission as projected by KSEBL. Actual income till 30.09.2019 has been Rs. 4.63 Cr. However, revision is not proposed under this head for the remaining control period as submitted below:

| Table 7.41 : Non-Tariff Income for SBU-G (Rs Cr) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Claimed in ARR for the year 2019-20 | Appvd. For FY 2019-20 | Actual upto Sept 2019 | Revised estimate for 2019-20 | Revised estimate for 2020-21 | Revised estimate for 2021-22 |  |
|  | Non Tariff income | 33.38 | 33.38 | 16.69 | 33.38 | 37.28 | 41.17 |  |

  - Net ARR of SBU-G

The net ARR of SBU-G claimed in the original petition, approved, actual up to Sept- 2019 and revised estimate for the future years of the control period is submitted below:

| Table 7.42 Net ARR for SBU-G |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Item |  |  |  |  |  |  |  |  |
|  |  | Claimed in ARR for 2019-20 | Appd for the FY 2019-20 | Actual up to Sept 2019 | Revised estimate for 2019-20 | Appd for the FY 2020-21 | Revised estimate for 2020-21 | Appd for the FY 2021-22 | Revised estimate for 2021-22 |
| 1 | Aggregate Revenue Requirement | 594.25 | 539.31 | 327.01 | 515.69 | 623.62 | 570.64 | 681.03 | 717.26 |
| 2 | Less: Other Income | 33.38 | 33.38 | 4.63 | 33.38 | 37.28 | 37.28 | 41.17 | 41.17 |
| 3 | Net ARR | 560.87 | 505.93 | 322.37 | 482.31 | 586.34 | 533.36 | 639.86 | 676.09 |

* * *

| Annexure G1 - Hydel Power Projects of KSEBL |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No | Name of Station | Installed Capacity |  |  | Date of Commissioning |  |
|  |  | No of Machines | Plant Capacity | Design Energy | Original Date | After Renovation |
| I | Major HEP |  |  |  |  |  |
| 1 | Idukki | 6 x 130 | 780 | 2398 | 22.03.1986 |  |
| 2 | Sabarigiri | 4 x 55 + 2x 60 | 340 | 1338 | 26.11.0967 | 06.05.2014 |
| 3 | Idamalayar | 2 x 37.5 | 75 | 380 | 03.02.1987 |  |
| 4 | Sholayar | 3x18 | 54 | 233 | 14.05.1968 |  |
| 5 | Pallivasal | 3 x 5+ 3x7.5 | 37.5 | 284 | 07.03.1951 | 26.08.2002 |
| 6 | Kuttiyadi | 3x25 | 75 | 268 | 28.11.1972 |  |
| 7 | Kuttiyadi Extension | 1x50 | 50 | 75 | 27.01.2001 |  |
| 8 | Kuttiyadi Add Ext | 2x50 | 100 | 223 | 11.05.1963 |  |
| 9 | Panniar | 2 x 16.2 | 32.4 | 158 | 26.01.1964 | 01.02.2003 |
| 10 | Neriamangalam | 3 x17.55 | 52.65 | 237 | 11.05.1963 | 31.11.2005 |
| 11 | NES | 1x25 | 25 | 58.27 | 27.05.2008 |  |
| 12 | Lower Periyar | 3 x 60 | 180 | 493 | 28.11.1997 |  |
| 13 | Poringalkuthu | 4x9 | 36 | 191 | 06.02.1960 | 29.05.2015 |
| 14 | Sengulam | 4 x 12.8 | 51.2 | 182 | 05.01.1955 | 30.11.2002 |
| 15 | Kakkad | 2x25 | 50 | 262 | 13.10.1999 |  |
|  | Sub Total (HEP) | 49 Nos | 1938.75 | 6780.27 |  |  |
| II | Small HEPs |  |  |  |  |  |
| 1 | Kallada | 2x7.5 | 15 | 65 | 05.09.1994 |  |
| 2 | Peppara | 1x3 | 3 | 11.5 | 15.06.1996 |  |
| 3 | Malankara | 3x3.5 | 10.5 | 44 | 23.10.2005 |  |
| 4 | Madupatty | 1x2 | 2 | 6.4 | 14.01.1998 |  |
| 5 | Malampuzha | 1x2.5 | 2.5 | 5.6 | 26.11.2001 |  |
| 6 | Lower Meenmutty | (1x0.5+ 2x1.5) | 3.5 | 7.63 | 25.03.2006 |  |
| 7 | Chembukadavu - 1 | 3x0.9 | 2.7 | 6.59 | 19.08.2003 |  |
| 8 | Chembukadavu - 2 | 3x1.25 | 3.75 | 9.03 | 04.09.2003 |  |
| 9 | Urumi -1 | 3x1.25 | 3.75 | 9.72 | 25.01.2004 |  |
| 10 | Urumi -2 | 3x0.8 | 2.4 | 6.28 | 25.01.2004 |  |
| 11 | KTR | 3x1.25 | 3.75 | 15 | 23.10.2009 |  |
| 12 | Poozhithode | 3 x 1.6 | 4.8 | 10.97 | 25.06.2011 |  |
| 13 | Ranni-Perinadu | 2x2 | 4 | 16.73 | 16.02.2012 |  |
| 14 | Peechi | 1x1.25 | 1.25 | 3.31 | 07.01.2013 |  |
| 15 | Vilangad | 3x2.5 | 7.5 | 22.63 | 19.01.2014 |  |
| 16 | Chimmony | 1x2.5 | 2.5 | 6.7 | 22.05.2015 |  |
| 17 | Adyanpara | 2x1.5 +0.5 | 3.5 | 9.01 | 09.03.2015 |  |
| 18 | Barapole | 3 x 5 | 15 | 36 | 29.02.2016 |  |
| 20 | PLBE | 1x16 | 16 | 74 | 20.03.1999 |  |
| 21 | Vellathooval | 2 x 1.8 | 3.6 | 12.7 | 08.09.2016 |  |
| 22 | Peruthenaruvi | 2 x3 | 6 | 25.77 | 24.10.2017 |  |
| 23 | Kakkayam | 2x 1.5 | 3 | 10.39 | 16.07.2018 |  |
|  | Sub Total (SHEP) | 24Nos | 120 | 414.96 |  |  |
|  | Total (Hydel) | 83 Nos | 2058.75 | 7195.23 |  |  |

Annexure G.2 Solar Projects commissioned and On-going

| Projects Commissioned during 2018-19 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Name of the stations | Installed Capacity | Annual Generation (MU) | Date of Commissioning | Project Cost(Cr) |
| 1 | Roof Top solar projects Distribution wing (12 Nos) | 0.233 |  | 02.02.2018 | 3.82 |
| 2 | Peerumedu (Idk) GM SPV | 0.50 |  | 23.04.2018 |  |
| 3 | Roof Top solar projects Transmission wing (27 Nos) | 0.91 |  | 09.07.2018 | 7.64 |
| 4 | Ponnani G. Mounted SPV | 0.50 | 0.77 | 16.01.2019 | 3.539 |
|  |  | 2.143 |  |  | 14.999 |

| Projects Commissioned /scheduled during 2019-20 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Name of the stations | Installed Capacity | Annual Generation (MU) | Date of Commissioning | Project Cost(Cr) |
| 1 | Roof top solar projects at Various locations | 0.445 | 0.526 | 30.06.2019 | 3.047 |
| 2 | Kottiyam | 0.6 | 0.84 |  | 3.75 |

| Projects scheduled 2020-21 |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Name of the stations | Installed Capacity | Annual Generation (MU) | Project Cost(Cr) |
| 1 | Brahmapuram | 4.0 | 4.9 | 22.8 |
| 2 | Kanjikode | 3.0 | 3.679 | 17.1 |
| 3 | Agali | 1.0 | 1.2 | 5.70 |
|  | Total | 8.0 | 9.779 | 45.60 |

| Annexure G3 INFLOW DETAILS FROM 1994-95 ONWARDS (WATER YEAR) (Annexure I) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl No. | Year | June | Jul | Aug | Sep | Oct | Nov | Dec | Jan | Feb | Mar | Apr | May | Total |
| 1 | 1998-99 | 678.7 | 1 476.49 | 2357.6 | 1036.91 | 1243.8 | 694.7 | 457.53 | 157.27 | 118 | 94.11 | 96.03 | 421.89 | 8833.03 |
| 2 | 1999-00 | 1046.46 | 1416.49 | 973.6 | 441.59 | 1198.43 | 451.57 | 197.19 | 130.38 | 148 | 82.9 | 104 | 98.6 | 6289.21 |
| 3 | 2000-01 | 835.07 | 946.1 | 2199.13 | 789.14 | 502.59 | 227.58 | 198.97 | 153.05 | 80.74 | 47.5 | 162.21 | 127.2 | 6269.28 |
| 4 | 2001-02 | 1062.21 | 1765.9 | 1079.95 | 501.1 | 865.14 | 712.89 | 231.18 | 116.03 | 86.99 | 81.26 | 71.06 | 161.75 | 6735.46 |
| 5 | 2002-03 | 489.76 | 750.9 | 1255.86 | 322.19 | 585.44 | 391.14 | 134.13 | 78.51 | 63.04 | 63.21 | 52.63 | 81.22 | 4268.03 |
| 6 | 2003-04 | 304.74 | 748.03 | 916.01 | 376.53 | 983.43 | 325.98 | 160 | 91.92 | 78.81 | 39.28 | 58.56 | 426.2 | 4509.49 |
| 7 | 2004-05 | 1401.61 | 986.94 | 1436.86 | 510.74 | 716.44 | 464.39 | 208.73 | 155.27 | 70.42 | 72.18 | 109.3 | 99.29 | 6232.17 |
| 8 | 2005-06 | 569.38 | 2261.04 | 1252.98 | 1475.76 | 715.88 | 714.76 | 442.33 | 217.85 | 110.07 | 170.31 | 154.37 | 434.29 | 8519.02 |
| 9 | 2006-07 | 725.23 | 1596.83 | 1117.81 | 963.41 | 1009.56 | 907.56 | 253.21 | 164.44 | 90.93 | 111.6 | 123.39 | 113.14 | 7177.11 |
| 10 | 2007-08 | 1002.94 | 2797.48 | 1499.21 | 1700.29 | 1095.25 | 596.39 | 283.09 | 140.5244 | 126.85 | 214.1 | 180.18 | 132.23 | 9768.534 |
| 11 | 2008-09 | 433.63 | 1144.88 | 1358.87 | 962.98 | 629.358 | 297.351 | 169.3 | 69.864 | 44.425 | 78.414 | 65.672 | 115.774 | 5370.518 |
| 12 | 2009-10 | 330.869 | 2023.7 | 779.469 | 1147.562 | 898.7 | 642.91 | 242.242 | 128.347 | 76.47 | 81.11 | 102.159 | 128.296 | 6581.834 |
| 13 | 2010-11 | 800.02 | 1407.075 | 1175.575 | 818.336 | 921.176 | 651.47 | 364.2 | 167.411 | 122.624 | 103.11 | 208.129 | 136.99 | 6876.116 |
| 14 | 2011-12 | 1236.187 | 1318.659 | 1654.185 | 1192.892 | 592.55 | 587.895 | 262.645 | 127.97 | 88.15 | 90.26 | 132.78 | 94.84 | 7379.013 |
| 15 | 2012-13 | 340.07 | 726.86 | 893.79 | 744.06 | 435.67 | 245.57 | 102.811 | 53.308 | 35.017 | 76.89 | 75.527 | 65.056 | 3794.629 |
| 16 | 2013-14 | 1642.94 | 2392.97 | 1895.437 | 1164.5 | 634.76 | 441.3921 | 213.84184 | 120.2269 | 68.19692 | 96.71712 | 88.4 | 180.09 | 8939.472 |
| 17 | 2014-15 | 583.25 | 1678.1 | 1798.89 | 1034.02 | 843.43 | 540.03 | 273.58 | 143.588 | 79.231 | 113.477 | 241.44 | 255.403 | 7584.439 |
| 18 | 2015-16 | 1053.950658 | 1106.8266 | 829.56 | 614.38 | 619.18 | 605.9906 | 420.6586 | 148.6973 | 92.3638 | 86.8715 | 90.83459 | 107.889449 | 5777.203 |
| 19 | 2016-17 | 582.8726738 | 1105.0913 | 748.03516 | 393.7508 | 267.5849 | 152.9335 | 81.911161 | 57.87296 | 40.62692 | 94.89989 | 70.59566 | 107.458398 | 3703.633 |
| 20 | 2017-18 | 569.72 | 633.92 | 1323.47 | 1269.17 | 706.25 | 487.89 | 326.86 | 110.03 | 68.37 | 113.65 | 122.90 | 242.448012 | 5974.675 |
| 21 | 2018-19G | 1569.08 | 3146.60 | 4471.27 | 668.14 | - | - | - | - | - | - | - | - |  |
|  | 2018-19N | 1460.22 | 2461.90 | 492.56 | 455.14 | 599.29 | 338.19 | 161.65 | 87.77 | 69.70 | 78.24 | 85.62 | 113.40 | 6403.69 |
| 22 | 2019-20 | 165.19 | 715.78 | 1938.03 | 1082.58 | 766.12 | 489.19 | 236.43 | 123.15 | 82.05 | 94.80 | 114.99 | 161.08 | 5969.39 |
| 23 | 2020-21 | 779.49 | 1428.45 | 1324.02 | 875.93 | 719.39 | 491.08 | 238.39 | 122.79 | 78.75 | 95.39 | 115.54 | 164.20 | 6433.423 |
| 24 | 2021-22 | 776.71 | 1452.57 | 1280.27 | 880.27 | 730.23 | 504.25 | 240.36 | 121.28 | 78.65 | 97.79 | 113.20 | 166.05 | 6441.63 |
| The inflow projected based on 20yrs average value. The Aug-2018 value has been replaced with 20yrs average value for the calculation of subsequent years of the control period.has been taken as 20yrs average for the future |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Annexure-G4 Details of Generation Restored (Flood 2019) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | Name of Station | Installed Capacity of the plant |  | Generation Restored in (MW) | Unit # | Date of restoration |
|  |  | No. of Generators. X Capacity in (MW) | Plant Capacity in (MW) |  |  |  |
| HEPs |  |  |  |  |  |  |
| 1 | Idamalayar | 2 x 37.5 | 75 | 75 |  | 19.08.2018 |
| 2 | Pallivasal | 3 x 5+ 3x7.5 | 37.5 | 37.5 |  | 22.08.2018 |
| 3 | Lower Periyar | 3 x 60 | 180 | 180 |  | 23.09.2018 |
| 4 | Poringalkuthu | 4x9 | 36 | 36 |  | 26.12.2018 |
| 5 | RPanniar | 2 x 16.2 | 32.4 | 32.4 | I | 14.01.2019 |
|  |  |  |  |  | II | 23.11.2018 |
| Small HEPs |  |  |  |  |  |  |
| 6 | PLBE | 1x16 | 16 | 16 |  | 27.08.2018 |
| 7 | Malampuzha | 1x2.5 | 2.5 | 2.5 |  | 17.08.2018 |
| 8 | Urumi -2 | 3x0.8 | 2.4 | 2.4 |  | 06.09.2018 |
| 9 | Poozhithode | 3 x 1.6 | 4.8 | 4.8 |  | 01.09.2018 |
| 10 | Peechi | 1x1.25 | 1.25 | 1.25 |  | 18.08.2018 |
| 11 | Vilangad | 3x2.5 | 7.5 | 7.5 |  | 25.08.2018 |
| 12 | Chimmony | 1x2.5 | 2.5 | 2.5 |  | 24.08.2018 |
| 13 | Adyanpara | 2x1.5+0.5 | 3.5 | 3.5 | I | 10.01.2019 |
|  |  |  |  |  | II | 17.01.2019 |
|  |  |  |  |  | III | 19.12.2018 |
| 14 | Madupatty | 1x2 | 2 | 2 |  | 22.02.2019 |
| 15 | Chembukadavu - 2 | 3x1.25 | 3.75 |  |  | 19.06.22019 |
| 16 | Ranni-Perinadu | 2x2 | 4 | 2 | I | 23.01.2019 |
|  |  |  |  |  | II | 29.03.2019 |
| 17 | Barapole | 3x5 | 15 |  |  | 18-06-2019 |
| 18 | Vellathooval | 2x1.8 | 3.6 |  |  |  |
| 19 | Perunthenaruvi | 2x3 | 6 | 6 | I | 13.01.2019 |
|  |  |  |  |  | II | 27.01.2019 |
|  |  |  | 435.7 | 411.35 |  |  |

| Annexure G5(1) : Generation 2019-20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gen. Stations | Apr'19 | May'19 | Jun'19 | Jul'19 | Aug'19 | Sep'19 | Oct'19 | Nov'19 | Dec'19 | Jan'20 | Feb'20 | Mar'20 | Total |
| Idukki | 276.70 | 358.39 | 204.04 | 99.60 | 73.98 | 99.03 | 93.60 | 114.81 | 153.74 | 183.00 | 262.77 | 392.18 | 2311.84 |
| Sabarigiri | 128.03 | 156.97 | 71.37 | 24.47 | 45.53 | 92.26 | 92.90 | 114.01 | 142.47 | 120.05 | 100.77 | 158.20 | 1247.05 |
| Idamalayar | 30.59 | 28.76 | 10.67 | 1.57 | 13.71 | 29.33 | 18.50 | 22.86 | 37.24 | 39.75 | 34.96 | 41.71 | 309.65 |
| Sholayar | 19.22 | 16.95 | 4.49 | 0.73 | 9.15 | 25.01 | 25.30 | 20.61 | 26.82 | 30.07 | 24.98 | 28.01 | 231.35 |
| Pallivasal | 12.73 | 10.91 | 10.41 | 10.48 | 11.43 | 12.61 | 12.87 | 12.70 | 12.12 | 10.88 | 11.70 | 14.65 | 143.49 |
| Kuttiady | 47.67 | 39.34 | 17.35 | 60.22 | 51.66 | 82.16 | 67.09 | 55.69 | 43.99 | 43.56 | 43.28 | 62.94 | 614.95 |
| Panniar | 13.29 | 7.71 | 2.67 | 1.52 | 17.40 | 21.72 | 22.36 | 18.80 | 15.00 | 12.26 | 9.35 | 12.70 | 154.77 |
| NMGM & NES | 19.50 | 16.18 | 11.13 | 20.15 | 46.59 | 52.27 | 48.64 | 38.30 | 25.77 | 15.36 | 15.12 | 18.92 | 327.93 |
| L.Periyar | 19.50 | 16.34 | 13.52 | 39.10 | 103.21 | 43.79 | 66.30 | 53.60 | 31.66 | 17.54 | 15.98 | 18.49 | 439.03 |
| Poringal | 1.89 | 2.04 | 0.13 | 15.05 | 20.58 | 19.64 | 20.50 | 16.00 | 2.48 | 2.59 | 3.62 | 3.42 | 107.94 |
| PLBE | 8.18 | 0.59 | 0.10 | 9.09 | 11.71 | 11.60 | 11.93 | 10.20 | 7.94 | 8.47 | 8.29 | 10.96 | 99.05 |
| Sengulam | 11.40 | 13.57 | 9.83 | 10.33 | 16.33 | 16.44 | 12.24 | 14.99 | 12.59 | 11.26 | 9.96 | 11.86 | 150.80 |
| Kakkad | 17.62 | 21.48 | 11.33 | 7.27 | 12.97 | 17.06 | 16.36 | 16.02 | 18.08 | 17.44 | 13.78 | 17.16 | 186.58 |
| Kallada | 5.39 | 4.00 | 2.72 | 0.76 | 1.04 | 0.61 | 0.68 | 5.23 | 6.15 | 2.61 | 3.19 | 5.95 | 38.33 |
| Peppara | 0.37 | 0.38 | 0.14 | 0.29 | 0.32 | 1.55 | 0.76 | 0.63 | 0.45 | 0.32 | 0.28 | 0.24 | 5.73 |
| Madupetty | 0.70 | 0.69 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.11 | 0.20 | 0.26 | 0.39 | 0.54 | 2.90 |
| Chembukadav | 0.00 | 0.00 | 0.28 | 2.53 | 0.90 | 1.60 | 1.38 | 0.27 | 0.04 | 0.00 | 0.00 | 0.00 | 7.00 |
| Malampuzha | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.31 | 0.42 | 0.41 | 0.78 | 0.61 | 0.12 | 0.00 | 2.66 |
| URUMI | 0.00 | 0.00 | 0.86 | 2.74 | 2.53 | 3.62 | 2.23 | 0.16 | 0.05 | 0.00 | 0.00 | 0.00 | 12.18 |
| Malankara | 3.03 | 3.92 | 2.93 | 2.53 | 1.96 | 2.03 | 1.62 | 2.26 | 1.86 | 2.00 | 2.08 | 2.73 | 28.94 |
| L. Meenmutty | 0.04 | 0.02 | 0.27 | 0.43 | 0.84 | 1.02 | 0.96 | 0.61 | 0.30 | 0.08 | 0.03 | 0.04 | 4.65 |
| Ktdy.tail race | 0.37 | 0.20 | 0.29 | 0.51 | 0.76 | 1.02 | 0.89 | 0.32 | 0.35 | 0.36 | 0.35 | 0.59 | 6.01 |
| Poozhithode | 0.00 | 0.00 | 0.62 | 2.28 | 0.60 | 1.49 | 1.02 | 0.26 | 0.10 | 0.04 | 0.00 | 0.00 | 6.42 |
| Ranni-Perunad | 0.60 | 0.75 | 0.71 | 0.46 | 0.69 | 0.82 | 0.69 | 0.58 | 0.29 | 0.13 | 0.13 | 0.49 | 6.34 |
| Peechi | 0.16 | 0.00 | 0.00 | 0.00 | 0.07 | 0.48 | 0.33 | 0.14 | 0.29 | 0.17 | 0.17 | 0.17 | 1.97 |
| Vilangad | 0.00 | 0.00 | 0.24 | 3.59 | 3.52 | 3.69 | 2.89 | 0.97 | 0.52 | 0.19 | 0.11 | 0.34 | 16.06 |
| Chimmini | 0.41 | 0.17 | 0.00 | 0.00 | 0.03 | 0.47 | 0.27 | 0.91 | 0.23 | 0.20 | 0.64 | 0.98 | 4.29 |
| Adiyanpara | 0.00 | 0.01 | 0.44 | 1.75 | 0.19 | 0.00 | 0.20 | 0.17 | 0.08 | 0.01 | 0.00 | 0.00 | 2.86 |
| Vellathuval | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.13 | 0.09 | 0.04 | 0.04 | 0.15 | 0.46 |
| Barapole | 0.00 | 0.00 | 0.06 | 1.68 | 1.98 | 6.65 | 3.25 | 3.36 | 2.72 | 1.68 | 1.81 | 5.77 | 28.97 |
| Perunthenaruvi | 0.03 | 0.03 | 1.04 | 1.27 | 1.98 | 2.74 | 1.10 | 1.51 | 0.75 | 0.11 | 0.14 | 0.02 | 10.73 |
| BDPP | 0.00 | 0.00 | 0.00 | 0.10 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.10 |
| KDPP | 5.38 | 6.39 | 0.00 | 0.00 | 0.00 | 0.00 | 0.04 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 11.81 |
| Wind mill | 0.06 | 0.18 | 0.16 | 0.13 | 0.13 | 0.09 | 0.30 | 0.05 | 0.10 | 0.10 | 0.20 | 0.06 | 1.37 |
| Ongoing Hydel | 0.83 | 0.77 | 0.14 | 0.96 | 0.35 | 0.88 | 0.37 | 0.22 | 0.21 | 0.15 | 0.25 | 0.49 | 5.61 |
| Solar KSEB | 3.01 | 1.90 | 1.29 | 1.04 | 1.53 | 1.28 | 1.93 | 1.76 | 1.77 | 2.21 | 2.17 | 2.68 | 22.57 |
| Total | 626.70 | 708.64 | 379.22 | 322.62 | 453.65 | 553.26 | 529.70 | 528.67 | 547.21 | 523.55 | 566.58 | 812.51 | 6552.57 |

|  | Annexure G5(2) : Generation 2020-21 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gen. Stations | Apr'20 | May'20 | Jun'20 | Jul'20 | Aug'20 | Sep'20 | Oct'20 | Nov'20 | Dec'20 | Jan'21 | Feb'21 | Mar'21 | Total |
| Idukki | 323.00 | 319.60 | 148.51 | 102.05 | 93.37 | 111.16 | 109.19 | 111.57 | 130.80 | 166.68 | 227.07 | 320.17 | 2163.16 |
| Sabarigiri | 143.60 | 139.60 | 84.69 | 62.87 | 61.20 | 87.60 | 97.60 | 106.30 | 119.60 | 101.20 | 84.60 | 130.60 | 1219.46 |
| Idamalayar | 33.55 | 29.60 | 11.36 | 22.60 | 34.60 | 30.60 | 31.60 | 19.60 | 28.70 | 31.60 | 27.79 | 33.15 | 334.75 |
| Sholayar | 23.60 | 17.32 | 4.51 | 8.10 | 17.90 | 25.01 | 25.30 | 18.90 | 22.30 | 25.00 | 20.77 | 23.29 | 232.00 |
| Pallivasal | 12.69 | 11.20 | 11.90 | 12.65 | 11.43 | 12.61 | 12.87 | 12.40 | 9.99 | 9.87 | 10.61 | 13.28 | 141.50 |
| Kuttiady | 49.69 | 44.60 | 47.69 | 77.60 | 83.64 | 69.60 | 60.20 | 36.60 | 30.60 | 30.80 | 30.60 | 44.50 | 606.12 |
| Panniar | 12.89 | 9.60 | 9.50 | 20.36 | 20.54 | 20.89 | 19.60 | 18.25 | 12.80 | 11.12 | 8.48 | 11.52 | 175.55 |
| NMGM & NES | 20.36 | 17.68 | 37.60 | 49.60 | 47.20 | 50.68 | 48.64 | 37.50 | 22.96 | 13.93 | 13.71 | 17.16 | 377.02 |
| L.Periyar | 19.50 | 21.30 | 52.80 | 99.80 | 107.60 | 61.20 | 59.60 | 53.40 | 26.89 | 15.90 | 14.50 | 16.77 | 549.25 |
| Poringal | 1.89 | 9.20 | 14.90 | 19.80 | 20.58 | 19.64 | 20.50 | 17.63 | 2.25 | 2.35 | 3.28 | 3.10 | 135.13 |
| PLBE | 8.18 | 0.59 | 10.98 | 11.20 | 11.71 | 10.89 | 11.93 | 10.20 | 7.20 | 7.68 | 7.51 | 9.94 | 108.01 |
| Sengulam | 11.40 | 13.57 | 12.98 | 16.80 | 16.33 | 15.89 | 12.24 | 12.65 | 11.41 | 10.21 | 9.03 | 10.76 | 153.27 |
| Kakkad | 23.61 | 24.30 | 14.98 | 15.65 | 13.01 | 16.98 | 16.36 | 16.98 | 16.40 | 15.82 | 12.50 | 15.56 | 202.15 |
| Kallada | 5.69 | 4.38 | 2.80 | 4.83 | 5.80 | 3.92 | 3.11 | 3.67 | 5.59 | 2.02 | 3.13 | 5.87 | 50.82 |
| Peppara | 0.23 | 0.11 | 0.17 | 0.42 | 0.43 | 0.49 | 0.89 | 0.76 | 0.62 | 0.46 | 0.40 | 0.27 | 5.26 |
| Madupetty | 0.60 | 0.60 | 0.19 | 0.00 | 0.02 | 0.19 | 0.11 | 0.12 | 0.24 | 0.27 | 0.35 | 0.67 | 3.37 |
| Chembukadav | 0.00 | 0.00 | 1.62 | 2.84 | 2.04 | 1.36 | 1.42 | 0.65 | 0.44 | 0.40 | 0.40 | 0.00 | 11.18 |
| Malampuzha | 0.00 | 0.00 | 0.00 | 0.00 | 0.22 | 0.10 | 0.06 | 0.22 | 0.57 | 0.46 | 0.09 | 0.00 | 1.72 |
| URUMI | 0.00 | 0.00 | 1.79 | 2.86 | 2.19 | 1.32 | 1.37 | 0.52 | 0.48 | 0.40 | 0.40 | 0.00 | 11.34 |
| Malankara | 3.45 | 3.25 | 2.78 | 2.97 | 2.80 | 2.48 | 2.60 | 2.18 | 1.83 | 2.05 | 2.14 | 2.95 | 31.48 |
| L. Meenmutty | 0.08 | 0.35 | 0.73 | 0.64 | 0.52 | 0.60 | 0.65 | 0.69 | 0.42 | 0.18 | 0.14 | 0.04 | 5.04 |
| Ktdy.tail race | 0.71 | 0.66 | 0.67 | 1.04 | 0.95 | 0.75 | 0.71 | 0.47 | 0.52 | 0.54 | 0.59 | 0.75 | 8.34 |
| Poozhithode | 0.01 | 0.06 | 1.77 | 2.83 | 2.25 | 1.56 | 1.44 | 0.64 | 0.47 | 0.42 | 0.38 | 0.00 | 11.84 |
| Ranni-Perunad | 0.54 | 0.64 | 0.78 | 0.80 | 0.60 | 0.64 | 0.59 | 0.46 | 0.34 | 0.36 | 0.34 | 0.55 | 6.64 |
| Peechi | 0.08 | 0.00 | 0.00 | 0.02 | 0.14 | 0.05 | 0.05 | 0.11 | 0.27 | 0.17 | 0.24 | 0.17 | 1.31 |
| Vilangad | 0.06 | 0.08 | 1.35 | 3.30 | 3.41 | 2.15 | 2.02 | 0.93 | 0.70 | 0.56 | 0.50 | 0.08 | 15.15 |
| Chimmini | 0.23 | 0.12 | 0.57 | 0.88 | 0.98 | 0.72 | 0.50 | 0.70 | 0.56 | 0.79 | 0.63 | 0.51 | 7.20 |
| Adiyanpara | 0.00 | 0.03 | 0.24 | 0.60 | 0.89 | 0.64 | 0.34 | 0.23 | 0.11 | 0.01 | 0.00 | 0.00 | 3.08 |
| Vellathuval | 0.71 | 0.76 | 1.55 | 1.32 | 1.53 | 1.06 | 0.95 | 0.55 | 0.44 | 0.25 | 0.24 | 0.41 | 9.76 |
| Barapole | 2.67 | 2.06 | 1.54 | 2.75 | 3.71 | 4.06 | 2.72 | 1.52 | 1.07 | 1.97 | 1.90 | 1.58 | 27.55 |
| Perunthenaruvi | 0.02 | 0.57 | 1.00 | 1.18 | 0.71 | 0.37 | 0.37 | 0.74 | 0.37 | 0.22 | 0.23 | 0.02 | 5.81 |
| BDPP | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| KDPP | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Wind mill | 0.09 | 0.12 | 0.13 | 0.17 | 0.49 | 0.31 | 0.31 | 0.06 | 0.11 | 0.10 | 0.14 | 0.07 | 2.08 |
| Ongoing Hydel | 4.58 | 4.52 | 3.89 | 4.71 | 4.10 | 4.63 | 11.28 | 11.13 | 11.12 | 11.06 | 11.16 | 11.40 | 93.60 |
| Solar KSEB | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 47.60 |
| Total | 707.66 | 680.44 | 489.93 | 557.21 | 576.86 | 564.11 | 561.10 | 502.30 | 472.14 | 468.83 | 497.82 | 679.11 | 6757.51 |

|  | Annexure G5(3) : Generation 2021-22 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gen.Stations | Apr'21 | May'21 | Jun'21 | Jul'21 | Aug'21 | Sep'21 | Oct'21 | Nov'21 | Dec'21 | Jan'22 | Feb'22 | Mar'22 | Total |
| Idukki | 311.60 | 317.50 | 142.25 | 103.16 | 98.68 | 111.60 | 110.53 | 106.36 | 127.51 | 173.22 | 239.00 | 360.10 | 2201.50 |
| Sabarigiri | 141.30 | 137.20 | 87.13 | 64.03 | 63.44 | 88.84 | 98.53 | 106.65 | 108.97 | 107.91 | 97.58 | 155.76 | 1257.34 |
| Idamalayar | 31.88 | 27.90 | 10.65 | 20.60 | 35.60 | 33.50 | 32.30 | 18.90 | 29.80 | 32.80 | 26.80 | 34.50 | 335.23 |
| Sholayar | 23.60 | 17.32 | 4.51 | 8.10 | 17.90 | 25.01 | 25.30 | 18.90 | 22.30 | 25.00 | 20.77 | 23.29 | 232.00 |
| Pallivasal | 12.90 | 12.20 | 11.90 | 12.65 | 11.43 | 12.61 | 12.87 | 12.40 | 9.99 | 9.87 | 10.61 | 13.28 | 142.71 |
| Kuttiady | 47.65 | 44.50 | 39.60 | 72.30 | 80.40 | 66.40 | 54.40 | 30.40 | 28.80 | 32.20 | 28.90 | 43.60 | 569.15 |
| Panniar | 12.00 | 8.90 | 9.10 | 19.80 | 20.04 | 19.60 | 19.60 | 18.25 | 12.80 | 11.12 | 8.48 | 11.52 | 171.21 |
| NMGM & NES | 18.80 | 17.23 | 36.60 | 47.60 | 46.50 | 48.80 | 48.64 | 37.50 | 22.96 | 13.93 | 13.71 | 17.16 | 369.43 |
| L.Periyar | 20.20 | 22.40 | 50.70 | 92.40 | 104.50 | 60.60 | 56.60 | 49.90 | 26.89 | 15.90 | 14.50 | 16.77 | 531.36 |
| Poringal | 1.66 | 9.01 | 14.40 | 17.80 | 20.01 | 18.94 | 19.80 | 17.63 | 2.25 | 2.35 | 3.28 | 3.10 | 130.23 |
| PLBE | 7.98 | 0.56 | 10.62 | 10.90 | 11.01 | 10.02 | 11.03 | 10.20 | 7.20 | 7.68 | 7.51 | 9.94 | 104.65 |
| Sengulam | 11.01 | 11.80 | 12.08 | 16.20 | 16.00 | 15.89 | 12.24 | 12.65 | 11.41 | 10.21 | 9.03 | 10.76 | 149.28 |
| Kakkad | 22.80 | 23.96 | 14.04 | 15.44 | 12.90 | 16.98 | 16.36 | 16.98 | 16.40 | 15.82 | 12.50 | 15.56 | 199.74 |
| Kallada | 4.93 | 3.91 | 3.02 | 5.59 | 6.79 | 5.07 | 4.01 | 4.38 | 5.18 | 2.40 | 3.41 | 6.02 | 54.73 |
| Peppara | 0.26 | 0.12 | 0.22 | 0.53 | 0.55 | 0.63 | 0.90 | 0.79 | 0.63 | 0.52 | 0.41 | 0.26 | 5.82 |
| Madupetty | 0.62 | 0.63 | 0.17 | 0.00 | 0.03 | 0.25 | 0.13 | 0.10 | 0.18 | 0.26 | 0.31 | 0.69 | 3.37 |
| Chembukadav | 0.00 | 0.00 | 1.80 | 3.11 | 2.27 | 1.38 | 1.48 | 0.70 | 0.54 | 0.51 | 0.51 | 0.00 | 12.31 |
| Malampuzha | 0.00 | 0.00 | 0.00 | 0.00 | 0.28 | 0.13 | 0.08 | 0.21 | 0.53 | 0.44 | 0.10 | 0.00 | 1.77 |
| URUMI | 0.00 | 0.00 | 1.94 | 3.12 | 2.44 | 1.46 | 1.45 | 0.59 | 0.55 | 0.51 | 0.51 | 0.00 | 12.57 |
| Malankara | 3.29 | 2.96 | 2.96 | 3.08 | 2.88 | 2.69 | 2.92 | 2.35 | 2.03 | 2.19 | 2.27 | 3.14 | 32.76 |
| L. Meenmutty | 0.05 | 0.35 | 0.81 | 0.65 | 0.61 | 0.60 | 0.64 | 0.67 | 0.43 | 0.21 | 0.17 | 0.05 | 5.22 |
| Ktdy.tail race | 0.74 | 0.65 | 0.69 | 1.09 | 1.04 | 0.83 | 0.79 | 0.53 | 0.55 | 0.59 | 0.61 | 0.79 | 8.90 |
| Poozhithode | 0.00 | 0.05 | 1.98 | 3.09 | 2.47 | 1.71 | 1.60 | 0.69 | 0.57 | 0.52 | 0.49 | 0.00 | 13.17 |
| Ranni-Perunad | 0.52 | 0.63 | 0.78 | 0.84 | 0.71 | 0.64 | 0.56 | 0.45 | 0.30 | 0.41 | 0.38 | 0.61 | 6.83 |
| Peechi | 0.09 | 0.00 | 0.00 | 0.02 | 0.17 | 0.07 | 0.06 | 0.10 | 0.29 | 0.16 | 0.24 | 0.16 | 1.38 |
| Vilangad | 0.08 | 0.10 | 1.48 | 3.49 | 3.63 | 2.40 | 2.17 | 1.07 | 0.85 | 0.69 | 0.63 | 0.11 | 16.69 |
| Chimmini | 0.29 | 0.16 | 0.73 | 1.13 | 1.13 | 0.74 | 0.55 | 0.67 | 0.61 | 0.79 | 0.80 | 0.65 | 8.25 |
| Adiyanpara | 0.00 | 0.03 | 0.31 | 0.76 | 1.14 | 0.72 | 0.23 | 0.18 | 0.08 | 0.01 | 0.00 | 0.00 | 3.48 |
| Vellathuval | 0.31 | 0.33 | 1.20 | 0.79 | 1.10 | 0.74 | 0.49 | 0.22 | 0.17 | 0.20 | 0.19 | 0.29 | 6.03 |
| Barapole | 2.67 | 2.06 | 1.54 | 2.75 | 3.71 | 4.06 | 2.72 | 1.52 | 1.07 | 1.97 | 1.90 | 1.58 | 27.55 |
| Perunthenaruvi | 0.03 | 0.73 | 1.28 | 1.51 | 0.91 | 0.48 | 0.48 | 0.95 | 0.47 | 0.28 | 0.30 | 0.03 | 7.44 |
| BDPP | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| KDPP | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Wind mill | 0.09 | 0.18 | 0.16 | 0.13 | 0.13 | 0.09 | 0.07 | 0.06 | 0.09 | 0.12 | 0.11 | 0.13 | 1.37 |
| Ongoing Hydel | 11.74 | 11.68 | 11.05 | 11.87 | 11.26 | 11.79 | 29.86 | 29.71 | 29.70 | 29.64 | 29.74 | 29.98 | 248.06 |
| Solar KSEB | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 3.97 | 47.60 |
| Total | 693.06 | 678.95 | 479.63 | 548.56 | 585.97 | 569.43 | 573.62 | 506.63 | 476.09 | 504.39 | 539.78 | 763.71 | 6919.83 |

| Annexure G6: Generation from ongoing Solar Projects (MU) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Jan | Feb | Mar | Total |
| 2018-19 | 0.8308 | 0.7196 | 0.3813 | 0.3695 | 0.3523 | 1.0129 | 1.044491 | 1.305804 | 1.333721 | 2.263952 | 2.089743 | 2.786577 | 14.49069 |
| 2019-20 | 3.01 | 1.90 | 1.29 | 1.04 | 1.53 | 1.28 | 1.93 | 1.76 | 1.77 | 2.21 | 2.17 | 2.68 | 22.57 |
| 2020-21 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 47.59508 |
| 2021-22 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 3.966257 | 47.59508 |

| Annexure G7 Annexure G7 - Annexure G7 - Auxiliary Consumption |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stations | Type of station |  | 2019-20 |  |  | 2020-21 |  | 2021-22 |  |
|  | surface hydro /underground | Excitation system | Aux Consp as per norms (%) | Anticipated energy (MU) | Aux as per norms (MU) | Anticipated energy (MU) | Aux as per norms (MU) | Anticipated energy (MU) | Aux as per norms (MU) |
| Kuttiady+KES | surface hydro | rotating | 0.7 | 614.949 | 4.304643 | 606.12 | 4.24284 | 569.15 | 3.98405 |
| Poringal | surface hydro | brushless | 1 | 107.9417 | 1.079417 | 135.1264 | 1.351264 | 130.2317 | 1.302317 |
| PLBE | surface hydro | brushless | 1 | 99.04811 | 0.990481 | 108.01 | 1.0801 | 104.6503 | 1.046503 |
| Sholayar | surface hydro | brushless | 1 | 231.3476 | 2.313476 | 232.0004 | 2.320004 | 232.0004 | 2.320004 |
| EDMR | surface hydro | static | 1 | 309.6503 | 3.096503 | 334.7519 | 3.347519 | 335.23 | 3.3523 |
| Pallivasal | surface hydro | brushless | 1 | 143.4877 | 1.434877 | 141.5024 | 1.415024 | 142.7124 | 1.427124 |
| Sengulam | surface hydro | static | 1 | 150.8022 | 1.508022 | 153.2735 | 1.532735 | 149.2835 | 1.492835 |
| Panniar | surface hydro | static | 1 | 154.774 | 1.54774 | 175.5525 | 1.755525 | 171.2125 | 1.712125 |
| NLM | surface hydro | static | 1 | 327.9263 | 3.279263 | 377.0198 | 3.770198 | 369.4298 | 3.694298 |
| LP | surface hydro | static | 1 | 439.0266 | 4.390266 | 549.254 | 5.49254 | 531.358 | 5.31358 |
| Idukki | underground | static | 1.2 | 2311.84 | 27.74208 | 2163.16 | 25.95792 | 2201.5 | 26.418 |
| Sabarigiri | surface hydro | static | 1 | 1247.05 | 12.4705 | 1219.46 | 12.1946 | 1257.34 | 12.5734 |
| Kakkad | surface hydro | rotating | 0.7 | 186.5827 | 1.306079 | 202.1512 | 1.415058 | 199.7412 | 1.398188 |
| Kallada | surface hydro | static | 1 | 38.33176 | 0.383318 | 50.81681 | 0.508168 | 54.72703 | 0.54727 |
| Peppara | surface hydro | static | 1 | 5.725991 | 0.05726 | 5.257831 | 0.052578 | 5.817951 | 0.05818 |
| Madupetty | surface hydro | static | 1 | 2.895619 | 0.028956 | 3.366487 | 0.033665 | 3.373995 | 0.03374 |
| Chembukadav | surface hydro | static | 1 | 7.004372 | 0.070044 | 11.17883 | 0.111788 | 12.30547 | 0.123055 |
| Malampuzha | surface hydro | static | 1 | 2.655347 | 0.026553 | 1.715409 | 0.017154 | 1.76599 | 0.01766 |
| Urumi | surface hydro | static | 1 | 12.18216 | 0.121822 | 11.33713 | 0.113371 | 12.56937 | 0.125694 |
| Malankara | surface hydro | static | 1 | 28.93958 | 0.289396 | 31.47764 | 0.314776 | 32.75803 | 0.32758 |
| Lower Meenmutty | surface hydro | static | 1 | 4.650462 | 0.046505 | 5.04015 | 0.050402 | 5.223493 | 0.052235 |
| Ktdy. tail race | surface hydro | static | 1 | 6.006984 | 0.06007 | 8.343492 | 0.083435 | 8.902074 | 0.089021 |
| Poozhithode | surface hydro | static | 1 | 6.415536 | 0.064155 | 11.83713 | 0.118371 | 13.16543 | 0.131654 |
| Ranni-Perunadu | surface hydro | static | 1 | 6.338098 | 0.063381 | 6.640708 | 0.066407 | 6.829647 | 0.068296 |
| Peechi | surface hydro | static | 1 | 1.970954 | 0.01971 | 1.310198 | 0.013102 | 1.376721 | 0.013767 |
| Vilangad | surface hydro | static | 1 | 16.06328 | 0.160633 | 15.14513 | 0.151451 | 16.69496 | 0.16695 |
| Chimini | surface hydro | static | 1 | 4.291149 | 0.042911 | 7.197399 | 0.071974 | 8.249738 | 0.082497 |
| Adiyanpara | surface hydro | static | 1 | 2.857132 | 0.028571 | 3.076822 | 0.030768 | 3.481686 | 0.034817 |
| Vellathuval | surface hydro | static | 1 | 0.45978 | 0.004598 | 9.757084 | 0.097571 | 6.030102 | 0.060301 |
| Barapole | surface hydro | static | 1 | 28.96594 | 0.289659 | 27.54621 | 0.275462 | 27.54621 | 0.275462 |
| Perunthenaruvi | surface hydro | static | 1 | 10.72507 | 0.107251 | 5.807387 | 0.058074 | 7.438775 | 0.074388 |
| Existing HYDRO Total |  |  | 1 | 6510.905 | 67.32814 | 6614.234 | 68.04385 | 6622.096 | 68.31729 |
| Ongoing Hydro |  |  | 1 | 5.612 | 0.05612 | 93.596 | 0.93596 | 248.06 | 2.4806 |
| Total Hydro |  |  |  | 6516.517 | 67.38426 | 6707.83 | 68.97981 | 6870.156 | 70.79789 |
| Net Existing Hydro |  |  |  | 6449.133129 |  | 6638.850057 |  | 6799.358505 |  |